3GPP TSG-RAN WG3 Meeting #130                                                             R3-258180

Dallas USA, 17 – 21 November 2025

Agenda item: 		10.2.2

Source: 		Qualcomm Incorporated

Title: 		(pCR to TR 38.760-3) Discussion on 6G RAN logical architecture

Document for:		Other

# 1	Introduction

TSG RAN#108 agreed on a new SI on 6G Radio. TSG RAN#109 further agreed the following requirements for architecture and migration for 6G [2]:

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

This document discusses the 6G RAN logical architecture.

# 2	Discussion

The 6G RAN logical architecture can be regarded as an evolution of the 5G RAN logical architecture. As in the 5G logical architecture, the 6G RAN supports a 6G RAN-node. The terminology of the 6G RAN-node is FFS.

Proposal 1: The 6G RAN to support the 6G RAN-node. FFS on the terminology used for the 6G RAN-node.

As in 5G, the 6G RAN-node supports an interface with the UE. This interface will be defined by RAN2.

Proposal 2: The 6G RAN-node supports an interface with the UE which is defined by RAN2.

As defined by the SID [1] and TSG RAN agreements [2], the 6G RAN-node should support standalone (SA) operation. Whether the 6G RAN-node can also support non-SA operation is up to further discussion by TSG RAN.

Proposal 3: The 6G RAN-node to support SA operation. Whether the 6G RAN-node supports NSA is up to TSG RAN discussion.

In SA operation, the 6G RAN-node supports an interface with the 6G CN. As in 4G and 5G, the 6G RAN node should also support an interface with peer 6G RAN-nodes. The nature of the 6G RAN-to-CN interface and the inter-6G-RAN-node interface, i.e., whether they can be represented as point-to-point or service-based interfaces, is part of the 6G Study and discussed in other contributions.

Proposal 4: In SA operation, the 6G RAN-node to support an interface with the 6G CN and with peer 6G RAN-nodes. FFS the nature of these interfaces, e.g., whether they are point-to-point or service-based.

Figure 1 shows the RAN logical architecture based on the above proposals.

Figure 1: RAN logical architecture

RAN3 needs to determine the functions in RAN3 scope that need to be supported by the 6G RAN node on Day 1. It can be assumed that 6G RAN supports at least a subset of the functions defined for a SA 5G RAN-node in TS 38.300 and TS 38.401.

The RAN-node functions defined by TS 38.300 are primarily in RAN2 scope. Based on TS 38.401, the following functions can be considered as the baseline for the 6G RAN-node:

- Support for U-plane and C-plane

- Data forwarding between UE and CN for U-plane traffic and NAS for C-plane

- Support of UE mobility

- Startup and cell activation

- RAN sharing

- Remote interference management

- Cross-link interference management

- Support for non-public networks

- RACH optimization function

- PCI optimization function

- Support for CCO, MRO for mobility and other SON functions

- Support of AI/ML for NG-RAN and air interface

- Support of RAN-visible QoE measurement

Proposal 5: RAN3 to study the functions to be supported by the 6G RAN-node on Day 1.

Proposal 6: The study to consider the following functions for the 6G RAN-node as a starting point:

- Support for U-plane and C-plane

- Data forwarding between UE and CN for U-plane traffic and NAS for C-plane

- Support of UE mobility

- Support of network slicing

- Support of service awareness

- Startup and cell activation

- RAN sharing

- Remote interference management

- Cross-link interference management

- Support for non-public networks

- RACH optimization function

- PCI optimization function

- Support for CCO, MRO for mobility and other SON functions

- Support of AI/ML for NG-RAN and air interface

- Support for NES

The Annex contains a pCR to TR 38.760-3 on the 6G RAN logical node, which captures the above aspects.

Proposal 7: Agre on pCR to TR 38.760-3 in the Annex.

# 3	Conclusion

This contribution discussed the 6G RAN logical architecture. The following proposals have been made:

Proposal 1: The 6G RAN to support the 6G RAN-node. FFS on the terminology used for the 6G RAN-node.

Proposal 2: The 6G RAN-node supports an interface with the UE which is defined by RAN2.

Proposal 3: The 6G RAN-node to support SA operation. Whether the 6G RAN-node supports NSA is up to TSG RAN discussion.

Proposal 4: In SA operation, the 6G RAN-node to support an interface with the 6G CN and with peer 6G RAN-nodes. FFS the nature of these interfaces, e.g., whether they are point-to-point or service-based.

Proposal 5: RAN3 to study the functions to be supported by the 6G RAN-node on Day 1.

Proposal 6: The study to consider the following functions for the 6G RAN-node as a starting point:

- Support for U-plane and C-plane

- Data forwarding between UE and CN for U-plane traffic and NAS for C-plane

- Support of UE mobility

- Support of network slicing

- Support of service awareness

- Startup and cell activation

- RAN sharing

- Remote interference management

- Cross-link interference management

- Support for non-public networks

- RACH optimization function

- PCI optimization function

- Support for CCO, MRO for mobility and other SON functions

- Support of AI/ML for NG-RAN and air interface

- Support for NES

Proposal 7: Agre on pCR to TR 38.760-3 in the Annex.

# 4	References

[1]  RP-251881, 3GPP TSG RAN #108, Prague, Czech Republic, June 2025. Further updates in RP-25912, 3GPP TSG RAN #109, Beijing, China, September 2025.

[2]  RP-252870, pCR for 38.914 on requirements for architecture and migration for 6G, 3GPP TSG RAN #109, Beijing, China, September 2025.

# 5	Annex: pCR to TR 38.760-3

-------------------------------------------Beginning of changes-------------------------------------------

## 5.3.2	RAN Functions

Editor’s note: The aim of this section is to describe the functions supported in RAN.

The 6G RAN supports the 6G RAN-node.

- FFS 6G RAN-node functional split.

The 6G RAN-node supports an interface with the UE, which will be defined by RAN2. The 6G RAN-node further supports an interface to the 6G CN and an interface to peer 6G RAN nodes. Figure 5.3.2-1 shows the RAN logical architecture.

- FFS the nature of these interfaces, e.g., on whether they are point-to-point or service-based.

Figure 5.3.1-1: RAN logical architecture

The 6G RAN-node may support the following functions in RAN3 scope:

- Support for U-plane and C-plane

- Data forwarding between UE and CN for U-plane traffic and NAS for C-plane

- Support of UE mobility

- Support of network slicing

- Support of service awareness

- Startup and cell activation

- RAN sharing

- Remote interference management

- Cross-link interference management

- Support for non-public networks

- RACH optimization function

- PCI optimization function

- Support for CCO, MRO for mobility and other SON functions

- Support of AI/ML for NG-RAN and air interface

- Support for NES

-------------------------------------------End of changes-------------------------------------------