3GPP TSG-RAN3 Meeting #130	R3-258285

Dallas, US, 17 - 21 November, 2025

Agenda item:	10.4

Source:	Xiaomi

Title:	RAN internal functional split and interfaces

Document for:	Discussion and Decision

1	Introduction

A new study item was agreed at RAN#108 and revived in RAN#109 that aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements [1].

One of the objectives of the study is to study the RAN internal functional split, interfaces.

(6)Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] a)Overall RAN architecture aspects [RAN3, RAN2] b)RAN-CN functional split, interface, protocol stack and procedures [RAN3] c)RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we discussed RAN internal function split based on the open issues in last RAN3 meeting below and also discussed the the functional aspects of the RAN-RAN interface.

From RAN3#129bis:

Capture pain points and benefits (and potentially derived requirements) of 5G HLS

2	Discussion

2.1 	RAN internal split

CU-CP/CU-UP split

In 5G RAN split architecture, there are two types of split, The CU-DU split, which separates the Centralized Unit (hosting RRC, PDCP) from the Distributed Unit (hosting RLC, MAC, PHY). The CU-CP/CU-UP split (or Control Plane/User Plane split) is a further disaggregation within the CU itself, separating the CU-Control Plane (CU-CP, hosting RRC and PDCP-C) from the CU-User Plane (CU-UP, hosting PDCP-U and the SDAP layer).

In general, the CU-DU separation, while introducing challenges like latency and signaling overhead, provides benefits such as resource pooling and centralized coordination. These issues are seen as manageable through continuous enhancements. In contrast, the further separation of the CU into Control Plane (CU-CP) and User Plane (CU-UP) is proposed to be excluded from the 6G due to its more severe and fundamental drawbacks, primarily:

- Overwhelming Complexity: Introduces a new network node (CU-UP) and a new critical interface (E1), exponentially increasing integration, testing, and operational overhead.

- Additional Latency and Signaling: Adds another coordination hop (CU-CP <-> CU-UP) for user plane management, increasing control latency and potential points of failure.

- Lack of Commercial Deployment and Immaturity: Has not proven its value in real-world 5G networks, showing a clear lack of industry need.

From the RAN3 discussion perspective, it is recommended to prioritize discussions on the CU-DU split for 6G, and revisit the need for further CU separation only if justified by future requirements.

Observation 1, The CU-DU separation offers benefits such as centralized coordination and resource pooling, and while it introduces latency and signaling challenges, these are considered manageable.

Observation 2, The further disaggregation of CU into CU-CP and CU-UP introduces significant complexity, latency, and operational overhead, with limited real-world adoption in 5G.

Proposal 1, RAN3 focus on CU-DU split discussion first, and may further evaluate the necessity of CU-CP/CU-UP separation if required.

CU-DU split

Regarding CU-DU split, we focus on the following pain points for RAN3 consideration.

- Multi-vendor interoperability

- Although the F1 interface standard is designed to enable multi-vendor interoperability between the CU and DU, real-world integration often faces significant challenges. These stem from proprietary extensions and implementation-specific variations, which frequently lead operators to deploy CU and DU components from a single vendor—limiting deployment flexibility and increasing operational costs. If the CU-DU split is to be adopted in 6G, it is important for operators to clearly articulate whether multi-vendor interoperability is a necessary requirement, as this will directly impact specification priorities and complexity.

- Proposal 2, Operators should clarify whether multi-vendor interoperability is required for CU-DU split.

- Delays and inefficiency

Control Plane Processing Delays: the centralized nature of the CU-RRC unnecessarily involves the CU in DU-level (L1/L2) procedures, creating a convoluted signaling path (CU->DU->CU->DU->UE). This significantly extends the time required for critical control plane procedures such as handovers, connection re-establishment, and service setup.

MAC Layer Inefficiency: The slow RRC procedures force the system to use the MAC layer as a suboptimal workaround for transmitting control information, consuming precious radio resources and reducing spectral efficiency.

Observation 3, The 5G RAN CU-DU split introduces control plane latency and MAC layer inefficiencies due to centralized RRC handling, resulting in excessive signaling and suboptimal resource management.

Proposal 3, If the CU-DU split is adopted, the functional design should consider the low-latency control signaling and efficient transmission mechanisms.

3. Dependency on the RAN2 UP design

The CU-DU functional split is closely tied to RAN2’s user plane (UP) design. For instance, while RAN2 is exploring proposals to merge PDCP and RLC layers, the existing 5G CU-DU split places the boundary between PDCP and RLC. This creates a direct dependency between the two groups. Some discussions have suggested that RAN2 should await RAN3’s conclusions on the CU-DU split, as those decisions will significantly impact the UP design. This underscores the need for coordinated progress between RAN2 and RAN3 to ensure alignment in both functional and protocol design.

Observation 4, The CU-DU functional split is directly influenced by RAN2’s user plane design and vice visa.

Proposal 4, RAN2 and RAN3 should collaborate closely on CU-DU split discussions to ensure consistent and aligned outcomes across both functional and user plane designs.

2.2 	RAN-RAN interface

- According to TS 38.300 [2], the 5G Xn interface consists of two components: Xn-C for the control plane and Xn-U for the user plane. For 6G, it is recommended that the user plane protocol and the control plane protocol and its underlying transport should be harmonized with the architecture adopted for the 6G RAN-CN interface—such as point-to-point (P2P) or service-based interface (SBI) models. This alignment is essential to avoid unnecessary complexity in protocol design and ensure consistency across the overall system architecture.

- Whether the 6G Xn interface includes a data plane will depend on the outcome of discussions regarding data transfer requirements. If data transfer is supported between RAN nodes, the inclusion of a data plane protocol will be necessary, and the corresponding protocol design should also be aligned with RAN-CN interface.

- Proposal 5, 6G Xn interface includes Xn User Plane and Xn Control Plane, the protocol design shall align with RAN-CN interface.

- Legacy Xn functions

- For 6G Xn functions, the existing 5G Xn functionalities serve as the baseline, including:

-  Data forwarding (Xn-U);

-	Flow control (Xn-U);

-	Xn interface management (Xn-C);

-	UE mobility management, including context transfer and RAN paging(Xn-C);

-	Dual connectivity ((Xn-C)).

The table below analyzes whether the 6G RAN-RAN interface requires functions similar to 5G Xn.

| 5G Xn function | Analysis | To be considered as 6G Xn function |
| --- | --- | --- |
| Data forwarding; Flow control. | Data forwarding and flow control are still required in 6G RAN-RAN interface | Yes |
| Xn interface management; | Should align with RAN-CN interface (P2P/SBI); SBI adoption may necessitate new/common interface management | Pending to P2P/SBI discussion |
| UE mobility management, including context transfer and RAN paging; | UE mobility management is needed, but context transfer/RAN paging depend on UE-state discussions | Yes, details are pending to UE state/paging discussion |
| Dual connectivity | To be addressed at RAN Plenary level, and in our view, Dual Connectivity is recommended for exclusion due to its complexity | No |

Observation 5, User-plane data forwarding and flow control remain essential for the 6G RAN-RAN interface.

Observation 6, The requirement for Xn interface management functionality depends on P2P/SBI protocol decisions.

Observation 7, UE mobility specifics (e.g., context transfer and paging) require are pending to UE state and paging mechanism discussions.

Observation 8, Dual connectivity complexity warrants further evaluation at the RAN Plenary level.

Proposal 6, RAN3 should investigate RAN-to-RAN interface functional aspects, using these legacy capabilities as starting point:

- Data forwarding

- Flow control

- UE mobility management

- AI related functions

As with legacy systems, supporting AI in the network requires data collection and forwarding between RAN nodes to enable AI lifecycle management (LCM). Furthermore, to realize native-AI operations, AI/ML for air interface (e.g., AI-based CSI, Beam management,...) and 6G RAN (e.g., AI-based load balancing, mobility, CCO,...) are expected to be extensively deployed on the network side. Given this trend, energy efficiency for AI-related operations within the network side must also be addressed, for the following reasons:

-High Computational Demand: AI/ML processing tasks performed by base stations (e.g., for channel state prediction, beamforming optimization, traffic forecasting, network slicing management, anomaly detection) are computationally intensive and consume significant power.

-Growing Contribution to Network Energy: As AI/ML becomes integral to optimizing RAN performance , its power consumption will constitute an increasingly large portion of the total energy usage of cellular networks.

- Operational Cost and Sustainability: Energy efficiency is paramount for network operators to reduce operational expenditure (OPEX) and meet environmental sustainability goals. Optimizing AI/ML processing power directly impacts the network's overall energy efficiency.

Energy-efficient AI operations in the network may involve strategies such as computational resource sharing and load balancing across gNBs to reduce power consumption during AI-driven tasks. However, the trade-off between energy savings and overall system performance must be carefully evaluated. To support these optimizations, the 6G Xn interface should accommodate AI-related data exchange—potentially including non-UE-associated information—between 6G RAN nodes, specifically for energy-saving purposes.

- Observation 9: Inter-node data collection remains essential for 6G Xn, consistent with legacy system requirements.

- Observation 10: AI/ML deployment in 6G RAN/air interfaces faces critical energy challenges due to rising computational demands.

- Proposal 7: RAN3 should investigate these Xn functionalities for Native AI: AI related Data collection and forwarding

- Sensing related functions

- As agreed in RAN#109 meeting in [3], all the sensing modes are agreed to be studied in 6G.

- Moderator proposal 4: - -6GR should study the sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic. - -6GR should study cooperative sensing/multi-static sensing mechanism involving multiple UEs and TRPs.

- When TRPs reside on separate RAN nodes, sensing coordination via the 6G Xn interface may be required for efficient resource configuration and updates. Direct inter-base station coordination is preferred over core-network-mediated approaches due to the time-sensitive nature of sensing services. Consequently, introducing an Xn-based sensing coordination function should be considered, the details should be based on the progress in RAN2/SA2.

- Observation 11: Direct inter-base station coordination is preferable to core-network-mediated approaches for sensing services due to time-sensitivity requirements.

- Proposal 8: RAN3 should investigate potential sensing coordination functions for 6G Xn, based on the RAN2/SA2 progresses on sensing.

In the Annex, we provide a pCR to 6G TR 38.760-3 on general principles, RAN-RAN interface .

Proposal 9, RAN3 agrees the pCR to 6G TR 38.760-3 in the Annex.

3	Conclusion

In this contribution, we discussed the RAN-RAN interface and RAN internal function split, and have the following observations and proposals.

RAN internal split

Observation 1, The CU-DU separation offers benefits such as centralized coordination and resource pooling, and while it introduces latency and signaling challenges, these are considered manageable.

Observation 2, The further disaggregation of CU into CU-CP and CU-UP introduces significant complexity, latency, and operational overhead, with limited real-world adoption in 5G.

Proposal 1, RAN3 focus on CU-DU split discussion first, and may further evaluate the necessity of CU-CP/CU-UP separation if required.

Proposal 2, Operators should clarify whether multi-vendor interoperability is required for CU-DU split.

Observation 3, The 5G RAN CU-DU split introduces control plane latency and MAC layer inefficiencies due to centralized RRC handling, resulting in excessive signaling and suboptimal resource management.

Proposal 3, If the CU-DU split is adopted, the functional design should consider the low-latency control signaling and efficient transmission mechanisms.

Observation 4, The CU-DU functional split is directly influenced by RAN2’s user plane design and vice visa.

Proposal 4, RAN2 and RAN3 should collaborate closely on CU-DU split discussions to ensure consistent and aligned outcomes across both functional and user plane designs.

RAN-RAN interface

- Proposal 5, 6G Xn interface includes Xn User Plane and Xn Control Plane, the protocol design shall align with RAN-CN interface.

- Legacy 5G functions

Observation 5, User-plane data forwarding and flow control remain essential for the 6G RAN-RAN interface.

Observation 6, The requirement for Xn interface management functionality depends on P2P/SBI protocol decisions.

Observation 7, UE mobility specifics (e.g., context transfer and paging) require are pending to UE state and paging mechanism discussions.

Observation 8, Dual connectivity complexity warrants further evaluation at the RAN Plenary level.

Proposal 6, RAN3 should investigate RAN-to-RAN interface functional aspects, using these legacy capabilities as starting point:

- Data forwarding

- Flow control

- UE mobility management

- AI related Functions

- Observation 9: Inter-node data collection remains essential for 6G Xn, consistent with legacy system requirements.

- Observation 10: AI/ML deployment in 6G RAN/air interfaces faces critical energy challenges due to rising computational demands.

- Proposal 7: RAN3 should investigate these Xn functionalities for Native AI: AI related Data collection and forwarding.

- Sensing related Functions

- Observation 11: Direct inter-base station coordination is preferable to core-network-mediated approaches for sensing services due to time-sensitivity requirements.

- Proposal 8: RAN3 should investigate potential sensing coordination functions for 6G Xn, based on the RAN2/SA2 progresses on sensing.

Proposal 9, RAN3 agrees pCR to 6G TR 38.760-3 in the Annex.

4	Reference

[1]	RP-252912, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator)

[2] 3GPP TS 38.300, NR; NR and NG-RAN Overall Description, stage2.

[3]RP-252943, Moderator's summary for RAN led 6G SI: Requirements of new and existing services, CMCC (Moderator)

5	Annex ( pCR to 6G TR 38.760-3)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

6.2	RAN Internal Interfaces

Editor’s Note: This chapter shall comprehensively address study on CU-DU and CP-UP aspects including functional split considering interim milestone to take a decision during TSG#115: March 2027

Editor’s Note: This chapter should also address the interfaces between 6G RAN nodes (equivalent to Xn).

6.2.1	 General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN Internal Interfaces

The 6G RAN architecture shall support simplified internal function split with reduced deployment options compared with 5G NR.

The 6G RAN architecture shall support internal function split with low control signalling latency and efficient transmission.

6.2.x1	RAN-RAN interface

6.2.x1.1  6G Xn User Plane

The 6G Xn User plane (6G Xn-U) interface is defined between two 6G RAN nodes. The user plane protocol stack on the 6G Xn interface is shown in Figure 6.2.x1.1-1. The transport network layer is built on IP transport and GTP-U is used on top of UDP/IP to carry the user plane PDUs.

Figure 6.2.x1.1-1: 6G Xn-U Protocol Stack

Xn-U provides non-guaranteed delivery of user plane PDUs and supports the following functions:

-	Data forwarding;

-	Flow control.

6.2.x1.2	6G Xn Control Plane

The 6G Xn control plane interface (6G Xn-C) is defined between two 6G RAN nodes.

Editor’s note 1: the protocol stacks for 6G Xn control plane should be aligned with 6G RAN-CN control plane interface, which need further discussion.

The 6G Xn-C interface supports the following functions:

-	Xn interface management;

-	UE mobility management;

-	Data collection and forwarding

-  AI resource sharing

-  	Sensing measurement coordination

Editor’s note 2: the details of sensing measurement coordination is pending to RAN2/SA2 discussion.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>