3GPP TSG-RAN WG3 Meeting #130	R3-258289

Dallas, USA, 17th – 21st November, 2025

Agenda Item:	13.2

Source: 	OPPO

Title:  	Discussion on protocol stack for sensing reporting

Document for:	Discussion and Decision

- Introduction

In Rel-20 5G-A ISAC SID [1], the objectives led by RAN3 are captured as follows.

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

This contribution would like to provide our considerations on these objectives, mainly covering protocol stack for sensing reporting.

- Discussion

How to transfer sensing data between RAN and SF

As for the measurements and relevant measurement quantization, sensing measurements can be systematically acquired at fourth hierarchical stages of the signal processing pipeline, which are discussing in RAN1.

- First, full channel state information (CSI) is estimated as the foundational input for sensing operations.

- Second, delay/doppler/angle profiles are generated via transform-domain processing (e.g., FFT/IFFT).

- Third, intermediate sensing results, e.g., point cloud information, are obtained by further processing such as filter/compression.

- Fourth, detection algorithms extract refined target parameters or final sensing results.

All of the measurement results obtained from these fourth stages can be listed as candidate measurements and reporting quantities, but these measurement quantities may cause significant signalling overhead, especially for the first three types of measurement quantities, necessitating efficient compression or reporting methods.

- Significant signalling overhead for sensing measurement reporting is foreseen, especially when full channel state information, delay/doppler/angle profiles, or point cloud information are reported.

Therefore, one open issue is how to transfer sensing data between RAN node and SF considering the potential heavy signalling overhead due to sensing measurement reporting.

Considering that sensing and positioning are somehow similar, where positioning aims to obtain the location/velocity/direction of UE, sensing aims to obtain the sensing info at least including location/velocity/direction of non-UE object (i.e., UAV sensing target), NRPPa protocol designed for positioning signalling between gNB and LMF could be used to analyse when we start to design sensing procedures and signalling between RAN and SF.

In legacy, NRPPa signalling is carried by NGAP signalling via the NG control plane interface, i.e., relying on AMF to forward the NRPPa signalling from/to RAN node to/from LMF. The protocol stack is shown as below.

The transfer of NRPPa signalling for positioning does not consider the enhancement of heavy signalling overhead. For sensing, if we still rely on AMF to forward sensing data, AMF would be a bottleneck for signalling transfer between RAN node and SF, since it would cause significant signalling overhead for AMF. So legacy manner to transfer NRPPa signalling via AMF over CP is not applicable for sensing.

- If relying on AMF to forward sensing data over CP, AMF would be a bottleneck for heavy signalling transfer between RAN node and SF.

As captured in SA2 Rel-20 ISAC TR 23.700-14 [2], some UP-like solutions are proposed, as shown below, where most of them aim to directly transfer sensing data between RAN node and SF without other NFs such as UPF in-between, and the difference is what protocol stack the sensing data is carried through for transfer.

- Based on SA2 Rel-20 ISAC TR 23.700-14, it seems that common view of SA2 is that sensing data and the associated information is transported via direct connection between RAN node and SF in the core network.

Option-1/ GTP-U-based protocol

In this solution, sensing data is transferred between RAN node and SF via direct UP-like data tunnel, i.e., GTP-U protocol. It is similar to NG UP interface between RAN node and UPF, which is designed especially for large-volume data transfer.

However, GTP tunnel is designed for UP data transfer from/to UE. For example, for uplink UP data transfer, RAN node relies on the mapping between QoS flow and GTP tunnel to encapsulate the uplink UP data received from UE into the corresponding GTP tunnel to send to the correct UPF, where RAN node obtains the mapping between QoS flow and GTP tunnel from SMF via AMF, i.e., via NGAP PDU SESSION RESOURCE SETUP REQUEST message. If we extend the GTP-U protocol stack to transfer sensing signalling generated by RAN node without association of UE, more RAN3 effort would be needed to design the new process procedures to allow this.

Option-2/ sensing UP protocol (like LCS-UPP)

In this solution, sensing data is transferred between RAN node and SF via a new sensing UP layer, which is like LCS-UPP protocol, i.e., TS 24.572. In Rel-18 Positioning, for LPP messages and LCS supplementary services messages between UE and LMF, SA2 introduced the UP solution with a new protocol between gNB and UPF, which is called LCS-UPP and designed by SA2/CT1. We understand LCS-UPP can be taken as baseline to design new protocol for direct connection between RAN node and SF for 5G-A ISAC.

The advantage is that consistent protocol stack for data collection from UE and data collection from gNB, thus it is more future-proof for 6G when UE based sensing is developed.

Similar to LCS-UPP, the main work for this solution is in CT1/CT4 to develop a new sensing UP protocol, while the problem is no such design in CT1/CT4 for non-UE related connection, and not sure if there will be enough TU to develop a completely new protocol in CT WGs within 5G-A timeframe. Furthermore, RAN3 impact is also expected, e.g., the data connection establishment procedure carried out on CP signalling interaction such as non-UE associated NGAP signalling.

Considering to be more future-proof for 6G, we prefer sensing reporting over a new LCS-UPP-like sensing UP protocol designed by SA/CT, since this new sensing UP protocol can easily extend to also support the sensing signalling between UE and SF in a unified manner in 6G.

- RAN3 study candidate solutions to transfer sensing data via direct connection between RAN node and SF, in coordination with SA2, including
Option-1/ GTP-U-based protocol, i.e., Sensing layer/GTP-U/UDP/IP/Lower Layer;
Option-2/ new LCS-UPP-like sensing UP protocol, i.e., Sensing layer/new sensing UP layer/TLS/TCP/IP/Lower Layer.

Furthermore, regarding where to capture the sensing reporting message, in Option-1, i.e., re-use GTP-U-based protocol solution, companies propose to rely on the current PDU Session User Plane Protocol (TS 38.415), i.e., using GTP-U header to carry sensing reporting message for sensing data, without any new spec introduced.

In our understanding, if re-using PDU session UPP to carry sensing reporting message by GTP-U header, considering that significant signalling overhead for sensing measurement reporting is foreseen as mentioned above, it would lead to a huge GTP-U header instead of GTP-U payload, which is quite strange from protocol design perspective.

Besides, the introduction and design for a new kind of GTP-U header for sensing reporting would require cross-WGs interaction with CT4, and RAN3 cannot make decision on this by itself.

Therefore, we propose to introduce a new sensing layer to capture sensing reporting message.

And for Option-2, i.e., introduce new LCS-UPP-like sensing UP protocol, it is also straightforward to introduce a new sensing layer to capture sensing reporting message as well.

- RAN3 introduce new sensing layer to capture sensing reporting message, instead of relying on PDU session UPP (TS 38.415).

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

How to transfer sensing signalling between RAN and SF

- Significant signalling overhead for sensing measurement reporting is foreseen, especially when full channel state information, delay/doppler/angle profiles, or point cloud information are reported.

- If relying on AMF to forward sensing signalling over CP, AMF would be a bottleneck for heavy signalling transfer between RAN node and SF.

- Based on SA2 Rel-20 ISAC TR 23.700-14, it seems that common view of SA2 is that sensing data and the associated information is transported via direct connection between RAN node and SF in the core network.

- RAN3 study candidate solutions to transfer sensing data via direct connection between RAN node and SF, in coordination with SA2, including
Option-1/ GTP-U-based protocol, i.e., Sensing layer/GTP-U/UDP/IP/Lower Layer;
Option-2/ new LCS-UPP-like sensing UP protocol, i.e., Sensing layer/new sensing UP layer/TLS/TCP/IP/Lower Layer.

- RAN3 introduce new sensing layer to capture sensing reporting message, instead of relying on PDU session UPP (TS 38.415).

4. Reference

[1] RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR, Xiaomi, China Telecom

[2] 3GPP TR 23.700-14: “Study on Integrated Sensing and Communication; Stage 2”, V1.0.0 (2025-09)