3GPP TSG-RAN WG3 Meeting #130	R3-258336

Dallas, US, 17th – 21st November, 2025

Title:	Discussion on Cause values for Ambient IoT

Agenda Item:	9.2.7

Source:	CATT

Document for:	Discussion

- Introduction

In Rel-19 Ambient IoT WI, we defined the procedures to support Inventory and Command services. In the NGAP, Cause IE is included in the following A-IoT related Messages and IEs:

- INVENTORY FAILURE message

- COMMAND FAILURE message

- Inventory Failure Transfer IE

- Command Failure Transfer IE

- AIOT Session Release Command Transfer IE

- AIOT Session Release Request Transfer IE

However, the cause values for Ambient IoT in different cases (e.g. failure cases) are not sufficiently discussed and clearly specified.

In this contribution, we will discuss the cause values for A-IoT and provide our proposals accordingly.

- Discussion

First of all, we understand that some of the cause values as specified in the section 9.3.1.2 Cause in TS 38.413 could be applied for A-IoT, e.g. “Unspecified”, “Normal release”, “Radio resources not available”, “Failure in the radio interface procedure”.

Observation 1: some of the existing cause values in NGAP could also be applied to A-IoT, e.g. “Unspecified”, “Normal release”.

Secondly, let’s consider the A-IoT specific cases to decide whether to introduce specific cause values. We provide some analysis on a few failure/abnormal cases as below:

- Case 1: unknown RAN A-IoT Device NGAP ID

Upon receiving a Command Request from the A-IoT CN node, which includes an unknown RAN A-IoT Device NGAP ID within the indicated A-IoT session, the gNB shall fail the Command Request procedure. For this case, we understand the existing cause value “Unknown target ID” could be reused.

- Case 2: A-IoT Service Area not served by the gNB

Upon receiving the Inventory Request with either the A-IoT Area ID list and/or the Reader List, in case neither of the indicated area is served by the gNB, the gNB shall fail the Inventory Request procedure. There’s a similar case in MBS, in case the indicated MBS Service Area is not supported, gNB fails the procedure with the cause value Indicated MBS Session Area Information not served by the gNB.

For A-IoT, it’s benefical to introduce a new cause value, e.g. Indicated A-IoT Service Area not served by the gNB for this case.

- Case 3: Unknown A-IoT Session ID

As the Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure, in case the combination of the AIOTF Identifier and the A-IoT Correlation Identifier is not known by the gNB, the gNB shall fail the procedure with a specific cause value. For this case, it’s benefical to introduce a new cause value, e.g. Unknown A-IoT Session ID. The cause value could also be applied to the case when AIoT CN receives the response message from the gNB (e.g. Inventory Response/Report, Command Response) but the A-IoT Session ID is unknown, A-IoT CN may initiate the Error Indicator with the cause value.

- Case 4: Multiple A-IoT Session instances

When gNB receives an Inventory Request with the session (identified by the combination of the AIOTF Identifier and the A-IoT Correlation Identifier) which has already been established, the gNB shall fail the proceduce by a specific cause value, e.g. Multiple A-IoT Session instances.

- Case 5: No command response from device

In case the command delivery to the device is not success or the response of the device is not received by the gNB, the gNB shall send a Command Failure message to the A-IoT CN node with appropriate cause value.

Currently, the gNB could not distinguish the cases “device not reached” or “no response from device”, or “reponse from the device is not received by the NG-RAN”, or some other reasons. Thus, in this case, gNB can simply indicate the A-IoT CN with a specific cause value, e.g. No command response from the A-IoT device.

As above, it’s proposed to introduce the following cause values for A-IoT:

- Indicated A-IoT Service Area not served by the gNB

- Unknown A-IoT Session ID

- Multiple A-IoT Session instances

- No command response from the A-IoT device

- For A-IoT, introduce the following new cause values in NGAP:

      - Indicated A-IoT service area information not served by the gNB

      - Unknown A-IoT Session ID

      - Multiple A-IoT Session instances

      - No command response from A-IoT device

Corresponding CR to TS 38.413 is provided in [1], adding the above Cause values to the NGAP: Cause IE. RAN3 is requested to discuss and agree the CR.

- Discuss and agree the CR to TS 38.413 on introduction of A-IoT specific cause values.

- Proposal

In this contribution, we discussed the cause values for Ambient IoT. Based on the discussion, the following observations and proposals are provided:

Observation 1: some of the existing cause values in NGAP could also be applied to A-IoT, e.g. “Unspecified”, “Normal release”.

- For A-IoT, introduce the following new cause values in NGAP:

      - Indicated A-IoT service area information not served by the gNB

      - Unknown A-IoT Session ID

      - Multiple A-IoT Session instances

      - No command response from A-IoT device

- Discuss and agree the CR to TS 38.413 on introduction of A-IoT specific cause values.

- Reference

[1]	R3-258337 Correction to A-IoT specific cause values (TS38.413 CR1388), CATT