3GPP TSG-RAN WG3 Meeting #130	R3-258427

Dallas, US, 17 – 21 November 2025

Agenda item:	10.3.2

Source:	Nokia

Title:	6G RAN-CN Cloud-friendly P2P Solution

WID/SID:	FS_6G_Radio

Document for:	Discussion and Decision

# 1	Introduction

At RANP#108 the new SID on 6G RAN was agreed in [1] and was further revised at RAN#109 in [2].

The main architectural impacts for RAN3 are listed below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

As part of point b/, RA3 was tasked with a first milestone to achieve:

#### 4.1.1	Interim Milestone

Interim results shall be delivered as per the milestones below, in coordination with the RAN Plenary 6G Study [RP-250810].

TSG#112 (June/2026):

RAN3 to provide interim study results to allow TSGs to make a decision on:

- RAN-CN interface: P2P vs SBI

- RAN internal interfaces: CU-DU split, CP-UP split.

Tdoc [3] has derived the key requirements and associated criteria to select the RAN-CN CP interface solution.

This paper proposes our solution for the section 6.1.3 of the TR with:

- a first part concerning the Control Plane of the RAN-CN interface for which the interim milestone applies, and

- a second part related to the User Plane of the RAN-CN interface.

# 2	Discussion

The RAN-CN interface solution proposed is an enhanced Cloud-friendly P2P (point to point) Control plane interface between the 6G RAN node and the 6G AMF node. Other planes that may be used for transfer of data related to new services is open at this stage, to be defined together with SA2.

Architecture

The architecture corresponding to the Cloud-friendly P2P solution can be illustrated at high level by the following figure:

Figure 1: Cloud-friendly P2P solution

In the perspective of the first milestone of June 2026 of P2P vs SBI comparison, the following section focuses in priority on Control Plane aspects.

- 6G RAN-CN Control plane Protocol Stack

The RAN-CN Control Plane interface solution proposed here builds up from the 5G RAN-CN interface by reusing the P2P (point to point) interface between the 6G RAN node and the 6G AMF node with 6GAP-based application protocol built on top of QUIC/UDP/IP which provides a modern and cloud-friendly transport protocol stack widely available in data centers. The proposal’s protocol stack is illustrated below:

Key characteristics of the P2P

The key features of the P2P are:

1/ The cloud-friendly P2P solution minimizes implementation and deployment costs

The SA2 study item in [4] assumes that the RAN-CN functional split is maintained:

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1: The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

Considering that the same split applies between RAN functions and Core functions, by reusing a P2P framework on the RAN-CN interface like in 5G, the solution minimizes the impact on the application protocol when evolving towards 6GAP, thus minimizing the implementation complexity and cost.

2/ The cloud-friendly P2P solution enables smooth and easy migration

With a P2P RAN-CN 6G interface the RAN node just exchanges signalling with one CN node and does not have to coordinate directly with different types of core network functions. This simplifies the RAN node logic which doesn’t have to manage collisions in the downlink between competing requests from multiple core network functions or implement different failure handling depending on the core network function it is interacting with e.g. in case of internal radio events like handovers. Similarly for the uplink it doesn’t require new UE configuration to prioritize the uplink traffic.

It allows reuse of nodal behaviour and proven failure handling logic developed and tested for the RAN-CN 5G interface, thereby making the migration easy and smooth and minimizing both implementation and deployment costs.

3/ The cloud-friendly P2P solution involves minimum standardization effort

For the same reason, by reusing the P2P framework and minimizing the impact on the application protocol when evolving towards 6GAP this solution also minimizes the effort spent by 3GPP to standardize the 6GAP layer.

4/ The cloud-friendly P2P solution facilitates the interoperability and limits testing efforts

The P2P framework of the solution facilitates the interoperability in multi-vendor context because the 6G RAN node connects to a single Core Network node even when involved in end-to-end services with other entities in the core network. This enables the tests to be concentrated on a single interface and limit the IOT failure cases to this interface.

As a result, this solution will minimize the testing efforts while minimizing the IOT issues when the new generation of 6G nodes are being deployed. In general, it will help minimize the testing efforts leading to OPEX savings.

5/ The cloud-friendly P2P solution minimizes signaling overhead

Besides, this solution based on P2P keeps the amount of control plane signalling comparable to 5G RAN-CN signalling, maintaining the level of traffic and avoiding performance degradation.

Key characteristics of QUIC

The key features provided by QUIC are:

1/ QUIC offers enhanced and performant Security

QUIC natively integrates most features of the TLS v1.3 security protocol, so encryption moves down from the application layer to the transport layer. Moreover, packet numbers, acknowledgements, and other functional parts in QUIC header are encrypted to enhance security.

By providing native encryption between two endpoints, deployment of IPSEC tunnels for the control plane is not needed.

Moreover, 6GAP over QUIC will provide enhanced ciphering performance than SCTP with DTLS. For example, QUIC only needs one round-trip to establish a secure connection between the client and server leading to fast connection setup time.

2/ QUIC offers Resiliency and Multi-homing

Resilience to packet drops: QUIC does not suffer from Head-of-Line (HoL) blocking issue which causes long downtime and performance degradation in cases of retransmission due to dropped packets..

Native multi-homing and resiliency to path failure: QUIC supports useful header extensions such as MP-QUIC where data is transmitted simultaneously in multiple paths by adding Path ID to the QUIC header. This allows dynamic and efficient message routing, cloud native scaling, load balancing, and enables multi-homing and failure recovery mechanisms.

3/ QUIC minimizes implementation and deployment costs

The solution tunnels QUIC over UDP (as shown in the protocol stack above) to enable native support, especially with legacy hardware and software. Hence, the P2P over QUIC/UDP layer can be carried over any installed base of transport network equipment of an operator and minimize the CAPEX.

Finally, in contrast to SCTP which was mostly developed for 3GPP, QUIC is a widely supported transport protocol. It is gaining strong momentum in the community and in standardization. The P2P over QUIC proposal will hence take advantage of new features and extensions as they are being introduced without cost increase.

4/ QUIC offers increased modularity and scalability

QUIC enables simplified and more efficient routing of the messages relying on QUIC Connection ID (CID)-based mechanisms. This enables hiding the RAN internal architecture from the control plane message routing, which helps resolving the scalability issues, especially in cloud deployments.

If utilized by Transport Network Layer Association (TNLA) principle, QUIC CIDs provide more freedom and dynamicity (e.g. network path migration) for connection control than the current semi-static association with the IP/port pairs. It is also worth noting that QUIC provides efficient support for Network Address Translation (NAT) with header modification.

Proposal 1: Capture the Cloud-friendly P2P Solution” solution in the section 6.1.3 of the TR

- 6G RAN-CN User plane Protocol Stack

The handling of the RAN–CN (N3) user plane protocol stack requires further discussion. CT4 is currently initiating a Study Item on the corresponding user plane protocol stack for the N3 and N9 interface, and their study will take place in parallel with the ongoing RAN3 6G study. In order to avoid duplication of work and ensure consistency, a coordinated process should be established.

As in 5G, the user plane protocol stack is expected to be aligned between the N3 and N9 interfaces. Since RAN3 currently has no visibility of the requirements applicable to the N9 interface, it is proposed that CT4 first progresses their study and, once preliminary conclusions are available, they are shared with RAN3.

In accordance with the respective Terms of Reference, RAN3 would then consider the CT4 recommendations and take them into account when finalizing the RAN–CN user plane protocol stack aspects.

Proposal 2: Study of the RAN–CN (N3) user plane protocol stack will be put on hold in RAN3 until CT4 has progressed their N3/N9 user plane study and provided preliminary conclusions to RAN3.

A corresponding section will be included in the RAN3 study TR to reflect this agreed process and to capture the final selection once CT4 results become available.

Proposal 3: Create a section 6.1.x RAN-CN User Plane in the TR 38.760-3 with relevant editor’s note corresponding to the status.

# 3	Conclusion

For the 6G RAN-CN Control plane study, this paper has proposed a solution called “Cloud friendly P2P solution” based on QUIC which is identified as cumulating the advantages of the P2P approach currently used over NG interface and adopting a widely used and supported transport protocol, bringing more advantages in terms of performance, security, resilience and cloud friendliness.

Proposal 1: Capture the Cloud-friendly P2P Solution” solution in the section 6.1.3 of the TR

For the 6G RAN-CN User plane study, the following way forward is proposed:

Proposal 2: Study of the RAN–CN (N3) user plane protocol stack will be put on hold in RAN3 until CT4 has progressed their N3/N9 user plane study and provided preliminary conclusions to RAN3.

Proposal 3: Create a section 6.1.x RAN-CN User Plane in the TR 38.760-3 with relevant editor’s note corresponding to the status.

# 4	References

- RP-251881, 6G RAN study

- RP-252912, 6G RAN Study (revision of RAN#109)

- R3-256886, 6G RAN-CN Interface Requirements and Principles

- SP-250806, Study on Architecture for 6G system

# pCR for TR 38.760-3

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 6.1.3	RAN-CN Interface Options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

### 6.1.3.x	Cloud-friendly P2P solution with QUIC

The RAN-CN control plane interface solution proposed here builds up from the 5G RAN-CN interface by reusing the P2P (point to point) interface between the 6G RAN node and the 6G AMF node with 6GAP-based application protocol built on top of QUIC/UDP/IP which provides a modern and cloud-friendly transport protocol stack widely available in data centers. The proposal’s protocol stack is illustrated below:

The key features of the solution in details are:

Key characteristics of the P2P

The key features of the P2P are:

1/ The cloud-friendly P2P solution minimizes implementation and deployment costs

The SA2 study item in [4] assumes that the RAN-CN functional split is maintained:

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1: The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

Considering that the same split applies between RAN functions and Core functions, by reusing a P2P framework on the RAN-CN interface like in 5G, the solution minimizes the impact on the application protocol when evolving towards 6GAP, thus minimizing the implementation complexity and cost.

2/ The cloud-friendly P2P solution enables smooth and easy migration

With a P2P RAN-CN 6G interface the RAN node just exchanges signalling with one CN node and does not have to coordinate directly with different types of core network functions. This simplifies the RAN node logic which doesn’t have to manage collisions in the downlink between competing requests from multiple core network functions or implement different failure handling depending on the core network function it is interacting with e.g. in case of internal radio events like handovers. Similarly for the uplink it doesn’t require new UE configuration to prioritize the uplink traffic.

It allows reuse of nodal behaviour and proven failure handling logic developed and tested for the RAN-CN 5G interface, thereby making the migration easy and smooth and minimizing both implementation and deployment costs.

3/ The cloud-friendly P2P solution involves minimum standardization effort

For the same reason, by reusing the P2P framework and minimizing the impact on the application protocol when evolving towards 6GAP this solution also minimizes the effort spent by 3GPP to standardize the 6GAP layer.

4/ The cloud-friendly P2P solution facilitates the interoperability and limits testing efforts

The P2P framework of the solution facilitates the interoperability in multi-vendor context because the 6G RAN node connects to a single Core Network node even when involved in end-to-end services with other entities in the core network. This enables the tests to be concentrated on a single interface and limit the IOT failure cases to this interface.

As a result, this solution will minimize the testing efforts while minimizing the IOT issues when the new generation of 6G nodes are being deployed. In general, it will help minimize the testing efforts leading to OPEX savings.

5/ The cloud-friendly P2P solution minimizes signaling overhead

Besides, this solution based on P2P keeps the amount of control plane signalling comparable to 5G RAN-CN signalling, maintaining the level of traffic and avoiding performance degradation.

Key characteristics of QUIC

The key features provided by QUIC are:

1/ QUIC offers enhanced and performant Security

QUIC natively integrates most features of the TLS v1.3 security protocol, so encryption moves down from the application layer to the transport layer. Moreover, packet numbers, acknowledgements, and other functional parts in QUIC header are encrypted to enhance security.

By providing native encryption between two endpoints, deployment of IPSEC tunnels for the control plane is not needed.

Moreover, 6GAP over QUIC will provide enhanced ciphering performance than SCTP with DTLS. For example, QUIC only needs one round-trip to establish a secure connection between the client and server leading to fast connection setup time.

2/ QUIC offers Resiliency and Multi-homing

Resilience to packet drops: QUIC does not suffer from Head-of-Line (HoL) blocking issue which causes long downtime and performance degradation in cases of retransmission due to dropped packets..

Native multi-homing and resiliency to path failure: QUIC supports useful header extensions such as MP-QUIC where data is transmitted simultaneously in multiple paths by adding Path ID to the QUIC header. This allows dynamic and efficient message routing, cloud native scaling, load balancing, and enables multi-homing and failure recovery mechanisms.

3/ QUIC minimizes implementation and deployment costs

The solution tunnels QUIC over UDP (as shown in the protocol stack above) to enable native support, especially with legacy hardware and software. Hence, the P2P over QUIC/UDP layer can be carried over any installed base of transport network equipment of an operator and minimize the CAPEX.

Finally, in contrast to SCTP which was mostly developed for 3GPP, QUIC is a widely supported transport protocol. It is gaining strong momentum in the community and in standardization. The P2P over QUIC proposal will hence take advantage of new features and extensions as they are being introduced without cost increase.

4/ QUIC offers increased modularity and scalability

QUIC enables simplified and more efficient routing of the messages relying on QUIC Connection ID (CID)-based mechanisms. This enables hiding the RAN internal architecture from the control plane message routing, which helps resolving the scalability issues, especially in cloud deployments.

If utilized by Transport Network Layer Association (TNLA) principle, QUIC CIDs provide more freedom and dynamicity (e.g. network path migration) for connection control than the current semi-static association with the IP/port pairs. It is also worth noting that QUIC provides efficient support for Network Address Translation (NAT) with header modification.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 6.1.x	RAN-CN User Plane

Editor’s note 1: The aim of this section is to describe the user plane protocol stack for the RAN-CN interface.

Editor’s note 2: On hold waiting for CT4 input.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>