3GPP TSG-RAN WG3 #131                                                 R3-260315

Goteborg, Sweden, 9th – 13th Feb, 2026

Agenda Item:	11.2.2

Source:	CATT

Title:	(TP for 38.473) Discussion on SON enhancement for conditional intra-CU LTM

Document for:	Discussion

Introduction

In last RAN3 meeting, the following agreements and FFS have been achieved:

Failure causes that should be optimized for intra-CU CLTM,

1.Inappropriate CLTM execution conditions (L3 or L1)

2. Inappropriate CLTM candidate cell list configured by CU

Whether cell not accepted by DU should be optimized is FFS

3. Inappropriate configuration on CLTM TAT

It is a common understanding that in intra-CU CLTM, optimizing beam selection means optimizing CLTM execution conditions/CLTM candidate cells.

Remove FFS whether both L1 and L3 apply to beam related selection optimization.

R19 use case from LTM (beam and TA related):

UE-based solution is used for outdated TA for intra-CU CLTM.

Current network-based solution for BFR shortly after successful CLTM cell switch is also applied to intra-CU CLTM?

UE-based solution is used for wrong beam selection for intra-CU CLTM?

LTM connection failure (too late, wrong cell, too early):

CLTM type: It indicates the last executed handover type is CLTM before the last detected connection failure.

-The last execution condition which was met: L1, L3 or neither.

-The first execution condition which was met: L1 or L3.

-The execution conditions which were met, which condition trigger the CLTM handover, apply to both L1 and L3

-Cell ID for which the execution is performed.

-Cell list for which the execution conditions are fulfilled.

-CLTM Time alignment timer remaining time.

-Time elapsed from CLTM cell switch execution to failure.

-Time from adding execution condition (upon receiving RRCReconfiguration message) to failure.

-Time when the execution was performed.

CLTM RACH-less access failure indication: It indicates whether a RACH-less access failure happens before the successful RACH-based access to the same target cell.

-The SSB index of the beam used in CLTM re-establishment or CLTM recovery

-The difference between TA value used in the failed CLTM and the TA value received in RAR (or msgB) at CLTM re-establishment

Near failure case: SHR successful HO:

Successful case: ping/pong:

CU sends Last Visited LTM Cell list to the DU in UE CONTEXT MODIFCATION REQUEST message.

Here we provide some analysis on MRO enhancement for conditional intra-CU LTM.

Discussion

2.1 BFR/wrong beam/outdated TA cases

In R19, network solution is introduced to support MRO on the cases: BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA. But for C-LTM, the FFSs are captured below:

UE-based solution is used for outdated TA for intra-CU CLTM.

Current network-based solution for BFR shortly after successful CLTM cell switch is also applied to intra-CU CLTM?

UE-based solution is used for wrong beam selection for intra-CU CLTM?

For legacy LTM, it is always source DU to perform optimization for above cases because it makes all the decision.

For C-LTM, considering L1 and L3 triggered CLTM, the related nodes are discussed as below:

CU is responsible for the following configuration:

- Candidate cell selection.

- Execution condition in case of L3 triggered LTM.

- Initiating Early TA acquisition procedure based on L3 measurement report.

Target DU in each C-LTM is responsible for the following configuration:

- TAT.

- Candidate beam selection for both L1 and L3 triggered LTM.

Source DU is responsible for the following configuration:

- Execution condition in case of L1 triggered LTM.

- Initiating early TA acquisition based on L1 measurement report.

Observation 1: For C-LTM, source DU, target DU and CU are responsible for above configurations which is different from legacy LTM.

In R19, target DU perform initial analysis based on CU-DU/DU-CU CELL SWITCH NOTIFICATION message. While for C-LTM, without CU-DU/DU-CU CELL SWITCH NOTIFICATION message, target DU is not aware of the following information:

- The time UE triggers the initiation of C-LTM.

- The target cell and beam toward which UE triggers C-LTM.

- After receiving RRCReconfigurationComplete message, target DU cannot detect successful handover access or recovery access.

- TA information for target beam.

Observation 2: In case of C-LTM, without CU-DU/DU-CU CELL SWITCH NOTIFICATION message target DU is not aware of the following information:

- The time UE triggers the initiation of C-LTM.

- The target cell and beam toward which UE triggers C-LTM.

- After receiving RRCReconfigurationComplete message, target DU cannot detect successful handover access or recovery access.

- TA information for target beam.

Wrong beam case

Without above information, for failure due to wrong beam, target DU cannot detect whether failure occurs. Consequently, the target DU may incorrectly send a successful TA response to the source DU in all cases, making the existing network-based solution redundant for C-LTM.

Proposal 1: It is proposed to use UE based solution for wrong beam case. It is up to RAN2 to decide what information is needed.

Outdated TA case

Similarly, for outdate TA case, one possible solution is to allow target DU to always send successful TA back to source DU and let source DU judge whether failure due to outdated TA. It may not a good approach because target DU always sending TA for each C-LTM cause too much overload in F1. So, UE based solution is preferred.

Proposal 2: It is proposed to use UE based solution for outdated TA. It is up to RAN2 to decide what information is needed.

It should be noted that the TA optimization is different from legacy. For intra-CU LTM in R19 SON, source DU will receive TA used at successful Random Access during LTM recovery or re-establishment, and source DU may use it directly or implicitly indicate the outdate TA. However, for C-LTM, whether TA is valid depends on TAT configured by target DU. It obvious target DU is responsible for outdated TA optimization. For source DU, it also can trigger early TA even if within TAT but it fully dependent on source DU implementation.

Proposal 3: For outdated TA, both source and target DU are responsible for optimization on TA configuration.

BFR shortly after successful C-LTM

For BFR shortly after successful C-LTM cell switch execution case, following issues may need further discussion.

If proposal 1, UE based solution, is agreed, wrong beam information introduced in RLF Report for wrong beam case may be reused for BFR shortly after successful C-LTM cell switch execution case. No additional network based solution is needed. It is up to RAN2 to decide what information can be added in RLF report for UE based solution.

Proposal 4: UE based solution for wrong beam case can be reused for BRF shortly case as much as possible, it is up to RAN2 to decide what information is needed.

2.2 Enhancement on RLF Report

For 	LTM connection Failures (too late, wrong cell, too early), RLF Report should be enhanced. The following info has been agreed in RLF Report:

CLTM type: It indicates the last executed handover type is CLTM before the last detected connection failure.

Following is FFS captured in last RAN3 meeting:

-The last execution condition which was met: L1, L3 or neither.

-The first execution condition which was met: L1 or L3.

-The execution conditions which were met, which condition trigger the CLTM handover, apply to both L1 and L3

-Cell ID for which the execution is performed.

-Cell list for which the execution conditions are fulfilled.

CLTM Time alignment timer remaining time.

-Time elapsed from CLTM cell switch execution to failure.

-Time from adding execution condition (upon receiving RRCReconfiguration message) to failure.

-Time when the execution was performed.

-CLTM RACH-less access failure indication: It indicates whether a RACH-less access failure happens before the successful RACH-based access to the same target cell.

-The SSB index of the beam used in CLTM re-establishment or CLTM recovery

-The difference between TA value used in the failed CLTM and the TA value received in RAR (or msgB) at CLTM re-establishment

| index | infor | reason | conclusion |
| --- | --- | --- | --- |
| 1 | Cell ID for which the execution is performed | Reuse legacy failedPCellId IE to record the target PCell of the failed handover. | Needed and reuse |
| 2 | Cell list for which the execution conditions are fulfilled | This info can be deduced from measResultL1 of serving cell and neighcells and LTM candidate cell list recorded by network. | Not needed |
| 3 | The first execution condition which was met: L1 or L3. | For each candidate cell, the related execution condition may be L1 or L3. And network can record all these configuration information. With above failedPCellId IE，network can deduce whether L1 or L3 C-LTM is performed. | Not needed |
| 4 | The last execution condition which was met: L1, L3 or neither. | When first execution condition was met, execution should be performed while other execution condition cannot be met after that. So, there is only one execution condition i.e., L1 C-TM or L3 C-LTM for one candidate cell, no last execution condition which was met. | Not needed |
| 5 | The execution conditions which were met, which condition trigger the CLTM handover, apply to both L1 and L3 | For each candidate cell in L1 C-LTM, the related execution condition may be L1(eventLTM3-r19 or eventLTM5-r19) or L3. Currently it is not support to configure both L1 and L3 to one candidate cell. With above failedPCellId, network can deduce whether L1 or L3 is configured or meet the condition. For L1 C-LTM, more specifically, eventLTM3-r19 or eventLTM5-r19 can be also deduced by network. | Not needed |
| 6 | Time elapsed from CLTM cell switch execution to failure. | Reuse legacy timeConnFailure IE | Needed and reuse |
| 7 | Time from adding execution condition (upon receiving RRCReconfiguration message) to failure. | Reuse legacy timeSinceCHO-Reconfig IE with necessary description update | Needed and reuse with necessary description update. |
| 8 | Time when the execution was performed | The same as index 6. | Needed and reuse |
| 9 | CLTM RACH-less access failure indication | Absent of RA-Information in RLF Report can indicate RACH-less access implicitly. | Not needed |

For too late/too early/to wrong cell cases, RAN3 can discuss what information is needed in RLF report.

Proposal 5: RAN3 consider above info in RLF Report for LTM connection Failures (too late, to wrong cell, too early).

2.3 Successful case: ping/pong

CU sends Last Visited LTM Cell list to the DU in UE CONTEXT MODIFCATION REQUEST message.

In last meeting, we agreed to add Last Visited LTM Cell list to the DU in UE CONTEXT MODIFCATION REQUEST message. After each cell switch, CU sends UHI to source DU but there is no TP was agreed.

Proposal 6: Capture Last Visited LTM Cell list in UE CONTEXT MODIFCATION REQUEST message for C-LTM in TP, as Annex.

Conclusions

Proposal 1: It is proposed to use UE based solution for wrong beam case. It is up to RAN2 to decide what information is needed.

Proposal 2: It is proposed to use UE based solution for outdated TA. It is up to RAN2 to decide what information is needed.

Proposal 3: For outdated TA, both source and target DU are responsible for optimization on TA configuration.

Proposal 4: UE based solution for wrong beam case can be reused for BRF shortly case as much as possible, it is up to RAN2 to decide what information is needed.

Proposal 5: RAN3 consider above info in RLF Report for LTM connection Failures (too late, to wrong cell, too early).

Proposal 6: Capture Last Visited LTM Cell list in UE CONTEXT MODIFCATION REQUEST message for C-LTM in TP, as Annex.

Reference

[1]

Annex TP for 38.473

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

- 8.3.4	UE Context Modification (gNB-CU initiated)

- 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

- 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

- Unchanged part is omitted

For each DRB that has been successfully established or modified and for which the Performance Delay Monitoring IE was included in the DRB to Be Setup List IE or in the DRB to Be Modified List IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store this information and use it to perform or update delay measurements on the successfully established or modified DRBs.

If the Last Visited LTM Cells IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU may take this into account to resolve the potential mobility problems (e.g. ping-pong) detected by the gNB-CU.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

For each GBR QoS flow whose DRB has been successfully established or modified and the Monitoring Request on Available Bitrate IE was included in the GBR QoS Flow Information IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store this information and, perform available bitrate monitoring, as specified in TS 23.501 [21].

For each QoS flow whose DRB to be established or modified, if the MMSID IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the QoS flow is related to a multi-modal service, as described in TS 23.501 [21] and TS 38.300 [6].

If the Indication of Bitrate Adaptation IE set to "uplink" is included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store the received indication and use it as defined in TS 38.300 [6].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for conditional LTM and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for conditional LTM. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE and may include the TA Remaining Information List IE in the UE CONTEXT MODIFICATION RESPONSE message.

- <<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

- 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| - Unchanged part is omitted |  |  |  |  |  |  |
| LTM TCI States Configurations List |  | 0..1 |  |  | YES | reject |
| >LTM TCI States Configurations Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| LTM Security Information | O |  | 9.3.1.359 |  | YES | reject |
| LTM Information SN Modification |  | 0..1 |  |  | YES | reject |
| >LTM with SCG Indicator | M |  | ENUMERATED(true, ...) |  | – |  |
| Last Visited LTM Cells | O |  | 9.3.1.358 |  | YES | ignore |

- <<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

- 9.4.4	PDU Definitions

-- **************************************************************

--

#### -- UE Context Modification ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-SpCell-ID								CRITICALITY ignore	TYPE NRCGI												PRESENCE optional	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE optional	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE optional	}|

{ ID id-TransmissionActionIndicator				CRITICALITY ignore	TYPE TransmissionActionIndicator					PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-RRCReconfigurationCompleteIndicator		CRITICALITY ignore	TYPE RRCReconfigurationCompleteIndicator			PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY reject	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-SCell-ToBeSetupMod-List					CRITICALITY ignore	TYPE SCell-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-SCell-ToBeRemoved-List					CRITICALITY ignore	TYPE SCell-ToBeRemoved-List 							PRESENCE optional }|

{ ID id-SRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE SRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE DRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeModified-List					CRITICALITY reject	TYPE DRBs-ToBeModified-List							PRESENCE optional	}|

{ ID id-SRBs-ToBeReleased-List					CRITICALITY reject	TYPE SRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeReleased-List					CRITICALITY reject	TYPE DRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-DRXConfigurationIndicator				CRITICALITY ignore	TYPE DRXConfigurationIndicator						PRESENCE optional	}|

{ ID id-RLCFailureIndication					CRITICALITY ignore	TYPE RLCFailureIndication								PRESENCE optional	}|

{ ID id-UplinkTxDirectCurrentListInformation	CRITICALITY ignore	TYPE UplinkTxDirectCurrentListInformation		PRESENCE optional	}|

{ ID id-GNB-DUConfigurationQuery				CRITICALITY reject	TYPE GNB-DUConfigurationQuery						PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE optional	}|

{ ID id-ExecuteDuplication						CRITICALITY ignore	TYPE ExecuteDuplication									PRESENCE optional}|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest						PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-NeedforGap								CRITICALITY ignore	TYPE NeedforGap											PRESENCE optional	}|

{ ID id-FullConfiguration						CRITICALITY reject	TYPE FullConfiguration									PRESENCE optional	}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex						PRESENCE optional }|

{ ID id-LowerLayerPresenceStatusChange			CRITICALITY ignore	TYPE LowerLayerPresenceStatusChange				PRESENCE optional	}|

{ ID id-BHChannels-ToBeSetupMod-List			CRITICALITY reject	TYPE BHChannels-ToBeSetupMod-List					PRESENCE optional	}|

{ ID id-BHChannels-ToBeModified-List			CRITICALITY reject	TYPE BHChannels-ToBeModified-List					PRESENCE optional	}|

{ ID id-BHChannels-ToBeReleased-List			CRITICALITY reject	TYPE BHChannels-ToBeReleased-List					PRESENCE optional	}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized						PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-PC5LinkAMBR								CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-SLDRBs-ToBeSetupMod-List				CRITICALITY reject	TYPE SLDRBs-ToBeSetupMod-List						PRESENCE optional	}|

{ ID id-SLDRBs-ToBeModified-List				CRITICALITY reject	TYPE SLDRBs-ToBeModified-List						PRESENCE optional	}|

{ ID id-SLDRBs-ToBeReleased-List				CRITICALITY reject	TYPE SLDRBs-ToBeReleased-List						PRESENCE optional	}|

{ ID id-ConditionalIntraDUMobilityInformation	CRITICALITY reject	TYPE ConditionalIntraDUMobilityInformation		PRESENCE optional}|

{ ID id-F1CTransferPath							CRITICALITY reject	TYPE F1CTransferPath									PRESENCE optional }|

{ ID id-SCGIndicator							CRITICALITY ignore	TYPE SCGIndicator										PRESENCE optional	}|

{ ID id-UplinkTxDirectCurrentTwoCarrierListInfo		CRITICALITY ignore	TYPE UplinkTxDirectCurrentTwoCarrierListInfo	PRESENCE optional	}|

{ ID id-IABConditionalRRCMessageDeliveryIndication				CRITICALITY reject	TYPE IABConditionalRRCMessageDeliveryIndication							PRESENCE optional	}|

{ ID id-F1CTransferPathNRDC							CRITICALITY reject	TYPE F1CTransferPathNRDC								PRESENCE optional }|

{ ID id-MDTPollutedMeasurementIndicator				CRITICALITY ignore	TYPE MDTPollutedMeasurementIndicator				PRESENCE optional }|

{ ID id-SCGActivationRequest						CRITICALITY ignore	TYPE SCGActivationRequest							PRESENCE optional }|

{ ID id-CG-SDTQueryIndication						CRITICALITY ignore	TYPE CG-SDTQueryIndication							PRESENCE optional	}|

{ ID id-FiveG-ProSeAuthorized						CRITICALITY ignore	TYPE FiveG-ProSeAuthorized							PRESENCE optional }|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-FiveG-ProSePC5LinkAMBR						CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-UpdatedRemoteUELocalID						CRITICALITY ignore	TYPE RemoteUELocalID				 					PRESENCE optional	}|

{ ID id-UuRLCChannelToBeSetupList					CRITICALITY reject	TYPE UuRLCChannelToBeSetupList						PRESENCE optional}|

{ ID id-UuRLCChannelToBeModifiedList				CRITICALITY reject	TYPE UuRLCChannelToBeModifiedList					PRESENCE optional}|

{ ID id-UuRLCChannelToBeReleasedList				CRITICALITY reject	TYPE UuRLCChannelToBeReleasedList					PRESENCE optional}|

{ ID id-PC5RLCChannelToBeSetupList					CRITICALITY reject	TYPE PC5RLCChannelToBeSetupList					PRESENCE optional}|

{ ID id-PC5RLCChannelToBeModifiedList				CRITICALITY reject	TYPE PC5RLCChannelToBeModifiedList				PRESENCE optional}|

{ ID id-PC5RLCChannelToBeReleasedList				CRITICALITY reject	TYPE PC5RLCChannelToBeReleasedList				PRESENCE optional}|

{ ID id-PathSwitchConfiguration						CRITICALITY ignore	TYPE PathSwitchConfiguration				 		PRESENCE optional	}|

{ ID id-GNBDUUESliceMaximumBitRateList				CRITICALITY ignore	TYPE GNBDUUESliceMaximumBitRateList				PRESENCE optional }|

{ ID id-MulticastMBSSessionSetupList				CRITICALITY reject	TYPE MulticastMBSSessionList						PRESENCE optional }|

{ ID id-MulticastMBSSessionRemoveList				CRITICALITY reject	TYPE MulticastMBSSessionList						PRESENCE optional }|

{ ID id-UE-MulticastMRBs-ToBeSetup-atModify-List	CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeSetup-atModify-List	PRESENCE optional	}|

{ ID id-UE-MulticastMRBs-ToBeReleased-List			CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeReleased-List			PRESENCE optional	}|

{ ID id-SLDRXCycleList								CRITICALITY ignore  TYPE SLDRXCycleList 										PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNModificationList		CRITICALITY ignore	TYPE MDTPLMNModificationList						PRESENCE optional }|

{ ID id-SDTBearerConfigurationQueryIndication		CRITICALITY ignore	TYPE SDTBearerConfigurationQueryIndication		PRESENCE optional }|

{ ID id-DAPS-HO-Status								CRITICALITY ignore	TYPE DAPS-HO-Status										PRESENCE optional }|

{ ID id-ServingCellMO-List							CRITICALITY ignore	TYPE ServingCellMO-List								PRESENCE optional	}|

{ ID id-UlTxDirectCurrentMoreCarrierInformation	    CRITICALITY ignore	TYPE UlTxDirectCurrentMoreCarrierInformation	PRESENCE optional	}|

{ ID id-CPACMCGInformation							CRITICALITY ignore	TYPE CPACMCGInformation								PRESENCE optional }|

{ ID id-NetworkControlledRepeaterAuthorized			CRITICALITY ignore	TYPE NetworkControlledRepeaterAuthorized			PRESENCE optional	}|

{ ID id-SDT-Volume-Threshold							CRITICALITY ignore	TYPE SDT-Volume-Threshold								PRESENCE optional  }|

{ ID id-LTMInformation-Modify						CRITICALITY reject	TYPE LTMInformation-Modify							PRESENCE optional	}|

{ ID id-LTMCFRAResourceConfig-List					CRITICALITY ignore	TYPE LTMCFRAResourceConfig-List					PRESENCE optional	}|

{ ID id-LTMConfigurationIDMappingList				CRITICALITY reject	TYPE LTMConfigurationIDMappingList				PRESENCE optional	}|

{ ID id-EarlySyncInformation-Request				CRITICALITY ignore	TYPE EarlySyncInformation-Request					PRESENCE optional	}|

{ ID id-EarlySyncCandidateCellInformation-List		CRITICALITY ignore	TYPE EarlySyncCandidateCellInformation-List		PRESENCE optional	}|

{ ID id-EarlySyncServingCellInformation				CRITICALITY ignore	TYPE EarlySyncServingCellInformation				PRESENCE optional }|

{ ID id-LTMCells-ToBeReleased-List					CRITICALITY reject	TYPE LTMCells-ToBeReleased-List					PRESENCE optional	}|

{ ID id-PathAdditionInformation					CRITICALITY reject	TYPE PathAdditionInformation								PRESENCE optional}|

{ ID id-NRA2XServicesAuthorized						CRITICALITY ignore	TYPE NRA2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEA2XServicesAuthorized						CRITICALITY ignore	TYPE LTEA2XServicesAuthorized							PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrateForA2X		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitrateForA2X	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-DLLBTFailureInformationRequest				CRITICALITY ignore	TYPE DLLBTFailureInformationRequest				PRESENCE optional }|

{ ID id-SLPositioning-Ranging-Service-Info		CRITICALITY ignore	TYPE SLPositioning-Ranging-Service-Info						PRESENCE optional	}|

{ ID id-NonIntegerDRXCycle						CRITICALITY ignore	TYPE NonIntegerDRXCycle									PRESENCE optional	}|

{ ID id-LTMResetInformation						CRITICALITY ignore	TYPE LTMResetInformation								PRESENCE optional	}|

{ ID id-LTMTCIStatesConfigurationsList			CRITICALITY reject	TYPE LTMTCIStatesConfigurationsList				PRESENCE optional	}|

{ ID id-LTMSecurityInformation					CRITICALITY reject	TYPE LTMSecurityInformation							PRESENCE optional	}|

{ ID id-LTMInformationSCGMod					CRITICALITY reject	TYPE LTMInformationSCGMod								PRESENCE optional	}|

{ ID id-LastVisitedLTMCells					CRITICALITY ignore   	TYPE LastVisitedLTMCells								PRESENCE optional	},

...

}

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>