3GPP TSG-RAN WG3 Meeting #130	R3-258784

Dallas, USA, 17 – 21 Nov. 2025

Agenda item:		14.2

Source:		NEC, Huawei

Title:		(TP to 38.300 BL CR) Topology 2 Call Flows

Document for:		Agreement

- Introduction

In this contribution, we introduce the Topology 2 RRC-based solution call flows based on the corresponding call flows in TR 38.769.

The AIOT procedures (Topology 2, RRC-based solution) in TR 38.769 can be as baseline.

- TP to TS 38.300

<<<<<<<<<<<< Start of Change>>>>>>>>>>>>

### 16.23.6	Inventory Procedure

#### 16.23.6.1 Topology 1

Figure 16.23.6-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure for Topology 1.

Figure 16.23.6-1: Inventory procedure for Topology 1

<<<<<<<<<<<< Omit unrelated part >>>>>>>>>>>>

#### 16.23.6.2 Topology 2

Figure 16.23.6-2 depicts the basic communication between the UE, gNB and the A-IoT CN node for the Inventory procedure for Topology 2.

Figure 16.23.6-2: Inventory procedure for topology 2

1. The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB.

2. The gNB allocates A-IoT radio resources for the UE to perform A-IoT operation.

Editor’s Note: FFS on the details, depending on RAN2.

3. A-IoT specific RRC signalling takes place (FFS) between the UE reader and the gNB.

Editor’s Note: Further details depending on RAN2.

4. The gNB sends an Inventory Response message to the A-IoT CN.

5. The Inventory operation over the A-IoT radio interface is performed.

6/7. The gNB sends the inventory report received from the UE towards the A-IoT CN via Inventory Report message(s).

8.	If there is no follow-on command, the A-IOT CN triggers the A-IoT CN node initiated A-IoT Session Release procedure as defined in clause 16.23.8.

### 16.23.7	Command Procedure

#### 16.23.7.1 Topology 1

Figure 16.23.7-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure for Topology 1.

Figure 16.23.7-1: Command procedure for Topology 1

<<<<<<<<<<<< Omit unrelated part >>>>>>>>>>>>

#### 16.23.7.2 Topology 2

Figure 16.23.7-2 depicts the basic communication between the UE, gNB and the A-IoT CN node for the Command procedure for Topology 2.

Figure 16.23.7-2: Command procedure for Topology 2

The Command procedure addresses only one A-IoT device.

1. Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices as specified in Figure 16.23.6.2 in step 1 to 8.

2. The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB in order to send a command to a single A-IoT device via the UE.

3. The gNB allocates A-IoT radio resources to the UE for A-IoT operation.

Editor‘s Note: Resource allocation depending on RAN1/2.

4. A-IoT specific RRC signalling takes place (FFS)

Editor‘s Note: Further depending on RAN2.

5. The UE performs the Command operation over the A-IoT radio interface.

6. After receiving RRC Command Response from the UE, the gNB sends Command Response message towards the A-IoT CN.

<<<<<<<<<<<<End of change >>>>>>>>>>>>