3GPP TSG-RAN3 Meeting #128	R3-255855

Bengaluru, IN, 25-29 Aug 2025

Title:	(TP to BLCR 38.423) SBFD on UE to UE CLI

Source:	ZTE Corporation, Ericsson, China Telecom

Agenda Item:	19.2

Document for:	Other

- 1  	Introduction

This TP is used to capture the RAN3 agreements on UE to UE CLI:

Add a new indication in CLI INDICATION procedure for the gNB serving victim UEs to request for SRS resource configuration from the receiving gNBs.

Use the following IEs for gNB1 serving aggressor UEs to provide SRS-Resource configuration to gNB2 serving victim UEs:

- XnAP: Served Cell Information NR IE

- F1AP: Served Cell Information IE (DU to CU), Neighbour Cell Information List IE in GNB-CU CONFIGURATION UPDATE message (CU to DU).

- 2  TP

=============================Start of change==============================

      - 9.1.3.y	CLI INDICATION

This message is sent by NG-RAN node1 to NG-RAN node2 to report the results of the CLI measurements or to indicate the need for SRS Resource Configuration information.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| CLI Measurement Result |  | 1 |  |  | YES | ignore |
| >CLI Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>SSB index | O |  | INTEGER (0..63) | Strongest DL SSB beam information | – |  |
| >>NZP CSI-RS Resource Indication | O |  | INTEGER (1..64) | Strongest DL NZP CSI-RS beam information | – |  |
| >>CLI Mitigation Indication | O |  | ENUMERATED (true, …) | Indicates to request CLI mitigation | – |  |
| SRS Resource Indication | O |  | ENUMERATED (true, …) | Indication to NG-RAN node2 that SRS-Resource configuration information is needed | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

=============================Next change==============================

      - 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID | – |  |
| NR CGI | M |  | 9.2.2.7 |  | – |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code | – |  |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs contained in the SIB1 message as specified in TS 38.331[10], associated to the NR Cell Identity in the NR CGI IE. | – |  |
| >PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| CHOICE NR-Mode-Info | M |  |  |  | – |  |
| >FDD |  |  |  |  |  |  |
| >>FDD Info |  | 1 |  |  | – |  |
| >>>UL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 | This IE is ignored for NR operating bands for which uplink range of NREF is not defined in section 5.4.2.3 of TS 38.104 [24]. | – |  |
| >>>DL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – |  |
| >>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 | This IE is ignored for NR operating bands for which uplink range of NREF is not defined in section 5.4.2.3 of TS 38.104 [24]. | – |  |
| >>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the UL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>DL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the DL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>gNB-DU Cell Resource Configuration-FDD-UL | O |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD UL resource configuration of gNB-DU’s cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | YES | ignore |
| >>>gNB-DU Cell Resource Configuration-FDD-DL | O |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD DL resource configuration of gNB-DU’s cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | YES | ignore |
| >TDD |  |  |  |  |  |  |
| >>TDD Info |  | 1 |  |  | – |  |
| >>>Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – |  |
| >>>Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 | This IE is ignored if the Transmission Bandwidth asymmetric IE is present. | – |  |
| >>>Intended TDD DL-UL Configuration NR | O |  | 9.2.2.40 |  | YES | ignore |
| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon contained in the SIB1 message as defined in TS 38.331 [10] | YES | ignore |
| >>>Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>gNB-DU Cell Resource Configuration-TDD | O |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains TDD resource configuration of gNB-DU’s cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | YES | ignore |
| >>>Transmission Bandwidth asymmetric |  | 0..1 |  | Indicates the asymmetric UL and DL transmission bandwidth. | YES | ignore |
| >>>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| >>>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| >>>SBFD Configuration | O |  | FFS (pending on RAN2 progress) |  | YES | ignore |
| Measurement Timing Configuration | M |  | OCTET STRING | Includes the MeasurementTimingConfiguration inter-node message for the served cell, as defined in TS 38.331 [10]. | – |  |
| <<<<SKIP UNRELATED PART>>>> |  |  |  |  |  |  |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, ...) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | ignore |
| NZP CSI-RS Resources Configuration | O |  | 9.2.2.x2 |  | YES | ignore |
| SRS Resource Configuration List | O |  | 9.2.2.x3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by one gNB. Value is 256. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U channel IDs in a cell. Value is 16. |
| maxnoofMTCItems | Maximum no. of measurement timing configurations associated with the neighbour cell. Value is 16. |
| maxnoofCSIRSconfigurations | Maximum number of CSI RS configurations reported in the MTC. Value is 96 |
| maxnoofCSIRSneighbourCells | Maximum number of cells neighbouring a CSI-RS coverage area. Value is 16 |
| maxnoofCSIRSneighbourCellsInMTC | Maximum number of CSI-RS coverage areas neighbouring a specific CSI-RS coverage area. Value is 16 |

=============================Next change==============================

9.2.2.x3 SRS Resource Configuration List

This IE contains a list of SRS-Resource of UEs in the current cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRS Resource Configuration List |  | 1 |  |  |
| >SRS Resource Configuration Item |  | 1..<maxnoofSRS-Resource> |  |  |
| >>SRS Resource | M |  | OCTET STRING | Includes the SRS-Resource IE as defined in TS38.331[10]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRS-Resource | Maximum number of SRS Resource. Value is 64 |

=============================Next change==============================

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

<<<<SKIP UNRELATED PART>>>>

PDUSessionsListToBeReleased-UPError,

UserPlaneFailureIndication,

SRSPositioningConfigOrActivationRequest,

NRPPaPositioningInformation,

CLI-MeasurementResult-List,

SRS-Resource-Indication

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

id-A2XPC5QoSParameters,

id-ActivatedServedCells,

<<<<SKIP UNRELATED PART>>>>

id-UserPlaneFailureIndication,

id-SRSPositioningConfigOrActivationRequest,

id-NRPPaPositioningInformation,

id-CLI-MeasurementResult-List,

id-SRS-Resource-Indication,

=============================Next change==============================

-- **************************************************************

--

-- CLI Indication

--

-- **************************************************************

CLI-Indication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{CLI-Indication-IEs}},

...

}

CLI-Indication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-CLI-MeasurementResult-List					CRITICALITY ignore	TYPE CLI-MeasurementResult-List							PRESENCE 					optional}|

{ ID id-SRS-Resource-Indication	CRITICALITY ignore	TYPE SRS-Resource-Indication	PRESENCE optional},

...

}

END

-- ASN1STOP

=============================Next change==============================

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

<<<<SKIP UNRELATED PART>>>>

id-NRPPaPositioningInformation,

id-SBFD-Configuration,

id-NZP-CSI-RS-Resources-Config,

id-SRS-Resource-Configurtaion-List,

=============================Next change==============================

maxFailedCellMeasObjects,

maxFailedMeasPerNode,

maxnoofUEReports,

maxnoofCandidateRelayUEs,

maxnoofCAGforMDT,

maxnoofMDTSNPNs,

maxnoofSecurityConfigurations,

maxnoofRSPPQoSFlows,

maxnoofNZP-CSI-RS-ResourcesPerSet,

maxnoofSRS-Resource

=============================Next change==============================

#### -- S

SRS-Resource-Indication ::= ENUMERATED {true, ...}

SRS-Resource-Configuration-List ::= SEQUENCE (SIZE(1..maxnoofSRS-Resources)) OF SRS-Resource-Configuration-Item

SRS-Resource-Configuration-Item ::= SEQUENCE {

SRS-Resource						OCTET STRING,

iE-Extensions						ProtocolExtensionContainer { {SRS-Resource-Configuration-Item-ExtIEs} } OPTIONAL,

...

}

SRS-Resource-Configuration-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

=============================Next change==============================

##### -- Served Cells NR IEs

ServedCellInformation-NR ::= SEQUENCE {

nrPCI								NRPCI,

cellID								NR-CGI,

tac									TAC,

ranac								RANAC						OPTIONAL,

broadcastPLMN						BroadcastPLMNs,

nrModeInfo							NRModeInfo,

measurementTimingConfiguration		OCTET STRING,

connectivitySupport					Connectivity-Support,

iE-Extensions						ProtocolExtensionContainer { {ServedCellInformation-NR-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-BPLMN-ID-Info-NR				CRITICALITY ignore	EXTENSION BPLMN-ID-Info-NR						PRESENCE optional }|

{ ID id-ConfiguredTACIndication			CRITICALITY ignore	EXTENSION ConfiguredTACIndication				PRESENCE optional }|

{ ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst					PRESENCE optional }|

{ ID id-NRCellPRACHConfig				CRITICALITY ignore	EXTENSION NRCellPRACHConfig						PRESENCE optional }|

{ ID id-NPN-Broadcast-Information		CRITICALITY reject	EXTENSION NPN-Broadcast-Information			PRESENCE optional }|

{ ID id-CSI-RSTransmissionIndication	CRITICALITY ignore	EXTENSION CSI-RSTransmissionIndication		PRESENCE optional }|

{ ID id-SFN-Offset						CRITICALITY ignore	EXTENSION SFN-Offset							PRESENCE optional }|

{ ID id-Supported-MBS-FSA-ID-List		CRITICALITY ignore	EXTENSION Supported-MBS-FSA-ID-List			PRESENCE optional }|

{ ID id-NR-U-ChannelInfo-List			CRITICALITY ignore	EXTENSION NR-U-ChannelInfo-List					PRESENCE optional }|

{ ID id-Additional-Measurement-Timing-Configuration-List		CRITICALITY ignore	EXTENSION Additional-Measurement-Timing-Configuration-List		PRESENCE optional }|

{ ID id-Redcap-Bcast-Information		CRITICALITY ignore	EXTENSION Redcap-Bcast-Information			PRESENCE optional }|

{ ID id-eRedcap-Bcast-Information		CRITICALITY ignore	EXTENSION ERedcap-Bcast-Information			PRESENCE optional }|

{ ID id-MobileIABCell					CRITICALITY ignore	EXTENSION MobileIABCell							PRESENCE optional }|

{ ID id-XR-Bcast-Information			CRITICALITY ignore	EXTENSION XR-Bcast-Information					PRESENCE optional }|

{ ID id-BarringExemptionforEmerCallInfo	CRITICALITY ignore	EXTENSION BarringExemptionforEmerCallInfo	PRESENCE optional }|

{ ID id-NZP-CSI-RS-Resources-Config		CRITICALITY ignore	EXTENSION NZP-CSI-RS-Resources-Config		PRESENCE optional }|

{ ID id-SRS-Resource-Configurtaion-List		CRITICALITY ignore	EXTENSION SRS-Resource-Configurtaion-List		PRESENCE optional },

...

}

=============================Next change==============================

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

<<<<SKIP UNRELATED PART>>>>

maxnoofCAGforMDT								INTEGER ::= 256

maxnoofMDTSNPNs								INTEGER ::= 16

maxnoofSecurityConfigurations				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofNZP-CSI-RS-ResourcesPerSet		INTEGER ::= 64

maxnoofSRS-Resource							INTEGER ::= 64

=============================Next change==============================

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

<<<<SKIP UNRELATED PART>>>>

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-CLI-MeasurementResult-List																		ProtocolIE-ID ::= FFS

id-SBFD-Configuration																				ProtocolIE-ID ::= FFS

id-NZP-CSI-RS-Resources-Config																		ProtocolIE-ID ::= FFS

id-SRS-Resource-Configuration-List																	ProtocolIE-ID ::= FFS

id-SRS-Resource-Indication																			ProtocolIE-ID ::= FFS

END

-- ASN1STOP

=============================End of change==============================