3GPP TSG-RAN WG3 Meeting #129	R3-255796

# Bengaluru, India, 25th - 29th August 2025

Agenda Item:	16.2

Source:	Ericsson, Huawei, China Telecom

Title:	(TP to TS 38.300 BL CR) Detailed IE encoding and miscellaneous

Document for:	Agreement

1	Introduction

This TP follows discussions during online and offline discusisons at RAN3#129 and captures the following changes.

(The changes are performed with different "users" (tdoc number or topic) and highlighted in yellow.)

-	"R3-255542": 
orrect references to AS related protocol elements as suggested in R3-255542

-	"mandatory D2R size":
implement the agreement " Confirm that the Estimate of Expected D2R Message Size IE is mandatory provided in INVENTORY REQUEST.", also removing Editor's Note 1.

-	"command type":
no consensus on including a "command type"(" Introduce Command Type (write, read, disable, …) in Command Request Transfer?"), so the respective text and the FFS was removed

-	" time interval":
implement the agreement " Introduce Time Interval as Inventory Assistance Information.", also removing Editor's Note 2.

2	Text Proposal (changes against 38.300 BL CR in R3-255072)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory. The A-IoT Device Identification Requested may indicate inventory for a single device, a group of devices or for all devices.

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list) and Inventory Assistance Information. The Inventory Assistance information includes the expected D2R message size and may also include e.g., approximate number of Target A-IoT devices, time interval.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The Inventory procedure over the A-IoT radio interface is performed, i.e A-IoT paging, A-IoT Access Procedure and D2R data transmission, as sepcified in section 16.x.5 A-IoT MAC Layer Functions.

5./6.	Upon receiving the response(s) from A-IoT paging from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices.

7.	Upon completion of the inventory procedure the gNB may send an Inventory Complete Indication.

Editor’s Note 3:	It is FFS whether this inventory complete indication is done via a new procedure or introducing indication IE in the current Inventory Report message.

16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.xx.x4-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices.

NOTE 4: the step 2 may happen in parallel of the Inventory procedure involving other A-IoT devices or in parallel to other command procedures targeting other A-IoT devices within the same session.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB in order to send a command to a single A-IoT device. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure.

The Command Request also includes the RAN A-IoT device NGAP ID and the A-IoT NAS PDU corresponding to the targeted A-IoT device and may include some command assistance information. This command assistance information may contain the estimate of expected D2R message size.

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface, i.e., R2D and D2R data transmission, as specified in section 16.x.5 A-IoT MAC Layer Functions.

4.	The gNB replies to the Command Request with the Command Response messageincluding the AIOTF Identifier the Correlation ID and the RAN A-IoT device NGAP ID received in the Command Request, and the A-IoT NAS PDU received from the device.

NOTE 5:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>