3GPP TSG-RAN WG3 Meeting #129	R3-255791

Bengaluru, India, 25 – 29 August 2025

Agenda item:	10.2

Source:	Samsung, Huawei, Nokia

Title:	(TP for SON BLCR for TS38.401) MRO for LTM

Document for:	Discussion & Decision

# Introduction

The contribution provides TP to BLCR for TS 38.401 on MRO for LTM.

# TP to 38.401 on MRO for LTM

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis. In case of failures due to inappropriate cell switch triggering or Wrong target cell selection for LTM cell switch, the gNB-CU forwards the RLF report to the last serving gNB-DU in case of too late LTM cell switch, or to the source gNB-DU in case of too early LTM cell switch or LTM cell switch to wrong cell.

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt). The target gNB-DU performs initial analysis and sends the recovery beam information to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell. The target gNB-DU sends the re-established or recovery beam information together with the TA value used at successful RACH-based access to the source gNB-DU via the gNB-CU.