3GPP TSG-RAN WG4 Meeting #116												R4-2509040

Bengaluru, India, 25 – 29 August, 2025

Agenda item:			7.10.3.1

Source:	Huawei, HiSilicon

Title:	on XR OTA tests

Document for:	Approval

# Introduction

In RAN4 #115, discussion on XR OTA metrics took place, but without final agreement [1]. This contribution further explores this topic.

# Discussion

Operators [4] put forward a proposal for metrics to emphasis specific directions resulting in two types of metric, namely traditional TRP and TRS metrics measured over a full sphere, together with direction specific Tx and Rx metrics. The reason for direction specific Tx and Rx metrics given in [4] is that XR devices are likely to transmit and receive from a few fixed directions due to their head worn nature. However such assumption is in contradiction with the XR use scenarios defined in TR 26.928 [2]. The degrees of freedom for XR devices are defined in clause 4.1.2 of TR 26.928 [2]. Figure 4.1.3 from clause 4.1.2 are reproduced below. Furthermore, Annex A of TR 26.928 lists various use cases with associated degrees of freedom given Table A.1-2.

As clearly shown in the above figure, the XR devices are not limited to any specific directions during their use. For example, conventional wisdom may assume base stations signals come from a limited range of angles above horizon with typical base station antenna height of 15 meters or so. However, such assumption may not hold due to the following reasons.

- XR device movement as defined by the degrees of freedom would change the signal arrival directions relative to XR devices.

- Most XR devices are used indoors as in given clause 6 (Deployment Scenarios) of TR 38.838 [5] with 80/20% indoor/outdoor for dense urban. Indoor hotspot is another deployment scenario in TR 38.838 [5].

- Multi-paths, especially indoors, would change the signal arrival directions

Observation 1: base stations signals are not limited to specific directions relative to XR devices due to degrees of freedom under use cases in TR 26.928, heavy indoor usage and multipaths.

Due to above reasons, it would be prudent to use traditional TRP and TRS metrics over a full sphere as emphasising specific directions in TRP and TRS could degrade XR user experience.

Proposal 1: use traditional TRP and TRS metrics over a full sphere for XR devices in view of Observation 1.

In addition, it is difficult to avoid conflicts between two metrics with different emphasis for the same performance. From antenna design viewpoint, it is sensible to have a single performance target.

Observation 2: avoid having two different metrics for the same performance characteristic.

# Conclusion

This contribution provides the following observations and proposal.

Observation 1: base stations signals are not limited to specific directions relative to XR devices due to degrees of freedom under use cases in TR 26.928, heavy indoor usage and multipaths.

Proposal 1: use traditional TRP and TRS metrics over a full sphere for XR devices in view of Observation 1.

Observation 2: avoid having two different metrics for the same performance characteristic.

# Reference

- R4-2508782 Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3

- TR 26.928 v18.0.0 Extended Reality (XR) in 5G

- TR 38.838 v18.0.1 Study on XR enhancements for NR

- R4-2507005 Discussion on OTA requirements for XR Headworn Devices

- TR 38.838 v17.0.0 Study on XR (Extended Reality) Evaluations for NR