3GPP TSG-RAN WG4 Meeting #116	R4-2509170

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.24.7

Source:	MediaTek Inc.

Title:	Discussion on demodulation requirements for R19 LP-WUS/WUR

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the remining issues of demodulation requirements for R19 LP-WUS/WUR [1].

- Discussion

Issue 2-1-1: Whether to define a test mode for LP-WUS/WUR - Agreement - Take RF approach as a starting point to test MDR for demodulation requirements. RF agreement Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods) - FFS whether test mode is recommended for MDR testing.

Regarding the test mode, we believe it is essential to avoid adopting test modes that introduce special NW/UE behaviours that deviate from the 3GPP specifications solely for testing purposes. Such approaches impose additional implementation complexity and effort on the UE side.

Observation 1: Test mode would impose additional implementation complexity and effort on the UE side.

Proposal 1: Avoid adopting test modes that introduce special NW/UE behaviours that deviate from the 3GPP specifications solely for testing purposes.

Issue 3-1-3: Whether to introduce FAR test - WF - Option 1: Yes. - Option 2: No.

In our view, the MDR is expected to have a more significant impact on demodulation performance, as a missed detection would prevent the MR from monitoring paging or PDCCH occasions. On the other hand, the FAR does not affect demodulation performance but results in a power penalty, as the MR would incorrectly wake up to monitor paging or PDCCH occasions. While false alarms in DL wake-up signal reception may increase UE power consumption, they do not impact overall network performance. Therefore, we believe it is more appropriate to focus on MDR as part of the demodulation performance requirements and consider it sufficient to define requirements for the missed detection rate of wake-up signals.

Proposal 2: Do not specify the FAR test.

Issue 3-1-8: Frequency range - WF - Option 1: Consider both FR1 and FR2. - Option 2: Prioritize FR1; FFS FR2. Issue 3-1-9: Duplex mode - WF - Option 1: Consider both TDD and FDD. Issue 3-1-10: SCS - WF - Option 1: 15kHz for FDD FR1 and 30kHz for TDD FR1. - Option 2: 15kHz for FDD FR1, 30kHz for TDD FR1 and 120KHz for FR2 Issue 3-1-11: Bandwidth - WF - Option 1: For the channel bandwidth and SCS, take below assumptions for the starting points - 10MHz CHBW with SCS 15KHz for FR1 FDD - 40MHz CHBW with SCS 30KHz for FR1 TDD - 100MHz CHBW with SCS 120KHz for FR2

FR1 is more widely deployed globally compared to FR2. This makes it more practical to focus on FR1 as it affects a larger number of users and networks. Therefore, we propose to prioritize FR1 as FR2. Correspondingly, we support to consider 15kHz SCS/10MHz CHBW for FDD FR1 and 30kHz SCS/40MHz CHBW for TDD FR1.

Proposal 3: Consider both TDD and FDD.

Proposal 4: Prioritize FR1; FFS FR2.

Proposal 5: Consider 10MHz CHBW with SCS 15KHz for FR1 FDD and 40MHz CHBW with SCS 30KHz for FR1 TDD.

Issue 3-1-17: Whether to define performance requirements for LP-SS - WF - Option 1: No - Option 2: Postpone further study on LP-SS until decided if performance requirements are to be introduced for LP-WUR/LP-WUS (OOK receiver)

LP-SS serves a similar role to NR PSS/SS, supporting synchronization and measurement functions. Although the performance of LP-SS estimation can affect synchronization, its impact is ultimately reflected in the overall demodulation performance. Consequently, we recommend focusing exclusively on testing LP-WUS, as RAN4 has not previously defined specific performance requirements for synchronous signals, rendering separate requirements for LP-SS unnecessary.

Proposal 6: Do not define performance requirements for LP-SS.

# Conclusion

In this contribution, we provide our views about performance requirements for LP-WUS/WUR. Observations and proposals are summarized as follows.

Observation 1: Test mode would impose additional implementation complexity and effort on the UE side.

Proposal 1: Avoid adopting test modes that introduce special NW/UE behaviours that deviate from the 3GPP specifications solely for testing purposes.

Proposal 3: Consider both TDD and FDD.

Proposal 4: Prioritize FR1; FFS FR2.

Proposal 5: Consider 10MHz CHBW with SCS 15KHz for FR1 FDD and 40MHz CHBW with SCS 30KHz for FR1 TDD.

Proposal 6: Do not define performance requirements for LP-SS.

# Reference

- R4-2508626, WF for [115][332] NR_LPWUS_Demod, Qualcomm