3GPP TSG-RAN WG4 Meeting #116	R4-2509201

Bengaluru, India, August 25th – 29th, 2025

Source:	Qualcomm Incorporated

Title:	A-MPR for Intra-band contiguous UL CA

Agenda item:	7.1.2.1.2

Document for:	Approval

# 1.	Introduction

For this WI companies were requested to measure/simulate MPR/AMPR numbers for several intra-band contiguous and non-contiguous scenarios. In RAN4#115 MPRs for ULCA contiguous CCs with non-contiguous RBs and both MPRs and A-MPRs for ULCA non-contiguous CCs as well as  A-MPR values for ULCA contiguous CC with contiguous RBs were agreed.  In this paper we present further views on the remaining items for intra-band ULCA A-MPRs.

# 2. 	Discussion

In RAN4#115 a WF [1] was created that encouraged companies to investigate the following open issues:

## 1.5 CA_NS_04 A-MPR for intra-band contiguous ULCA with non-contiguous RB allocation

Comment:

Companies are encouraged to investigate the following proposal and the accompanying modification to the A-MPR algorithm captured in R4-2509203:

In section 6.2A.3.1.1.1.2 for non-contiguous allocations make the following correction:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

IF (Ffilter,low < FIM3,low_block,high) OR  (2490.5 MHz> FIM3,low_block,low)

-  A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3

Else

-	if RB allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if RB allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

………….

For A-MPR for intra-band contiguous ULCA with non-contiguous RB allocation study whether the following A-MPR values are acceptable for 2Tx PC1.5

AMPRIM3=MA, Where MA is defined as follows for 2 Tx PC1.5

MA = 	16.5; 	0 ≤ B < 1.44

15.5; 	1.44 ≤ B < 2.88

14.5;       2.88 ≤ B < 5.76

12.5;       5.76 ≤ B < 10.8

10.5;       10.8 ≤ B <23.04

10;        23.04 ≤ B

As discussed in RAN4#115 [2] measurements were done for contiguous CC/non-contiguous RBs however when the A-MPR formula in the standard [2] was examined an error was noticed in the current formula for PC3 and PC2. The standard states the following for contiguous CC/non-contiguous RB CA_NS_04:

6.2A.3.1.1.1.2	Non-contiguous allocations

For intra-band contiguous CA_n41B and CA_n41C and it receives IE CA_ NS_04, the UE determines the allowed Additional Maximum Power Reduction (AMPR) for the maximum output power as specified in this clause. The AMPR is specified by AMPRIM3 to meet -25dBm/MHz when IM3 falls in -25dBm/MHz region of Table 6.5A.2.3.1.1-1 or Table 6.5A.3.3.1.1-1. And uses MPR for all other cases.

The UE determines the AMPR type as follows:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA ≥ 2490.5 MHz,

-	if allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

-	if RB allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if RB allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3.

where

-	MPR is the MPR as defined in Table 6.2A.2.1-2, Table 6.2A.2.1-3 and Table 6.2A.2.1-4 for PC3 and PC2 respectively and the respective CA bandwidth class

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.1.1

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.1.1

6.2A.3.1.1.1.3	AMPRIM3 to meet -25dBm/MHz

AMPR in this clause is for intra-band contiguous CA_n41B and CA_n41C. The allowed maximum output power reduction is defined as:

AMPRIM3=MA, Where MA is defined as follows

MA = 	13; 	0 ≤ B < 2.16

11.5; 	2.16 ≤ B < 3.24

10.5;       3.24 ≤ B < 5.04

9.5; 	5.04 ≤ B < 10.08

8; 	10.08 ≤ B < 16.56

7;        16.56 ≤ B < 21.96

6; 	      21.96 ≤ B

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 (MHz), where SCS1 and SCS2 are expressed in kHz.

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency.

Our understanding is that if Ffilter,low < FIM3,low_block,high <2490.5 MHz Then the A-MPR values in the ELSE section should be used.

Proposal 1: In section 6.2A.3.1.1.2 for non-contiguous allocations make the following correction:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

IF (Ffilter,low < FIM3,low_block,high) OR  (2490.5 MHz> FIM3,low_block,low)

-  A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3

Else

-	if RB allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if RB allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3.

where

-	MPR is the MPR as defined in Table 6.2A.2.1-2, Table 6.2A.2.1-3, Table 6.2A.2.1-4, Table 6.2A.2.1-5 for PC3 for bandwidth classes B and C, PC2 for bandwidth classes B and C and PC1.5 bandwidth class C respectively and the respective CA bandwidth class

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-     FIM3,low_block,low = (2 * Flow_alloc,low_edge ) – Fhigh_alloc,high_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.1.1

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.1.1

A CR, R4-2509203, has been submitted in the maintenance agenda to make the above-described corrections to the non-contiguous A-MPR algorithm.

Also, it seems that the A-MPRs given in section 6.2A.3.1.1.1.3 are only for PC3 and not PC2. This can be seen by comparing NCCA AMPRs for PC3 and PC2 given for -25dBm/MHz in sections 6.2A.3.1.2.1.1 and 6.2A.3.1.2.1.3. Also, if you compare the PC3 contiguous CC/non-contiguous RBs in section 6.2A.3.1.1.1.3 with the PC3 NCCA in section 6.2A.3.1.1.3 the following delta AMPR is observed between the 2 sets of values:

| NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  | NS04 NCCA (section 6.2A.3.1.2.1.1) |  | Delta between C-CA and NCCA |
| --- | --- | --- | --- | --- |
| MA[dB] | Allocation BW[MHz] | MA[dB] | Allocation BW[MHz] | [dB] |
| 13 | 0 ≤ B < 2.16 | 12 | 0 ≤ B < 2.16 | 1 |
| 11.5 | 2.16 ≤ B < 3.24 | 11 | 2.16≤ B < 3.24 | 0.5 |
| 10.5 | 3.24 ≤ B < 5.04 | 10 | 3.24 ≤ B < 5.04 | 0.5 |
| 9.5 | 5.04 ≤ B < 10.08 | 9 | 5.04 ≤ B < 10.08 | 0.5 |
| 8 | 10.08 ≤ B < 16.56 | 8 | 10.08 ≤ B < 16.38 | 0 |
| 7 | 16.56 ≤ B < 21.96 | 7 | 16.38 ≤ B < 21.78 | 0 |
| 6 | 21.96 ≤ B | 6 | 21.78 ≤ B | 0 |

Table2: A-MPR comparison between contiguous CC/NC RBs and NCCA for PC3

Based on the deltas in the above table the A-MPRs for contiguous CC/NC RBs can be derived by adding the same A-MPR deltas to the NCCA PC2 values given in section 6.2A.3.1.2.1.3. Therefore, we have the following values as illustrated below:

| NS04 NCCA (section 6.2A.3.1.2.1.1) |  | Delta between C-CA and NCCA | NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  |
| --- | --- | --- | --- | --- |
| MA[dB] | Allocation BW[MHz] | [dB] | MA[dB] | Allocation BW[MHz] |
| 14.5 | 0 ≤ B < 1.44 | 1 | 15.5 | 0 ≤ B < 1.44 |
| 14 | 1.44 ≤ B < 2.88 | 0.5 | 14.5 | 1.44 ≤ B < 2.88 |
| 13 | 2.88 ≤ B < 5.76 | 0.5 | 13.5 | 2.88 ≤ B < 5.76 |
| 11 | 5.76 ≤ B < 10.8 | 0.5 | 11.5 | 5.76 ≤ B < 10.8 |
| 9.5 | 10.8 ≤ B < 23.04 | 0 | 9.5 | 10.8 ≤ B < 23.04 |
| 9 | 23.04 ≤ B | 0 | 9 | 23.04 ≤ B |

Table3: A-MPR for contiguous CC/NC RBs PC2

Proposal 2: In section 6.2A.3.1.1.3 adopt the following A-MPR table for 1Tx PC2

| NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  |
| --- | --- |
| MA[dB] | Allocation BW[MHz] |
| 15.5 | 0 ≤ B < 1.44 |
| 14.5 | 1.44 ≤ B < 2.88 |
| 13.5 | 2.88 ≤ B < 5.76 |
| 11.5 | 5.76 ≤ B < 10.8 |
| 9.5 | 10.8 ≤ B < 23.04 |
| 9 | 23.04 ≤ B |

The NS04 contiguous CA with NC RB allocations for section 6.2A.3.1.1.3 the PC1.5 A-MPR table can be derived by adding MPR deltas similar to that seen between PC3(1Tx) and PC2(2Tx) bandwidth class C in sections 6.2A.2.1 of approximately 1dB to the PC2 table derived for 1 TxPC2. The PC1.5 A-MPR table is shown below

| NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  |
| --- | --- |
| MA[dB] | Allocation BW[MHz] |
| 16.5 | 0 ≤ B < 1.44 |
| 15.5 | 1.44 ≤ B < 2.88 |
| 14.5 | 2.88 ≤ B < 5.76 |
| 12.5 | 5.76 ≤ B < 10.8 |
| 10.5 | 10.8 ≤ B < 23.04 |
| 10 | 23.04 ≤ B |

Table3: A-MPR for contiguous CC/NC RBs PC1.5

Proposal 3: In section 6.2A.3.1.1.3 adopt the following A-MPR table for 2Tx PC1.5

| NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  |
| --- | --- |
| MA[dB] | Allocation BW[MHz] |
| 16.5 | 0 ≤ B < 1.44 |
| 15.5 | 1.44 ≤ B < 2.88 |
| 14.5 | 2.88 ≤ B < 5.76 |
| 12.5 | 5.76 ≤ B < 10.8 |
| 10.5 | 10.8 ≤ B < 23.04 |
| 10 | 23.04 ≤ B |

# Conclusion

In this paper we present the following proposals on the remaining items for intra-band PC1.5 ULCA

Proposal 1: In section 6.2A.3.1.1.2 for non-contiguous allocations make the following correction:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

IF (Ffilter,low < FIM3,low_block,high) OR  (2490.5 MHz> FIM3,low_block,low)

-  A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3

Else

-	if RB allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if RB allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3.

where

-	MPR is the MPR as defined in Table 6.2A.2.1-2, Table 6.2A.2.1-3, Table 6.2A.2.1-4, Table 6.2A.2.1-5 for PC3 for bandwidth classes B and C, PC2 for bandwidth classes B and C and PC1.5 bandwidth class C respectively and the respective CA bandwidth class

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-     FIM3,low_block,low = (2 * Flow_alloc,low_edge ) – Fhigh_alloc,high_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.1.1

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.1.1

Proposal 2: In section 6.2A.3.1.1.3 adopt the following A-MPR table for 1Tx PC2

| NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  |
| --- | --- |
| MA[dB] | Allocation BW[MHz] |
| 15.5 | 0 ≤ B < 1.44 |
| 14.5 | 1.44 ≤ B < 2.88 |
| 13.5 | 2.88 ≤ B < 5.76 |
| 11.5 | 5.76 ≤ B < 10.8 |
| 9.5 | 10.8 ≤ B < 23.04 |
| 9 | 23.04 ≤ B |

Proposal 3: In section 6.2A.3.1.1.3 adopt the following A-MPR table for 2Tx PC1.5

| NS04 contiguous CA with NC RB allocations (section 6.2A.3.1.1.3) |  |
| --- | --- |
| MA[dB] | Allocation BW[MHz] |
| 16.5 | 0 ≤ B < 1.44 |
| 15.5 | 1.44 ≤ B < 2.88 |
| 14.5 | 2.88 ≤ B < 5.76 |
| 12.5 | 5.76 ≤ B < 10.8 |
| 10.5 | 10.8 ≤ B < 23.04 |
| 10 | 23.04 ≤ B |

# References

[1] R4-2507934, WF on HPUE RF requirements, 19-23 May, 2025

[2] ] R4-2505963, MPR and A-MPR for Intra-band contiguous UL CA for PC1.5, 19-23 May, 2025