3GPP TSG-RAN WG4 Meeting #116	R4-2509288

# Bengaluru , IN, 25th – 29th Aug, 2025

Title: 	Discussion on Event triggered L1 measurement reporting for Rel-19 LTM enhancements

Source: 	CATT

Agenda item:	7.25.3.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, the RRM requirements for event triggered L1 measurement reporting were discussed. The conclusions were captured in the approved WF [1]. In this paper, we further discuss the RRM impact for event triggered L1 measurement for LTM and give our views.

- Discussion

- Issue 1-1: applicability of L1-RSRP reporting delay

In the last meeting, the candidate options for applicability of L1-RSRP reporting delay is copied as below [1]:

Candidate options: - Option 1: RAN4 considers following conditions in the test for L1-RSRP reporting delay requirements: (vivo, Xiaomi) - No indicate TCI change during the evaluation period. - No configuration change w.r.t measurement RS during the evaluation period - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period - Option 2: Seems no need to capture assumptions listed under option-1 into core requirements. (Apple) - Option 2a: The proposed L1-RSRP reporting assumptions are not captured in core requirements. During performance phase, RAN4 can consider to ensure serving beam kept unchanged (e.g., no indicate TCI change, and no configuration change) for L1 event trigger reporting test cases. (HW)

In our view, there is no need to capture the conditions listed in option 1 in core requirements, RAN4 can consider the conditions to ensure serving beam kept unchanged for L1 event trigger reporting test cases in performance part.

Proposal 1: There is no need to capture the conditions listed in option 1 in core requirements, RAN4 can consider the conditions to ensure serving beam kept unchanged for L1 event trigger reporting test cases in performance part.

- Issue 1-2: Whether to introduce/update requirements for reporting criteria per measurement category

For whether to introduce/update requirements for reporting criteria per measurement category and whether to define a separate capability for supported number of events triggered L1 measurement reporting, the candidate options are copied as follows [1]:

Candidate options: - Option 1: Define a separate capability for supported number of events triggered L1 measurement reporting. (CTC, Nokia, Xiaomi, CATT, vivo, MTK, ZTE, CMCC, E///) - Details: - Option 1a:(vivo) - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability. - Reporting criteria for LTM event triggered reporting is 4 per frequency layer. - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured. - Option 1b: E/// - RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. - RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS - Option 2: update the existing requirements to cover L1 event triggered reporting criteria per measurement category. (OPPO) - Option 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

In our view, the existing reporting criteria per measurement category did not consider LTM L1 events. Considering that sharing the same UE capabilities regarding support of event triggering between LTM events and L3 events is complicated, so it is better to introduce new UE capability for LTM. In addition, the existing Ecat was defined for frequency layer and different Ecat is defined for intra-frequency, inter-frequency and so on. However, whether to define intra-frequency and inter-frequency CSI-RS based L1 measurement are still FFS, so a separate UE capability is more conducive to defining supported number of events triggered L1 measurement reporting.

Proposal 2: Updating requirements for reporting criteria per measurement category is needed.

Proposal 3: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM.

- Conclusion

This document discussed the issues for event triggered L1 measurement for LTM and the following proposals are presented:

Proposal 1: There is no need to capture the conditions listed in option 1 in core requirements, RAN4 can consider the conditions to ensure serving beam kept unchanged for L1 event trigger reporting test cases in performance part.

Proposal 2: Updating requirements for reporting criteria per measurement category is needed.

Proposal 3: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM.

- Reference

[1]     R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple