3GPP TSG-RAN WG4 Meeting #116	R4-2509291

# Bengaluru , IN, 25th – 29th Aug, 2025

Title: 	Discussion on Rel-19 NR mobility Phase4 RRM performance requirements

Source: 	CATT

Agenda item:	7.25.4

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 has reached good progress in defining the core RRM requirements and some draft CRs has been endorsed [1]. In this contribution, we discuss and provide our initial views on the performance requirements based on the agreed core requirements.

- Discussion

  - 2.1 Event triggered L1 measurement reporting

For event triggered L1 measurement reporting, since RAN4 has agreed to define the related requirements for SSB based and CSI-RS based measurements, so the test cases also need to cover SSB based and CSI-RS based measurements, and the new test cases for event triggered L1 measurement reporting need to defined in the clauses defined for R18 LTM.

Proposal 1: Test cases for event triggered L1 measurement reporting need to cover SSB based and CSI-RS based measurements.

Proposal 2: The new test cases for event triggered L1 measurement reporting need to defined in the clauses defined for R18 LTM.

For the test scope, at least for event triggered SSB based L1 measurement reporting, the folloeing test cases can be defined:

- LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX

- LTM Inter-frequency event triggered L1 measurement reporting tests with gaps under non-DRX

- LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX with SSB index reading

Proposal 3: For event triggered SSB based L1 measurement reporting, the folloeing new test cases can be defined:

- TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX

- TC for LTM Inter-frequency event triggered L1 measurement reporting tests with gaps under non-DRX

- TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX with SSB index reading

For event triggered CSI-RS based L1 measurement reporting, only test case for SA event triggered reporting tests without gap under non-DRX should be defined.

Proposal 4: For event triggered CSI-RS based L1 measurement reporting, only new TC for LTM Intra-frequency event triggered reporting tests without gap under non-DRX should be defined.

  - 2.2 CSI-RS based L1 measurement

- CSI-RS accuracy requirements

For CSI-RS accuracy requirements for neighbor cell, RAN4 didn’t discuss this issue in the last meeting and this issue should be discussed in performance part. The candidate options are copied as follows [1]:

Candidate options: - Option 1: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline. (ZTE, Apple, Nokia)

In our view, CSI-RS based L1-RSRP accuracy requirements defined in clauses 10.1.19.2 and 10.1.20.2 can be a baseline for CSI-RS based L1-RSRP requirements for neighbor cell. And RAN4 need to define new clauses for CSI-RS based L1-RSRP requirements for neighbor cell in clauses 10.1.19D, 10.1.19E, 10.1.20A and 10.1.20B as R18 SSB based L1-RSRP requirements for neighbor cell.

Proposal 5: CSI-RS based L1-RSRP accuracy requirements defined in clauses 10.1.19.2 and 10.1.20.2 can be a baseline for CSI-RS based L1-RSRP requirements for neighbor cell.

Proposal 6: RAN4 to define new clauses for CSI-RS based L1-RSRP requirements for neighbor cell in clauses 10.1.19D, 10.1.19E, 10.1.20A and 10.1.20B as R18 SSB based L1-RSRP requirements for neighbor cell.

- Test scope

For CSI-RS based L1-RSRP requirements for neighbor cell, since RAN4 has agreed not to define the core requirements for L1-RSRP measurement with measurement gap, so the test case also doesn’t need to consider the measurement with measurement gap.

For the definition of intra-frequency and inter-frequency, RAN4 has not reached a conclusion yet, but there is a high probability that RAN4 will have a related definition. However, we think that the definition just to distinguish terms, and the core requirements for intra-frequency and inter-frequency measurement will have no much difference. Therefore, we believe that RAN4 only need to define the test case for LTM CSI-RS based Intra-frequency L1-RSRP measurement in FR1 and FR2.

Proposal 7: RAN4 to define the test case for LTM CSI-RS based Intra-frequency L1-RSRP measurement in FR1 and FR2.

  - Conditional Intra-CU LTM

- Test configurations and parameters

Regarding the test cases for conditional Intra-CU LTM, the legacy test configurations and parameters used in R18 LTM test cases can be a baseline.

Proposal 8: The legacy test configurations and parameters used in R18 LTM test cases can be a baseline for conditional LTM test cases.

- Test scope

In RAN4#112-bis, only ‘PCell switch to a neighboring LTM candidate cell’ and ‘PCell switch to an LTM candidate cell that is a serving SCell in MCG’ were agreed for applicable scenarios, so we believe that only conditional LTM PCell Switch test cases should be defined.

Proposal 9: RAN4 to define new test cases for conditional LTM PCell Switch.

In core part, RAN4 has agreed to define the requirements for L3 measurement based conditional LTM for FR1 and not to define the requirements for L3 measurement based conditional LTM for FR2, so we think that the test case for conditional LTM PCell switch without L1-RSRP measurement should be defined as R18 LTM.

Proposal 10: RAN4 to define the test case for conditional LTM PCell switch without L1-RSRP measurement should be defined.

Moreover, RAN4 agreed to cover both RACH-based and RACH-less conditional intra-CU LTM, so the test case for conditional LTM also need to cover RACH-based and RACH-less cases.

Proposal 11: The test cases for conditional LTM need to cover RACH-based and RACH-less cases.

For PDCCH ordered RACH, RAN4 has agreed that there is	no need to change existing PDCCH ordered RACH delay requirements, so we think that there is no need to define the new test cases for PDCCH ordered RACH for conditional LTM.

Proposal 12: There is no need to define the new test cases for PDCCH ordered RACH for conditional LTM.

- Conclusion

This document discussed the issues for R19 ATG RRM performance requirements and presented following proposals:

Proposal 1: Test cases for event triggered L1 measurement reporting need to cover SSB based and CSI-RS based measurements.

Proposal 2: The new test cases for event triggered L1 measurement reporting need to defined in the clauses defined for R18 LTM.

Proposal 3: For event triggered SSB based L1 measurement reporting, the folloeing new test cases can be defined:

- TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX

- TC for LTM Inter-frequency event triggered L1 measurement reporting tests with gaps under non-DRX

- TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX with SSB index reading

Proposal 4: For event triggered CSI-RS based L1 measurement reporting, only new TC for LTM Intra-frequency event triggered reporting tests without gap under non-DRX should be defined.

Proposal 5: CSI-RS based L1-RSRP accuracy requirements defined in clauses 10.1.19.2 and 10.1.20.2 can be a baseline for CSI-RS based L1-RSRP requirements for neighbor cell.

Proposal 6: RAN4 to define new clauses for CSI-RS based L1-RSRP requirements for neighbor cell in clauses 10.1.19D, 10.1.19E, 10.1.20A and 10.1.20B as R18 SSB based L1-RSRP requirements for neighbor cell.

Proposal 7: RAN4 to define the test case for LTM CSI-RS based Intra-frequency L1-RSRP measurement in FR1 and FR2.

Proposal 8: The legacy test configurations and parameters used in R18 LTM test cases can be a baseline for conditional LTM test cases.

Proposal 9: RAN4 to define new test cases for conditional LTM PCell Switch.

Proposal 10: RAN4 to define the test case for conditional LTM PCell switch without L1-RSRP measurement should be defined.

Proposal 11: The test cases for conditional LTM need to cover RACH-based and RACH-less cases.

Proposal 12: There is no need to define the new test cases for PDCCH ordered RACH for conditional LTM.

- Reference

[1]     R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple