3GPP RAN WG4 Meeting #116	R4-2509302

Bengaluru, India, August 25th – August 29th, 2025

Title: 	Discussion on impacts of AIML RRM measurement prediction on RRM requirements

Source: 	CATT

Agenda item:	7.19.3.1

Document for:	Discussion

Introduction

In RAN4#115 meeting, RAN4 discussed the RRM impacts of AI/ML based mobility and the agreements were captured in the WF [1]. In this contribution, we will further discuss the open issues listed in [2] and present our views.

Discussion

Accuracy requirement for predicted measurements

In SI phase, the scenarios that are considered are copied below for information (highlighted scenarios are prioritized):

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

According to TR 38.744 [3], two study goals that are specified for the above scenarios are as follows:

1st goal: to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements.

2nd goal: to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

In RAN4#115 meeting, proposals that are related to performance requirements were brought up:

Proposals and Observations: - Proposal 1 (Ericsson): Predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurement reporting for mobility purposes. - Proposal 2 (Nokia): For temporal domain Case-B prediction, RAN4 may consider accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement. - Proposal 3 (ZTE): RAN4 should discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases.

Regarding the first proposal, it is quite straightforward for the case with the 2nd study goal since the goal is to improve handover performance. For the case with 1st study goal which is to reduce measurement efforts, it is possible that the prediction accuracy slightly degrades when UE saves measurement efforts. Therefore RAN4 need further discuss whether a relaxed prediction accuracy requirement is defined.

On whether separate or unified prediction accuracy requirement is defined for 3 cell level sub-use cases, RAN4 can discuss after concluding whether to consider relaxed accuracy requirement for scenario 4 where the 2nd study goal applies.

Proposal 1: For scenario 4 (2nd study goal), the predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurements for mobility purposes.

Proposal 2: For scenarios with 1st study goal, RAN4 discuss whether a relaxed prediction accuracy requirement is defined.

Impacts of measurement error on prediction accuracy

In RAN4#115 meeting, RAN4 discussed the Impacts of measurement error on prediction accuracy without agreement:

RAN4#115 Recommended WF: - RAN4 to capture following factors to be considered for error models to consider the impact of measurement errors on prediction accuracy - RF and baseband measurement error modeling may be considered. - A uniform or a Gaussian distribution can be used as a model for measurement error for RF and baseband. - Alternative modeling approaches are not precluded and can be explored in the Work Item phase.

RAN4 has been discussed this issue for multiple meetings without much progress. In general, both baseband error and RF error included in the measurement results have impacts on prediction accuracy. RAN4 now need to numerically evaluate the impacts, facilitating defining the performance requirements in WI phase. For baseband error, link-level simulation can be used to evaluate its impact with different SNR levels considered. For RF error, we also think a uniform or Gaussian distribution can be used to model the RF error. The exact baseband error model and RF model can be further determined in WI phase. Considering that a TP needs to be submitted in this meeting, we suggest all options are captured for further discussions.

Proposal 3: RAN4 to evaluate the impact of baseband error using link-level simulation with different SNR levels considered. For RF error, a uniform or Gaussian distribution can be used.

Proposal 4: RAN4 to capture all options in the TR for further discussions.

Impacts from observation and prediction windows

Regarding the impacts of observation and prediction windows, agreements reached in RAN4#114 meeting were copied below:

RAN4#114 Agreement: - RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions. - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

RAN4 agreed to consider the impact of UE speed, MRRT and OW and PW lengths on RRM requirements for temporal predictions. But since this is a study item now and the framework of AI/ML based mobility is not designed by RAN2 yet, RAN4 can discuss the details of impact of UE speed, MRRT and OW and PW lengths in the work item phase based on more concrete foundations after RAN2 have progresses.

Proposal 5: RAN4 to discuss the impact of UE speed, MRRT and OW and PW lengths in the work item phase unless any issues are identified to be discussed.

Conclusions

This contribution discussed the impacts of AI/ML RRM measurement prediction on RRM requirements, and some observations and proposals are provided:

Proposal 1: For scenario 4 (2nd study goal), the predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurements for mobility purposes.

Proposal 2: For scenarios with 1st study goal, RAN4 discuss whether a relaxed prediction accuracy requirement is defined.

Proposal 3: RAN4 to evaluate the impact of baseband error using link-level simulation with different SNR levels considered. For RF error, a uniform or Gaussian distribution can be used.

Proposal 4: RAN4 to capture all options in the TR for further discussions.

Proposal 5: RAN4 to discuss the impact of UE speed, MRRT and OW and PW lengths in the work item phase unless any issues are identified to be discussed.

References

[1] R4-2508268, WF on AI/ML Mobility Part 1, Nokia, RAN4#115.

[2] R4-2505564, Topic summary for [115][215] FS_NR_AIML_Mob_Part1, Nokia, RAN4#115.

[3] 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, V0.0.4.