3GPP TSG-RAN WG4 Meeting # 116                                                              R4-2509330

Bengaluru, India, August 25th – 29th, 2025

Title: 	draft TP for TS 38.194 to introduce base station output power and transmit ON/OFF power

Source: 	CATT

Agenda item:	7.22.3.1

Document for:	Approval

- Introduction

This contribution provides a draft TP to TS 38.194 for A-IoT BS Tx requirements. The agreements of Tx requirements in the WF [2] are included in the TP.

Reference

[1] TS 38.194, V0.1.0

[2] R4-2508101, “WF on A-IoT BS and CW requirements”, Huawei, RAN4#115

TP for TS 38.194

## 6.2	Base station output power

### 6.2.1	General

The A-IoT BS conducted output power requirement is at antenna connector for A-IoT BS type 1-C.

The rated carrier output power of the A-IoT BS type 1-C shall be less than or equal to 38dBm.

## 6.4	Transmit ON/OFF power

### 6.4.1	Transmitter OFF power

- 6.4.1.1 General

Transmitter OFF power for A-IoT BS is defined as the mean power measured over 70/N us filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the BS (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

- 6.4.1.2 Minimum requirement for A-IoT BS type 1-C

For A-IoT BS type 1-C, the requirements for transmitter OFF power spectral density shall be less than -85 dBm/MHz per antenna connector.

### 6.4.2	Transmitter transient period

- 6.4.2.1 General

The transmitter transient period for A-IoT BS is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

Transmitter output power

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transmitter transient

period

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 6.4.2.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period

For A-IoT BS type 1-C, this requirement shall be applied at the antenna connector supporting transmission in the operating band.

- 6.4.2.2 Minimum requirement for A-IoT BS type 1-C

For A-IoT BS type 1-C, the transmitter transient period shall be shorter than the values listed in the minimum requirement table 6.4.2.2-1.

Table 6.3.2.2-1: Minimum requirement for the transmitter transient period for A-IoT BS type 1-C

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |