3GPP TSG-RAN WG4 Meeting #116														R4-2509443

Bengaluru, India, 25th – 29th August, 2025

Agenda item:	9.3

Source:	Apple

Title:	FR2 TX requirements verification in beam locked mode

Document for:	Discussion

# Introduction

In this contribution, we discuss two issues and propose how to fix them.

- Issue 1: In TS38.101-2 [1], TX requirements are specified with a statement on how to verify them, since all the requirements are verified through OTA testing. Furthermore, the TX requirements are verified in beam locked mode. However, the specification text on verification seems a bit inconsistent in that for some TX requirements, beam locked mode is not stated. In this contribution, we point out the inconsistency and propose to address it.

- Issue 2: In R18, the beam correspondence requirements in initial access and RRC_INACTIVE are derived under the condition that beamlock function is used when the requirements are verified. However, this agreement has not been captured in 38.101-2, although it is capture in TR38.891 [3]. Moreover, RAN5 sent an LS to RAN4 seeking clarification related to requirement verification [5]. We provide our views in this contribution.

# Discussion

Issue 1: Legacy specification inconsistency

To verify FR2 TX requirements, OTA methods are used. As a result, when a TX requirement is specified, it is important to also specify at what spatial direction the requirement is verified. This can be illustrated in the requirement of UE maximum output power for PC3, where the relevant text on requirement verification is highlighted in yellow. However, if one looks at the components of the maximum output power requirements, namely min. peak EIRP, max. TRP, max. EIRP, and spherical coverage, beam locked mode is mentioned for TRP only.

We have further checked other TX requirements and found that beam locked mode is mentioned for the following requirements:

- Minimum output power

- Transmit ON/OFF time mask

- Power control

- Frequency error

- Transmit modulation quality

- Occupied bandwidth

- Spectrum emission mask

- ACLR

- Spurious emissions

But not for the following requirements:

- Transmit OFF power

#### 6.2.1.3 UE maximum output power for power class 3 The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). ). The minimum output power values for EIRP are found in Table 6.2.1.3-1. The requirement is verified with the test metric of total component of EIRP (Link=TX beam peak direction, Meas=Link angle). The requirement for the UE which supports a single FR2 band is specified in Table 6.2.1.3-1. The requirement for the UE which supports multiple FR2 bands is specified in both Table 6.2.1.3-1 and Table 6.2.1.3-4. Table 6.2.1.3-1: UE minimum peak EIRP for power class 3 Operating band; Min peak EIRP (dBm). n257; 22.4. n258; 22.4. n260; 20.6. n261; 22.4. NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: Void. The maximum output power values for TRP and EIRP are found on the Table 6.2.1.3-2. The max allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and the total component of EIRP (Link=TX beam peak direction, Meas=Link angle). Table 6.2.1.3-2: UE maximum output power limits for power class 3 Operating band; Max TRP (dBm); Max EIRP (dBm). n257; 23; 43. n258; 23; 43. n260; 23; 43. n261; 23; 43.

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.3-3 below. The requirement is verified with the test metric of the total component of EIRP (Link=Spherical coverage grid, Meas=Link angle). The requirement for the UE which supports a single FR2 band is specified in Table 6.2.1.3-3. The requirement for the UE which supports multiple FR2 bands is specified in both Table 6.2.1.3-3 and Table 6.2.1.3-4. Table 6.2.1.3-3: UE spherical coverage for power class 3 Operating band; Min EIRP at 50 %-tile CDF (dBm). n257; 11.5. n258; 11.5. n260; 8. n261; 11.5. NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance NOTE 2: Void NOTE 3: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1..

Observation 1: In TS 38.101-2, beam locked mode is not stated for verification of all the TX requirements.

In our understanding, all the TX requirements should be verified in beam locked mode. This is also the understanding in RAN5, where all TX requirements are tested with UE Beamlock Function (UBF) activated.

Proposal 1: It is proposed to state in TS38.101-2 that requirements are verified in beam locked mode for min. peak EIRP, max. EIRP, spherical coverage, and transmit OFF power.

A CR is provided in [2].

Issue 2: The agreed use of beamlock function for R18 beam correspondence requirements in initial access and RRC_INACTIVE

The beam correspondence requirements in initial access and RRC_INACTIVE are derived in R18 under the condition that beamlock function is used when the requirements are verified. Although this agreement has not been captured in 38.101-2, it has been captured in Section 6.4 of TR38.891 [3], copied below:

- Beam lock function Currently, the beam lock function is only available in the RRC_CONNECTED state, and if the beam lock function is absent during the verification in RRC_IDLE, many complex issues will appear, e.g., when the RAR is held to trigger PRACH retransmission, UE may attempt other beams since the RAR is not received which may lead to unstable test. It also will be difficult to measure the θ (or φ) component EIRP under φ (or θ) link polarization without beam lock function. In [3], the LS from RAN5 concludes it is feasible to introduce beam lock function for RRC_IDLE, so the concern above can be solved. RAN4 concludes that dedicated beam lock function for RRC_IDLE state is needed, and the details of new beam lock function design can be left to RAN5.

Furthermore, the RAN4 agreement was sent to RAN5 by LS [4].

To our knowledge, in recent RAN5 discussion the RAN4 agreement of using beam lock function for testing seems to be in question. To move forward, RAN5 sent an LS to RAN4 as copied below, seeking RAN4 clarification.

1. Overall Description: RAN5 has discussed the test procedure and feasibility of 6.6.4 Beam Correspondence in RRC_INACTIVE requirements. RAN5 seeks the clarification from TS38.101-2 sub clause 6.6.4 Beam correspondence for power class 3 to support test procedure with UE Beamlock test function in the IDLE mode and without UE Beamlock test function in the IDLE mode. As stated in 6.6.4.1, UE shall meet the spherical coverage requirement for initial access and RRC_INACTIVE according to Table 6.2.1.3-3, with its autonomously chosen UL beams and without uplink beam sweeping, using the side conditions for initial access and in RRC_INACTIVE as defined in Clause 6.6.4.3.4. RAN5 seeks the following clarifications: - RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements? - Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams? - Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such? - Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined? 2. Actions: To RAN4 group. ACTION: RAN5 respectfully requests RAN4 to respond to the above questions.

We provide our views below for each question.

Q1: RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either  procedure be used to verify RAN4 core requirements?

RAN4 core requirements are specified under the assumption that UE beamlock function is used in testing. The reason to use beamlock function is to address the issues raised during the core requirement discussion, e.g., “when the RAR is held to trigger PRACH retransmission, UE may attempt other beams since the RAR is not received which may lead to unstable test” This is important as in the core requirement, there is no tolerance allowed.

Therefore, the conclusion we can draw is that test procedure with UE beamlock function is required to properly verify the RAN4 requirements, while it is uncertain whether test procedure without UE beamlock function can properly verify the RAN4 requirements.

Q2: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

It is RAN4 understanding that in real-field operation, UE chooses its UL beams based on DL measurements of BS TX beams, which is the reason of specifying beam correspondence requirements in RAN4 for UEs requiring no UL beam sweeping. Therefore, using beamlock function in testing does not prevent UE from choosing UL beams in the real field. Instead, beamlock function is used to ensure testing accuracy, reliability, and repeatability.

Q3: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

As said above, RAN4 core requirements are specified under the assumption that UE beamlock function is used in testing. Therefore, verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode is not aligned to verifying RAN4 core requirements when the requirements were defined.

# Conclusion

In this contribution, we pointed out that in 38.101-2 beam locked mode is not stated for the verification of some TX requirements. Furthermore, we answered the questions in the RAN5 LS. As such, we make the following proposal.

Observation 1: In TS 38.101-2, beam locked mode is not stated for verification of all the TX requirements.

Proposal 1: It is proposed to state in TS38.101-2 that requirements are verified in beam locked mode for min. peak EIRP, max. EIRP, spherical coverage, and transmit OFF power.

Proposal 2: To provide the following clarification to RAN5:

- Q1: RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either  procedure be used to verify RAN4 core requirements?

- RAN4 core requirements are specified under the assumption that UE beamlock function is used in testing. The reason to use beamlock function is to address the issues raised during the core requirement discussion, e.g., “when the RAR is held to trigger PRACH retransmission, UE may attempt other beams since the RAR is not received which may lead to unstable test” This is important as in the core requirement, there is no tolerance allowed.

- Therefore, the conclusion we can draw is that test procedure with UE beamlock function is required to properly verify the RAN4 requirements, while it is uncertain whether test procedure without UE beamlock function can properly verify the RAN4 requirements.

- Q2: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

  - Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

- It is RAN4 understanding that in real-field operation, UE chooses its UL beams based on DL measurements of BS TX beams, which is the reason of specifying beam correspondence requirements in RAN4 for UEs requiring no UL beam sweeping. Therefore, using beamlock function in testing does not prevent UE from choosing UL beams in the real field. Instead, beamlock function is used to ensure testing accuracy, reliability, and repeatability.

- Q3: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

- As said above, RAN4 core requirements are specified under the assumption that UE beamlock function is used in testing. Therefore, verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode is not aligned to verifying RAN4 core requirements when the requirements were defined.

# References

- TS 38.101-2, V15.26.0

- R4-2509445, "(NR_newRAT-Core) CR on adding beam locked mode for some TX requirements," Apple

- TR 38.891

- R4-2317764, LS on testability issues of beam correspondence requirement for initial access, RAN4

- R5-253653, LS on Beam Correspondence Initial Access, Apple