# 3GPP TSG-RAN WG4 Meeting #116	R4-2509500

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.24.5.2

Source:	Apple

Title:	On RRM core requirements for LP-WUR

Document for:	Discussion

# Introduction

In last RAN4#115 meeting, the WF for RRM requirements for LP-WUS/WUR was agreed in [1], and open issues have been summarized in [2].

In this contribution, we continue the discussion on the open issues in [1].

# General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

According to last meeting WF [1], this issue has been closed.

Issue 1-1-2: Threshold for switch between different cases

According to last meeting WF [1], this issue has been closed.

However, RAN2 still didn’t have clear conclusion about whether neighbor cell measurement can be triggered by LR measurement or not. Our understanding is LR measurement can be used for UE to decide switch from case#1 to case#3, that is, serving cell measurement on LR will be used to decide (1)if the neighbor cell measurement is triggered and (2)if the neighbor measurement can be relaxed. Previously in R16/17/18, the neighbor cell triggering and relaxation criteria checking is based on the MR measurement.

In previous RAN2 meeting it was agreed that,

- RAN2 #130 meeting - The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation), if there is such configuration. Capture this in the field description. Details will be discussed in the running CR. - It is up to NW to configure the condition for LP-WUS monitoring and/or [R19 serving/ neighboring cell RRM relaxation /R19 serving cell RRM offloading], as in the current RRC running CR. - The threshold of the MR based entry condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, which is the maximum of {SIntraSearchP, SnonIntraSearchP}. Capture this in the field description. Details will be discussed in the running CR. - RAN2 #129bis meeting - RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any). - It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell. - The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ. - How to define LP-RSRP and LP-RSRQ is up to RAN1.

This issue has been discussed in RAN4 for couple of meetings, but we cannot foresee RAN2’s plan on discussing this dedicated issue. Thus, one possible way is to directly send LS to ask for RAN2’s guidance.

Proposal 1: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1).

If RAN4 cannot make conclusion on issue 1-1-2, we recommend RAN4 sending LS to RAN2 for guidance.

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR serving cell measurement and MR RRM measurement relaxation

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

Issue 1-1-7: On jointly consideration on Criteria (entry/exit conditions) for fully offloading/MR RRM measurement relaxation and LP-WUS monitoring

According to last meeting WF [1], these issue 1-1-5/6/7 have been closed.

Issue 1-1-8: LP-WUR operating carrier frequency

According to last meeting WF [1], this issue has been closed.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

It was agreed in RAN4 #114 meeting, it was agreed that,

At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

It’s not possible to verify a UE behavior which is purely up to UE implementation, and when UE will turn on LR is either not testable. Thus, we agree that: at legacy state, when LP-WUR is “ON” for evaluating LR threshold, LP-WUR measurement and evaluation requirement will not apply.

Proposal 2: No LP-WUR measurement and evaluation requirement applies at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case).

Issue 1-1-10: Considerations on higher priority frequency layer

This issue has been concluded in last meeting WF [1].

Issue 1-1-11: RRM requirements for FR2

- Proposals - P1: Prioritize defining RRM requirements for FR1 in this WI (Apple Nokia) - P2: There is not any concrete proposal on FR2 requirements so far. Suggest RAN4 to confirm whether FR2 RRM requirement needs be defined in this release, or confirm the status of FR2 requirement will not impact the completion of WI (vivo, Nokia) Recommendations: Check whether P1 or P2 is agreeable.

In the RAN#105 plenary meeting, the FR2 LP-WUS scope has been discussed and agreed in [RP‑242361 Way forward on LP-WUS for FR2] as following:

It is proposed to endorse the following in RAN#105 for Rel-19 LP-WUS/WUR WI regarding FR2 support - Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible - No additional guidance in RAN#105 for RAN4 work and TU allocation

Since the FR2 is confirmed as a scope for LP-WUS WI, RAN4 needs to discuss (1) whether we need to design requirement for FR2 LP-WUR based RRM, and (2) when to start the discussion for such requirement design. Our understanding is FR2 LP-WUR based RRM requirement can be discussed after we concluded on the FR1 LP-WUR based RRM requirement, since FR2 case would be more complicated than FR1 by considering the Rx beam sweeping.

In RAN4 Idle/inactive mode requirement for SSB, the Rx beam sweeping has been applied for serving cell measurement and evaluation as following examples:

And RAN1 has agreement that,

Agreement If LP-SS is configured, the periodicity is configured with the candidate value set of {160ms, 320ms}. - A time offset is configured for the first LP-SS occasion with reference to SFN0 - If the periodicity is 160ms, the candidate value set for the time offset is {0, 1, …, 159}ms. - If the periodicity is 320ms, the candidate value set for the time offset is {0, 1, …, 319}ms.

In LP-SS based serving cell measurement, UE will follow the LP-SS periodicity (160ms or 320ms) to do the measurement instead of DRX in legacy case, then we need to decide if the same sweeping factor for different LP-SS periodicity shall be assumed. The most straightforward requirement could be that: the same Rx beam sweeping factor(N1=8) shall be applied for different LP-SS periodicities(160ms/320ms) since legacy case uses N1=8 for DRX cycle=320ms considering the mobility performance. All these points need to be discussed in RAN4 first, but RAN4#116 is the last meeting before planned core part completion. Thus, we propose to deprioritize FR2 requirement in R19 LP-WUR based RRM.

Proposal 3: FR2 LP-WUR based RRM requirement can be deprioritized in R19.

# Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

- Proposals - P1: Use 2.5 dB as the RF impairment margin for LP-RSRP accuracy requirements. (vivo Huawei Apple) - P2: Consider 3.5 dB RF impairment margin for LR (xiaomi)

The legacy RF impairment margin in FR1 MR requirement is 2.5dB and we think the same assumption can be used for LR requirement as well. Either proposal is acceptable to us.

Proposal 4: Use 2.5 dB as the RF impairment margin for LP-RSRP accuracy requirements.

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

According to last meeting WF [1], this issue has been closed.

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity

- Proposals - P1: No need to define upper bound for SSB-based LP-WUR measurement periodicity. (oppo xiaomi Nokia CT Huawei Apple) - P2: Prefer the periodicity for measurement and evaluation delay requirements for LP-WUR based on SSB in IDLE/Inactive mode is 1.28s if the LO periodicity is 2.56s. (vivo ZTE)

RAN4 agreed followings in the RAN4#114bis meetings(R4-2504908):

Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Thus, to just follow the previous agreement, there is no need to define any upper bound.

Proposal 5: No need to define upper bound for SSB-based LP-WUR measurement periodicity.

Issue 1-2-4-2: On LR requirements for entry/exit threshold evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3)

Issue 1-2-4-2-1: On applicability LR evaluation requirements

According to last meeting WF [1], this issue can be discussed in the CR.

Issue 1-2-4-2-3: On how to define LR evaluation requirements

The agreements in RAN4#115 were:

Working assumption: Option 1: using x1=2*x and y1=2*y for the evaluation requirement. Option 2: The value x1 for LP-SS based and y1 for SSB based LR evaluation requirement could be defined by adding extra [1 or 2 ] samples

The evaluation requirement (x1/y1) can be decided after we have simulation alignment for measurement requirement (x/y). Based on the observation of topic summary in previous meeting, the general sample number is 1~3 to meet the accuracy requirement, and whether and how UE to implement sliding window is purely to UE; thus, we can consider using x1=2*x and y1=2*y for the evaluation requirement regardless of whether UE is using sliding window for evaluation or not.

Proposal 6: The evaluation requirement (x1/y1) can be decided after RAN4 has simulation alignment for measurement requirement (x/y). Consider using x1=2*x and y1=2*y for the evaluation requirement.

# LP-WUR CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

Based on previous discussion, no RRM impact was observed for LP-WUR at CONNECTED mode.

# Others

Issue 1-5-1: LR based RRM with EMR in IDLE/inactive mode

- Proposals - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple Nokia) - P1-1: UE shall not enter relaxation / offloading if configured simultaneously with EMR. (Nokia) - P2: Postpone the discussion on the EMR issue (vivo) - P3: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact (Ericsson Nokia)

In RAN4 #111 and 112 meeting, the different cases for MR offloading and relaxation are captured in the WF R4-2410296 and R4-2413899, and so far, at least case#1 and case#3 are valid to consider in this WI,

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

However, the EMR measurement has not been considered yet for both case #1 and case #3. In legacy IDLE/Inactive mode RRM requirement, the relaxation is not allowed when EMR is configured on that frequency layer and T331 is running, like followings:

Then, if EMR measurement is also configured and TS331 is running for case#1 and case#3, the MR status may need further discussion. For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, 	UE may keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

Proposal 7: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

Issue 1-5-2: eDRX related

- Proposals - P1: RAN4 to discuss followings LP-SS based RRM issue in IDLE/Inactive mode: how to enter and exit offloading status if eDRX is configured with PTW. (Apple) - P2: Discuss whether RAN4 needs to introduce RRM requirements for LP-WUR operation with eDRX. (Qualcomm)

As discussed previously, if the LP-SS based measurement is not following eDRX cycle or DRX cycle in IDLE/Inactive mode, i.e., LP-SS based measurement will follow the LP-SS periodicity, then when UE decides to turn on MR inside a PTW window (if PTW is used), UE needs to check whether rest of the PTW window is sufficient for MR based RRM measurement or evaluation time (MR based RRM measurement/evaluation shall be contained in one PTW window). We think it’s still worthwhile to discuss how to perform offloading based on LP-SS based measurement when eDRX is configured.

Figure 1. example for MR offloading(based on LP-SS based measurement) when eDRX is configured with PTW

Proposal 8: RAN4 to discuss followings LP-SS based RRM issue in IDLE/Inactive mode:

- how to enter and exit offloading status if eDRX is configured with PTW.

New issue: LR based RRM when stationary or low-mobility criteria is configured for power saving feature

In RAN2, there were some agreements related with power saving criteria as followings:

In RAN2#127bis meeting, followings were agreed: - RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE - Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. In RAN2#129bis meeting, followings were agreed: - RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any). - RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both. - Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion). In RAN2#130 meeting, followings were agreed: - The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation), if there is such configuration. Capture this in the field description. Details will be discussed in the running CR. - It is up to NW to configure the condition for LP-WUS monitoring and/or [R19 serving/ neighboring cell RRM relaxation /R19 serving cell RRM offloading], as in the current RRC running CR. - The threshold of the MR based entry condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, which is the maximum of {SIntraSearchP, SnonIntraSearchP}. Capture this in the field description. Details will be discussed in the running CR.

The low mobility related relaxation was not concluded yet in RAN2, and UE behavior is unclear for relaxation if low-mobility criteria is configured and not met. If mobility condition/criteria are configured for UE power saving (either configured with not-at-cell-edge criteria or configured without NCE not-at-cell-edge criteria) and UE does not meet the low mobility condition, the UE behavior for relaxation shall be further discussed. Like in figure 2, UE is in case#1 based on the fulfilled condition of case#1, but UE does not meet the low mobility criteria from the UE power saving feature, we think it might be very risky from mobility perspective for UE to turn off the MR completely. Since there might be different UEs in cell coverage, some of them may support LP-WUR and power saving feature and some of them may only support power saving feature, network configures both criteria for MR offloading and UE power saving for different UEs via the system information, thus it’s worthwhile to investigate this case when low mobility criteria is configured but not met in case#1 and case#3. Since RAN4#116 is the last meeting before core part completion of this WI, RAN4 can discuss this issue in maintenance phase.

Figure 2. example for UE with high mobility in case#1

Proposal 9: in core maintenance phase, RAN4 to discuss the case when low mobility criteria is configured for UE power saving but not met in the LP-WUR based RRM case#1 and case#3.

# Conclusion

In this contribution, we continue the discussion on the open issues in [2].

Proposal 1: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1).

Proposal 2: No LP-WUR measurement and evaluation requirement applies at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case).

Proposal 3: FR2 LP-WUR based RRM requirement can be deprioritized in R19.

Proposal 4: Use 2.5 dB as the RF impairment margin for LP-RSRP accuracy requirements.

Proposal 5: No need to define upper bound for SSB-based LP-WUR measurement periodicity.

Proposal 6: The evaluation requirement (x1/y1) can be decided after RAN4 has simulation alignment for measurement requirement (x/y). Consider using x1=2*x and y1=2*y for the evaluation requirement.

Proposal 7: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

Proposal 8: RAN4 to discuss followings LP-SS based RRM issue in IDLE/Inactive mode:

- how to enter and exit offloading status if eDRX is configured with PTW.

Proposal 9: in core maintenance phase, RAN4 to discuss the case when low mobility criteria is configured for UE power saving but not met in the LP-WUR based RRM case#1 and case#3.

# References

[1] R4-2508280, WF on RRM requirements for NR_LPWUS, Moderator (vivo), RAN4 #115

[2] R4-2505572, Topic summary for [115][223] NR_LPWUS, vivo, RAN4 #115