3GPP TSG-RAN WG4 Meeting #116	R4-2509780

# Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on other RRM core requirements in NTN for NR Phase 3

Agenda Item:	7.27.5.2

Document for:	Discussion

# 1Introduction

In the last RAN4#115 meeting, the RRM impact on the SMTC enhancement discussion was triggered by RAN2 LS [1], and the discussion status was capture below based on the approved WF [2]. In this contribution, we would like to continue the discussion considering the latest RAN2 progress.

## Issue 0: LS from RAN2 on multiple SMTCs with different periodicities Agreement: (Ad-hoc) From RAN4 perspective, it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. - In the reply LS (to be prepared by Xiaomi), the following information highlighted in yellow for both intra- and inter-frequency will be added for reference: - FFS on the exact RRM requirement change. - FFS whether to limit the number of SMTCs to no more than 4 is desirable in an earth-moving cell deployment Agreement: online From RAN4 perspective, it is feasible to support SMTCs with up to 2 periodicities and different offsets but requires some clarifications/modifications to the existing RRM requirements. - RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer. Agreement: online The progress of RRM requirement definition for multiple SMTCs with up to two periodicities does not block the completion of RAN4 RRM core part of this WI.

- Discussion

Agreements in RAN2#129bis:

3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

Agreements in RAN2#130:

1.	the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue)

2.	To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

3.	We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

4.	No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme)

Working Assumption:

1.	We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options:

Alt 1: UE provide the closest N reference locations/neighbor cells to network.

Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

We understand the motivation of SMTC enhancement is to facilitate UE measurement due to beam hopping. Based on RAN2 progress listed above, two main enhancement was introduced:

- the configuration of two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode;

- the configuration of the maximum 6 SMTCs per frequency for idle/inactive UEs. Also RAN2 agreed to introduce location-based SMTC selection procedure to facilitate UE SMTC selection.

In the following, we analyze the exact SMTC enhancement impact on RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode from RAN4 RRM requirements perspective respectively.

RRC_CONNECTED mode

In RRC_CONNECTED mode, many requirements has a dependency on SMTC. However, the configuration of SMTCs with two different periodicities is not brand new. NR already support dual SMTC periodicity configurations since Rel-15, i.e. smtc1 and smtc2. The smtc2 only apply to intra-frequency measurement, and also has shorter periodicity than smtc1. And during Rel-15 time frame, RAN4 took complete discussion on how to reflect the dual SMTC configuration in the requirements.

Here below, we capture the related clarifications related to the R15 dual SMTC configuration:

- For requirements of handover, the parameter Trs, a component for the interruption time calculation, has dependency on SMTC. The current spec clarify that “If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell”.

- For requirements of RRC Re-establishment, the parameter TSMTC is used to drive the time to identify target NR cell. The current spec clarify that “If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell”.

- For requirements of RRC Connection Release with Redirection, the parameter of Trs used to derive the time to identify target NR cell has has dependency on SMTC. The current spec clarify with a NOTE1 in Table 6.2C.3.2.1-1 that “If the UE has been provided with higher layer signalling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell”.

- For requirements of RLM, LR (BFD and CBD) and L1-RSRP measurement, the scaling factor P introduced to account for the measurement opportunity sharing between measurement gap, SSB/CSI-RS and SMTC. It also has dependency on SMTC. For FR1, the related clarification is “If the higher layer in TS 38.331 [2] signalling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.”

- For requirements of L3 measurement, SMTC is used to drive the measurement delay for at least measurement without measurement gap case. The SMTC is clarified with a NOTE that “If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified”. Furthermore, for the requirement of cell identification, when UE does not know the PCI of the target cell, the clarification is “if the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1”.

Based on the above summary, the main clarifications in the requirement related to different SMTC periodicities are:

- When UE knows the PCI of the target cell, clarify the SMTC used in the related requirements is the one used by the cell being identified;

- When UE does not know the PCI of the target cell, i.e. during PSS/SSS detection, clarify which SMTC is supposed to be used. And in R15 dual SMTC configuration, smtc2, which has shorter periodicity, is used. We understand this selection is to make sure the PSS/SSS sync performance.

Observation 1: For the case of R15 dual SMTC configuration in RRC_CONNECTED mode, RAN4 specified the applicability condition to clarify the SMTC periodicity used in the related requirement.

The requirements for NR NTN inherit all the above-mentioned requirements. Even though the relationship between the two different SMTCs configured for NR NTN is not clear yet. But from our understanding, if we take the similar logic as for dual SMTC in R15, the spec impact could be similar.

Proposal 1: To support the SMTC enhancements on two different SMTC periodicities configuration in RRC_CONNECTED mode, RAN4 to update the applicability conditions for SMTC taken the same principle for dual SMTC introduced in Rel-15 as baseline.

Proposal 2: For handover requirements, RAN4 to add clarification on the parameter Trs that if the UE has been provided with different SMTC periodicities prior to the handover command, Trs follows the SMTC according to the physical cell ID of the target cell.

Proposal 3: For RRC Re-establishment requirements, RAN4 to add clarification on the parameter TSMTC that if the UE has been provided with different SMTC periodicities, TSMTC follows the SMTC according to the physical cell ID of the target cell.

Proposal 4: For RRC Connection Release with Redirection requirements, RAN4 to add clarification on the parameter of Trs that if the UE has been provided with different SMTC periodicities prior to the redirection command, Trs follows the SMTC according to the physical cell ID of the target cell.

Proposal 5: For requirements of RLM, LR (BFD and CBD) and L1-RSRP measurement in FR1, RAN4 to add clarification on the SMTC that if the UE has been provided with different SMTC periodicities, TSMTCperiod corresponds to the shortest value, otherwise TSMTCperiod corresponds to the value of smtc1.

Proposal 6: For of L3 measurement requirements of cell identification for both intra-frequency and inter-frequency, RAN4 to add clarification on the SMTC that if the UE has been provided with different SMTC periodicities, the assumed periodicity of SMTC corresponds to the shortest value, otherwise the assumed periodicity of SMTC corresponds to the value of smtc1.

RRC_IDLE/INACTIVE mode

In most of RRM requirements for RRC_IDLE/INACTIVE mode, DRX cycle is used to define the measurement delay. The SMTC periodicity is only used under some cases. Based on RAN2 agreements, we can see that the configuration of two different SMTC periodicities (with different offsets) in one frequency layer is only for neighbor cell measurements. So, we go through the requirements for RRC_IDLE mode, and capture the related text below:

#### 4.2C.2.3 Measurements of intra-frequency NR cells Table 4.2C.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). FR1 and FR2-NTN. 0.32; 1; 11.52 x N1 x M2 (36 x N1 x M2); 1.28 x N1 x M2 (4 x N1 x M2); 5.12 x N1 x M2 (16 x N1 x M2). 0.64; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). NOTE 1: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 4 upon more than 1 SMTC configured at the UE; M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; otherwise M2=1. Where, NSMTC is the number of SMTCs configured by SAN If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. NOTE 2: The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.. Table 4.2C.2.3-2: Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh DRX cycle length [s]; Tdetect,NR_Intra_enh [s] (number of DRX cycles); Tmeasure,NR_Intra_enh [s] (number of DRX cycles); Tevaluate,NR_Intra_enh [s] (number of DRX cycles). 0.32; 2.56 x M2 (8 x M2)Note 1; 0.32 x M3 (1 x M3) Note 1; 0.96 x M4 (3 x M4) Note 1. 0.64; 5.12 (8); 0.64 (1); 1.92 (3). 1.28; 8.96 (7); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). NOTE 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 2, M3 = M4 = 2.5.

#### 4.2C.2.4 Measurements of inter-frequency NR cells The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in table 4.2C.2.4-1 under the following conditions: - TSMTC_intra = TSMTC_inter = 160 ms; where - TSMTC_intra is the periodicity of the SMTC configured for the intra-frequency carrier if no identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier; TSMTC_intra is the periodicity of the smtc2-LP configured for the intra-frequency carrier if at least one identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed for TSMTC_intra. If the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. - TSMTC_inter is the actual SMTC periodicity used by the inter-frequency cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the inter-frequency carrier is assumed for TSMTC_inter. If the actual SSB transmission periodicity is greater than the SMTC configured for the inter-frequency carrier, longer Tdetect, NR_inter is expected. - SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and - SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion in TS 38.304 [1].

#### 4.2C.2.5 Maximum interruption in paging reception At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms. At inter-frequency cell re-selection with TN carrier, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms. ========omitted the unrelated part========= #### Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc.

As highlighted, in the text, the current spec already clarify UE behavior when UE is configured with different SMTC periodicities. We understand these clarifications are for smtc2-LP, which is introduced in Rel-16 TEI, at first. The smtc2-LP is configurable to both intra-frequency and inter-frequency measurement. Compare with the legacy smtc, the smtc2-LP can only be set with strictly larger periodicity values while same offset. UE will select either smtc or smtc2-LP based on whether a target neighboring cell in the PCI list is detected. Current applicability conditions clarify the specific SMTC which is supposed to be used in the requirements to make sure UE can satisfy the requirements.

Observation 2: For the case of smtc2-LP configuration in RRC_IDLE mode, RAN4 specified the applicability condition to clarify the SMTC periodicity used in the related requirement.

When the smtc2-LP is configured, UE would get total two sets of SMTC configurations with different periodicities but same offset. We can see the similarity between the function of smtc2-LP and the SMTC enhancements to be introduced in NTN, which support two sets of SMTCs with different periodicities and different offsets. From our perspective, to support the SMTC enhancements on two different SMTC periodicities configuration, RAN4 can follow the similar logic as for smtc2-LP in Rel-16. The spec impact is clear.

Proposal 7: To support the SMTC enhancements on two different SMTC periodicities configuration in RRC_IDLE/INACTIVE mode, RAN4 to update the applicability conditions for SMTC taken the same principle for smtc2-LP as baseline.

Proposal 8: For measurements of intra-frequency NR cells requirements, RAN4 to add clarification on the SMTC that during PSS/SSS detection, the shortest SMTC periodicity among all the SMTCs configured for the intra-frequency carrier is assumed.

Proposal 9: For measurements of inter-frequency NR cells requirements, RAN4 to add clarification on the TSMTC_intra and TSMTC_inter that during PSS/SSS detection, the shortest SMTC periodicity among all the configured SMTCs is assumed.

Proposal 10: For maximum interruption in paging reception requirements, RAN4 to add clarification on the Ttarget_cell_SMTC_period that the SMTC periodicity is according to the physical cell ID of the target cell.

RAN2 has agreed to support configuring 6 SMTCs per frequency for idle/inactive UEs. Based on current UE capability, UE can at most support 4 SMTCs per frequency. Going back to R17 discussion, RAN4 already considered the case that the number of configured SMTCs per Frequency layer is beyond UE capability. And the agreement was achieved for RRC_IDLE/INACTIVE mode that:

- For RRC Idle/Inactive mode: - UE can be configured with more SMTCs than its capability. - UE performs RRM measurement for the broadcasted SMTCs up to its capability in terms of the number of supporting SMTCs at a given period of time. - If the number of SMTCs broadcasted by NW is beyond the UE capability, UE may select SMTCs sequentially until all of the SMTCs can be measured . - The selection of SMTCs to be used is up to UE implementation, e.g. RSRP, equal priority, etc. - (note) longer measurement delay is expected, if UE is configured with more SMTCs than its capability

Based on the latest RAN2 progress, RAN2 agreed to introduction location based SMTC selection for RRC idle/inactive mode to help UE do the SMTC selection when configured with multiple SMTCs. From our perspective, as it is already agreed in RAN2 that it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode, and RAN4 also define no requirement for the case UE configured with more SMTCs than its capability, we think no update in RAN4 requirements is needed.

Proposal 11: No updates on the requirements for the scenario when UE is configured more SMTCs than UE capability in RRC_IDLE/INACTIVE mode is needed.

- Conclusion

Observation 1: For the case of R15 dual SMTC configuration in RRC_CONNECTED mode, RAN4 specified the applicability condition to clarify the SMTC periodicity used in the related requirement.

Proposal 1: To support the SMTC enhancements on two different SMTC periodicities configuration in RRC_CONNECTED mode, RAN4 to update the applicability conditions for SMTC taken the same principle for dual SMTC introduced in Rel-15 as baseline.

Proposal 2: For handover requirements, RAN4 to add clarification on the parameter Trs that if the UE has been provided with different SMTC periodicities prior to the handover command, Trs follows the SMTC according to the physical cell ID of the target cell.

Proposal 3: For RRC Re-establishment requirements, RAN4 to add clarification on the parameter TSMTC that if the UE has been provided with different SMTC periodicities, TSMTC follows the SMTC according to the physical cell ID of the target cell.

Proposal 4: For RRC Connection Release with Redirection requirements, RAN4 to add clarification on the parameter of Trs that if the UE has been provided with different SMTC periodicities prior to the redirection command, Trs follows the SMTC according to the physical cell ID of the target cell.

Proposal 5: For requirements of RLM, LR (BFD and CBD) and L1-RSRP measurement in FR1, RAN4 to add clarification on the SMTC that if the UE has been provided with different SMTC periodicities, TSMTCperiod corresponds to the shortest value, otherwise TSMTCperiod corresponds to the value of smtc1.

Proposal 6: For of L3 measurement requirements of cell identification for both intra-frequency and inter-frequency, RAN4 to add clarification on the SMTC that if the UE has been provided with different SMTC periodicities, the assumed periodicity of SMTC corresponds to the shortest value, otherwise the assumed periodicity of SMTC corresponds to the value of smtc1.

Observation 2: For the case of smtc2-LP configuration in RRC_IDLE mode, RAN4 specified the applicability condition to clarify the SMTC periodicity used in the related requirement.

Proposal 7: To support the SMTC enhancements on two different SMTC periodicities configuration in RRC_IDLE/INACTIVE mode, RAN4 to update the applicability conditions for SMTC taken the same principle for smtc2-LP as baseline.

Proposal 8: For measurements of intra-frequency NR cells requirements, RAN4 to add clarification on the SMTC that during PSS/SSS detection, the shortest SMTC periodicity among all the SMTCs configured for the intra-frequency carrier is assumed.

Proposal 9: For measurements of inter-frequency NR cells requirements, RAN4 to add clarification on the TSMTC_intra and TSMTC_inter that during PSS/SSS detection, the shortest SMTC periodicity among all the configured SMTCs is assumed.

Proposal 10: For maximum interruption in paging reception requirements, RAN4 to add clarification on the Ttarget_cell_SMTC_period that the SMTC periodicity is according to the physical cell ID of the target cell.

Proposal 11: No updates on the requirements for the scenario when UE is configured more SMTCs than UE capability in RRC_IDLE/INACTIVE mode is needed.

- Reference

- R4-2508288, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm

- R2-2503068, LS on SMTC enhancements, RAN2