3GPP TSG-RAN WG4 Meeting #116				                               R4-2509803

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.22.7

Title: 	Discussion RRM requirements test cases for Ambient IoT

Source: 	Xiaomi

Document for:		Discussion

- Introduction

The work item of Ambient IoT was approved in [1, RP-243326]. And in the las meeting, the corresponding RRM core requirements were agreed [2, R4-2504905]. In this paper we start the discussion on whether and how to define the test cases for AIoT RRM requirements.

- Discussion

In the last RAN4 meeting, RAN4 agreed to define the following RRM core requirements for AIoT device in Rel19.

Issue 1-1: A-IoT random access procedure Agreement: - RAN4 to include following aspects for RRM requirements for A-IoT random access. The details of UE behaviour will be updated based on RAN2 spec. - CBRA procedure - the correct behaviour when transmitting A-IoT MSG1(Random ID transmission) - the correct behaviour when receiving A-IoT MSG2 - the correct behaviour when not receiving A-IoT MSG2 - CFRA procedure - the correct behaviour when transmitting First D2R message Issue 1-2-1: Device transmit timing Agreement: - RAN4 define D2R transmit timing requirements. - Define D2R transmit timing error requirements as follows: - For MSG1 X=1, - The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1. - For MSG1 X=2, follow the same principle as starting point. - Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements. - Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any.

For the random access, it is necessary to define the test case to verify the correct measurement behavior. But for the timing issue, the expected performance impacts can be observed in the demod part.

Observation 1: The performance impact due to D2R transmit timing error (e.g. SFO) can be reflected by the demod test.

Thus, we can propose that:

Proposal 1:RAN4 can define the following RRM test cases.

- CBRA

- CBFA

According to the physical layer design [5] (e.g. SCS =15kHz only), the test cases with 15KHz SCS is needed only.

And for BW, since there are amount of TBW size for R2D, RAN4 needs to consider part of them to be tested in order to reduce the testing efforts.

Therefore, we can also propose the common testing configuration as:

Proposal 2: The common testing configuration for Rel19 AIoT RRM test cases can be defined as:

- SCS = 15KHz

- BW =[180kHz]

- Device 1 only

- Summary

This paper provided our consideration on RRM test cases for AIoT.

Observation 1: The performance impact due to D2R transmit timing error (e.g. SFO) can be reflected by the demod test.

Proposal 1:RAN4 can define the following RRM test cases.

- CBRA

- CBFA

Proposal 2: The common testing configuration for Rel19 AIoT RRM test cases can be defined as:

- SCS = 15KHz

- BW =[180kHz]

- Device 1 only

- References

- RP-243326

- R4-2504905

- 3GPP TR38.769

- 3GPP TS 38.133 v18.7.0

- 3GPP TS38.191 v19.0.0