3GPP TSG-RAN WG4 Meeting #116		R4-2509835

Bengaluru, India, August 25th – 29th, 2025

Agenda item:		7.20.5

Source:		Samsung

Title:	Discussion on RRM performance requirements of Rel-19 MIMO Phase5

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of performance part of this WI are described in [1] as:

- Specify necessary performance requirements for the enhancements listed above.

In last RAN4#115meeting, the open issues of RRM performance part of Rel-19 MIMO Phase 5 are captured in [2]. In this contribution, we provide our view of the RRM performance part of this WI.

- Discussion

In previous meeting, RAN4 discussed and achieved some consensus on RRM core requirements of Rel-19 MIMO.

The performance requirements usually can be divided into two parts: Measurement accuracy and Test cases.

- UE-initiated/event-driven beam management

Measurement accuracy

For measurement accuracy for UEIBM, we have already achieved the agreements in [2]:

#### Issue 2-1-1: Measurement accuracy Agreement: - Confirm previous agreement in RAN4#114. No further discuss is needed.

For the test case of L1-RSRP measurement accuracy, we think it can be combined with the reporting delay together in the same test. No dedicated TC are needed.

Test cases

In last RAN4#115meeting, the test scope issues are captured in [2]:

#### Issue 2-1-2: Test scope <Way forward > - Further discuss based on the table below: QC; Nokia; Samsung; Apple; CMCC; Huawei; Ericsson. High level: Define TCs to verify UEIBM; Y; Y; Y; Y; Y; Y; Y. Whether to cover Event-2/Event-1/Event-7?; Y; Event-2 1 TC for Event-2 No for Event-7; Y; Event-2 FFS on others; Event-2&7. Whether to cover Mode A and Mode B?; Mode A FFS on Mode B; Y; Mode A at least. Event-trigger for Intra-cell BM for SSB; Y; Y; Y; Y; Y. Event-trigger for Intra-cell BM for CSI-RS; Y; Y; Y; Y; Y. Event-trigger for Inter-cell BM for SSB; Y; Y; Y; FFS; ?. Event-trigger for Inter-cell BM for CSI-RS; FFS. Whether to cover no time window and eventInstanceCount-r19 > 1?; Y; Y; Y. FR2?; Y. Beam switching delay for TCI states switching; Y Both known and unknown TCI; No MAC CE-based TCI state switching FFS on DCI-based TCI state switching.

According to the core requirements, RAN4 to add new core requirements for both SSB and CSI-RS based event triggered L1-RSRP measurement and event triggered reporting delay.

RAN4 to discuss the test coverage of UEIBM, at least the following should be tested:

- sTRP serving cell SSB based L1-RSRP

- sTRP serving cell CSI-RS based L1-RSRP

- sTRP additional PCI cell SSB based L1-RSRP

For the test coverage of different events, prioritize to test event-2. We are open to choose limited TCs for event-1. For event-7, we prefer not to add test case for it.

For the testability of event-7, we don’t think legacy chamber for FR2 test can be used for event-7.

First of all, in RAN1 and RAN4 RRM test, the event-2 is the prioritized one.

  - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

  - The number of N beams (new) can be {1, 2, 3, 4} which is configured

In OTA test, one TCI states means one beam in FR2.

In typical legacy Chamber, for example, there are only 4 fixed probes with fixed separation angel.

Left: Keysight                                                          Right: R&S

Even in Rel-18 MIMO OTA, for FR2 MIMO OTA testing, 3D Multi-Probe Anechoic Chamber (3D-MPAC) was introduced by 6 probes. The 6 probes are designed with different power weights and delay to simulate the required channel model at the test zone. The probes are more like a quasi 1AoA which is not suitable for more AoA setting in RRM tests.

Figure B.2.1-1: 3D MPAC system layout for NR FR2 MIMO OTA testing

Although we haven’t agreed details of how to test event-2, the most possibility or our proposal is that RAN4 will design two beams: one is for current beam, the other is for new beam. Then UE estimates the L1-RSRP for current beam and/or new beam and trigger the event report to TE. No matter the tests focus on L1-RSRP only or TCI state switching, the testability has no issue for FR2 chamber with 4 probes with 2 active probes if the number of new beams is limited.

In Rel-15/Rel-16/Rel-17 RRM test, RAN4/RAN5 usually used single probe or up to 2 active probes for up to 2 beams in FR2.

if 2AoAs (Rel-15, e.g. Setup.3),

30 degrees, = “1&2”

60 degrees, = “2&3” or “3&4”

90 degrees, = “1&3”

120 degrees, = “2&4”

150 degrees, = “1&4”

In Rel-18 multi-Rx WI and MIMO WI, RAN4 discussed whether to use more AoAs for more beams. (TR38.751 for RF)

If 3AoAs from 4 fixed probes,

1&2&3 = 30(1 to 2) +90 (1 to 3)

1&2&4 = 30(1 to 2) +150(1 to 4)

1&3&4 = 90(1 to 3) +150(1 to 4)

2&1&3 = 60(2 to 3) +90 (1 to 3)

2&3&4 = 60(2 to 3) +120(2 to 4)

1&3&4 = 60(3 to 4) +150(1 to 4)

1&3&4 = 90(1 to 3) +150(1 to 4)

1&2&4 = 120(2 to 3) + 150 (1 to 4)

30+90, 30+150, 60+90, 60+120, 60+150, 90+150, 120+150 are valid AoAs.

The definition of event-7 in RAN1 is:

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

  - Q is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

In Rel-17 MIMO unified TCI framework, the activated TCI states can be up to 8. Then Q is configured by gNB. For example, find the 3rd best from the 8 TCI states which means UE shall evaluate the L1-RSRP for all active TCI states then pick up which is the 3rd best.

If the event-7 is using just two active TCI states, it is the same as event-2. Q-th can be 1st or 2nd. Otherwise, it’s hard to support more beams from 4 fixed probes.

Therefore, we have concern for testability for event-7.

For the test coverage of modeA and modeB:

RAN1 introduced both ModeA and ModeB as:

Mode A (dynamically scheduling UCI by gNB)

Mode B (UCI in pre-configured resource(s) for second UL channel)

Mode A is for UE request an uplink resource for beam report while mode B uses the pre-configured resource for beam report. In the UE feature list discussion, mode A is the basic UE capability. Mode B depends on UE capability. However, mode B is quite typical used due to it can save the scheduling info. We support to cover both mode A and mode B.

In core requirements discussion, it is still open whether to define such core requirements if eventInstanceCount-r19 > 1. We can wait for decision for the core requirements. But even there is no core requirements if eventInstanceCount-r19 > 1, RAN4 can design TCs to verify it.

Observation 3: The testability of Event-7 has issue by using existing chamber for more AoAs.

Proposal 1: RAN4 to add new test cases to cover:

- sTRP serving cell SSB based L1-RSRP

- sTRP serving cell CSI-RS based L1-RSRP

- sTRP additional PCI cell SSB based L1-RSRP

Prioritized for the tests are for event-2. We are open to choose limited TCs for event-1.

No TCs are added for event-7.

TCs are defined to cover both Mode A and Mode B.

FFS on whether to define TC if eventInstanceCount-r19 > 1. We support to define such TC no matter there is core requirements or not.

Specify the test cases for FR2 test.

- CJTC reporting

In last RAN4#115 meeting, the open issues are captured in [2]:

#### Issue 2-2-1: Whether to define reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P? <Way forward > - Option 1: (Samsung, Apple, CMCC) - RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P. #### Issue 2-2-2: Whether to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P? <Way forward > - Option 1: (Samsung) - RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F. - Option 2: (Apple) - RAN4 to discuss feasibility of defining accuracy requirements for time offset, frequency offset reporting based on number of samples agreed in core part. - Option 3: (Ericsson) - Develop accuracy requirements for delay, frequency and phase offset UE measurement reports for: - Different number of samples - High and low SNR - Normal and extreme conditions Issue 2-2-3: Test scope <Way forward > - Further discuss based on the table below: High level:; QC; Nokia; Samsung; CMCC; Huawei; Ericsson. Whether to define TCs to verify CJTC-Dd, measurement period; Y; Y; Y; Y; FFS; Y. Whether to define TCs to verify CJTC-f，measurement period; Y; Y; Y; Y; FFS; Y. Whether to define TCs to verify CJTC-P， (To be verify?); Y; FFS; Y. Whether to define TCs to verify scheduling restriction; Y; FFS.

Firstly, for the performance part, the report mapping tables are needed to reflect the new measurements defined in RAN1 for CJTC-Dd, CJTC-F, CJTC-P.

Proposal 2: RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P.

RAN4 introduced the core requirements for delay and frequency offset, UE shall measure and report the measurement during measurement period and pass accuracy test.

Proposal 3: RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F.

For the TCs design for CJTC reporting, RAN4 can add TCs to verify the measurement period and accuracy in the same test.

Proposal 4: RAN4 to add TCs to test measurement period and accuracy in the same test. The TCs covers CJTC-Dd and CJTC-F.

Proposal 5: RAN4 not to add accuracy requirements and TCs for CJTC-P.

- Asymmetric DL sTRP/UL mTRP scenarios

In last RAN4#115 meeting, the open issues are captured in [2]:

### Sub-topic 2-3: Performance requirements for enhancement for Asymmetric DL sTRP / UL mTRP #### Issue 2-3-1: Test scope <Way forward > - Further discuss based on the table below: High level:; QC; Nokia; Apple; Samsung; CMCC; Huawei. Whether to define TCs to verify timing requirements?; Y; Y; Y; Y. Whether to define TCs to verify TCI state switching requirements?; Y; N. FR1 and/or FR2?; both. Whether to cover Scenario#1 and Scenario#2?; both; both; both; both; Scenarios#1.

In this objective, two TAs are enhanced for the UL-TRPs for core requirements. For the TCI state switching, the legacy requirements are reused, no need to add new test cases.

Proposal 6: RAN4 to add TCs for two TA core requirements verification. No need to add TCs for TCI state switching.

- Conclusion

In this contribution, we analyze the potential impacts on RRM performance requirements for Rel-19 MIMO and provide our proposals:

Observation 1: The performance requirements focus on other 3 objectives including UEIBM, CJTC reporting, enhancement for asymmetric DL sTRP/UL mTRP scenarios.

Observation 2: The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements.

Observation 3: The testability of Event-7 has issue by using existing chamber for more AoAs.

Proposal 1: RAN4 to add new test cases to cover:

- sTRP serving cell SSB based L1-RSRP

- sTRP serving cell CSI-RS based L1-RSRP

- sTRP additional PCI cell SSB based L1-RSRP

Prioritized for the tests are for event-2. We are open to choose limited TCs for event-1.

No TCs are added for event-7.

TCs are defined to cover both Mode A and Mode B.

FFS on whether to define TC if eventInstanceCount-r19 > 1. We support to define such TC no matter there is core requirements or not.

Specify the test cases for FR2 test.

Proposal 2: RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P.

Proposal 3: RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F.

Proposal 4: RAN4 to add TCs to test measurement period and accuracy in the same test. The TCs covers CJTC-Dd and CJTC-F.

Proposal 5: RAN4 not to add accuracy requirements and TCs for CJTC-P.

Proposal 6: RAN4 to add TCs for two TA core requirements verification. No need to add TCs for TCI state switching.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2508271, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung