3GPP TSG-RAN WG4 Meeting #116						      R4-2509934

Bengaluru, India, August 25th – 29th, 2025

Source:	Sony

Title:	Further Views on MPR Reduction

Agenda item:	7.1.2.2.1

Document for:	Approval

# Background

In RAN4#115, the enhancement of the power domain via relaxed ACLR has been further discussed [1], and a WF [2] is agreed, where it is agreed that the boundary and integral region of OOBE will be based on original UE CBW rather than the extended UE CBW to meet the ITU requirement.

The major remaining issues are how to determine the extended CBW and how the IBE can be applied, where our views on how to resolve the remaining issues are discussed in this contribution.

# Extended RB numbers and UE capability

For the extended RB number, the following agreement has been made in RAN4#115:

- Except for 1/2, only considering 1/4 as the extension ratio, i.e., 0.25*NRB and/or 0.25*CBW

Moreover, for the UE capability, the follow agreement has been made:

  - No separate UE capabilities for scenario 1 and scenario 2

  - Optional UE capabilities to indicate whether it supports the CBW extension based MPR reduction feature

    - Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with symmetric and asymmetric 1/2*UE CBW extension.

    - Capability 2: Indicates whether UE supports MPR reduction for single UL carrier with symmetric and asymmetric 1/X*UE CBW extension, where 1/X is a set of ratios smaller than 1/2 [{1/4, …}].

      - Ratio number depends on discussion of issue 1-1-1

As discussed in our previous contributions, adding more configurations of the extended RB number less than 50% UE's original bandwidth would inevitably add more effort on testing and verification, with very little gain in MPR reduction. Therefore, the set of additional values needs to be well justified for a clear purpose.

To set the capability properly, the following definition of capability can be considered:

  - Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with 1/2*[UE CBW] extension for dual-sided symmetric and single-sided case

    - {1/2, 0}, {0, 1/2}, {1/2, 1/2}

  - Capability 2: Indicates whether UE supports MPR reduction for a single UL carrier with 1/X*UE CBW extension for dual-sided symmetric and single-sided cases, where 1/X is a set of ratios equal to and smaller than ½

    - {1/2, 0}, {0, 1/2}, {1/2, 1/2}, {1/4, 0}, {0, 1/4}, {1/4, 1/4}

Considering that each frequency band may have different region requirements that could prohibit MPR reduction, it is proposed that UE capability be defined by band capability.

Proposal 1: UE capability on MPR reduction shall be defined as per band capability.

In addition, the network should be able to enable or disable the feature in the field. Therefore, an RRC or MAC-based signalling should be introduced for this purpose, while the UE shall follow the network command to apply either the original MPR or the reduced MPR.

Proposal 2: The feature in the field shall be controllable by the network to determine whether it is active or inactive.

# Guard band

The guard band for the extended UE channel bandwidth was also discussed in RAN4#114bis. To some extent, the extended UE bandwidth already serves as a guard band, shifting the SEM and ACLR further away from the UE transmission bandwidth. The motivation behind the guard band proposal in our understanding is to ensure the ACLR and SEM is actually shifted far enough to guarantee the UE can apply the inner MPR. Therefore, if needed, the so-called "guard band" can be included in the number of extended UE bandwidth, and no need to define an explicit guard band in this case.

Proposal 3: If needed, the so-called "guard band" can be included in the number of extended UE bandwidth, and no need to define an explicit guard band.

# IBE

As previously agreed, IBE will be applied to the edge between the original UE CBW and the extended UE CBW. However, the exact IBE limitations are still under debate. For (e)Redcap UE, it does not apply any IBE requirement out of 20 MHz CBW. Therefore, it is somewhat unusual that, with the intention of relaxing the emission limit, we are actually introducing additional requirements on (e)Redcap. Meanwhile, a (e)Redcap UE is capable of meeting SEM, which is a more stringent requirement in the extended UE CBW without any MPR reduction. Now, with just a roughly 1 dB MPR reduction, it would likely be within any IBE limits, even without explicit specification. Therefore, as an alternative, RAN4 may consider do not introducing any IBE requirement in the region of the extended UE CBW.

Proposal 4: RAN4 can consider not introducing any IBE requirement in the extended UE CBW at least for (e)Redcap UEs.

# Test reduction

For testing purposes, some test reduction was proposed in previous meetings. However, it is challenging to determine how to reduce the number of tests, as each RB configuration requires a different MPR. Therefore, neither the test setup nor the core requirement is related to the other, and it is challenging to do any test reduction in our view.

Proposal 5: No test reduction can be applied, as the MPR value remains unchanged under different RB configurations.

# Conclusion

In this contribution, we provide our views on the MPR reduction via modified out-of-band emission limits, where the following observations and proposals are given:

Proposal 1: UE capability on MPR reduction shall be defined as per band capability.

Proposal 2: The feature in the field shall be controllable by the network to determine whether it is active or inactive.

Proposal 3: If needed, the so-called "guard band" can be included in the number of extended UE bandwidth, and no need to define an explicit guard band.

Proposal 4: RAN4 can consider not introducing any IBE requirement in the extended UE CBW at least for (e)Redcap UEs.

Proposal 5: No test reduction can be applied, as the MPR value remains unchanged under different RB configurations.

# References

- R4-2507555	 	Topic summary for [115][116] NR_ENDC_RF_Ph4_part1	Moderator (Huawei)

- R4-2507930		WF on NR_ENDC_RF_Ph4_part1	Huawei