3GPP RAN4 Meeting #116		R4-2509966

Bangalore, India, 25th – 29th August 2025

Agenda item:	11

Source:	Apple

Title:	Views on 29dBm high-power handheld devices for the FDD terrestrial and satellite bands in Rel-20

Release:	Rel-20

Document for:	Information

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. That time the only available power class for NTN was PC3 offering the maximum nominal output power of 23dBm. During the RAN#104 meeting a new WI was agreed [2] with one of the objectives to enable higher power classes for NTN.

During the previous meetings RAN WG4 discussed which power classes and UE architectures should be considered for the Rel-19 normative work. While it was agreed quickly to specify 26dBm (PC2) power class for NTN, there were ongoing discussions on how to enable 29dBm power class for NTN. At the end it was concluded to specify 29dBm power class only for the non-handheld devices postponing handheld discussion to Rel-20.

As discussed during the RAN#108 meeting, RAN WG4 will aim at developing further the NTN HPUE track whereupon it is anticipated that 29dBm power class will be considered in Rel-20 for the NR NTN devices. Thus, in this discussion paper we present our general view on technical challenges associated with the existing 29dBm power class, which can be addressed in the corresponding Rel-20 WI.

# 2	Discussion

During the previous RAN WG4 meetings companies discussed which power classes should be considered further for the NTN core specifications and under which UE RF architecture assumptions. It is worth noting that the existing NTN power class 3 has 1TX architecture that is common for both terrestrial and satellite bands FDD bands. Similarly, the existing power class 2, which also has 1TX architecture, can be easily extended from the existing terrestrial FDD bands to the satellite FDD bands.

There was also a discussion on whether and how to enable the PC1.5 power class for the NTN bands, for which several major concerns were raised. Firstly, this power class is defined only for the terrestrial TDD bands and is not applicable to the FDD bands. Secondly, since the PC1.5 power class assumes 2TX architecture, the resulting 2TX-based performance can be acceptable for the terrestrial FDD bands but could be quite sub-optimal for the FDD satellite links. The matter is that 2TX architecture might require configuring UL MIMO, performance of which is questionable for the satellite channels. Another important aspect to consider is the fact that 2Tx architecture and UL MIMO will require CP-OFDM, power back-off values for which are larger than DFT-s-OFDM. So, it could be the case that 26+26dBm configuration will perform even worse than a single 26dBm chain in case of the satellite channel. And referring specifically to the satellite FDD bands, most of them have strict emission requirements requiring A-MPR for some allocations. As already presented for some NTN bands, such as band n254 and n256, CP-OFDM has much larger A-MPR values. Based on that there is no clear evidence that PC1.5 based on 2TX with UL MIMO and CP-OFDM will perform better than e.g. PC2 1TX with DFT-s-OFDM. One potential direction that can be explored further is to use single-stream DFT-s-OFDM with 2Tx, final performance of which should be checked further because of the delays on the feedback channel.

There were also discussions on whether it would be possible to consider 29dBm power class based on the 1TX architecture. However, companies were not able to conclude on availability of the corresponding PA for the portable and handheld devices operating in the FDD bands.

Nevertheless, since there is an interest from the industry to enable 29dBm transmission power for the NTN devices, we suggest studying it further in the follow-up work once RAN WG4 completes the work on PC2 and PC1 classes. In fact, since most of the FR1 NTN bands are FDD the 29dBm power class can be an interesting option also for the terrestrial FDD bands. Thus, our view is to consider the corresponding Rel-20 WI that can enable 29dBm for both terrestrial and satellite FDD bands.

Proposal 1:	For Rel-20, consider enabling the 29dBm power class for FDD covering both terrestrial and satellite bands.

Proposal 2:	There can be a study phase to conclude on which architectural options(s) can be suitable for 29dBm in FDD bands.

# 3	References

RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"