3GPP TSG-RAN WG4 Meeting # 116	R4-2510008

Bengaluru, In, May 25 – 29, 2025

Source:	LG Electronics

Title:	Discussion on Power domain enhancement for single carrier

Agenda item:	7.1.2.2.1

Document for:	Discussion

Introduction

In 3GPP RAN4 #115, WF[1] was approved. In this paper, we provide our views on Power domain enhancements for single carrier.

- Topic #1: Power domain enhancements for single carrier - Sub-topic 1-1: Approaches to enable MPR reduction - Issue 1-1-1: Extension values/ratios - Agreement in main session: - At least ½ and ¼ are introduced as the extension ratio to apply to both single-sided and dual-sided cases - It is FFS on the details of symmetric and asymmetric cases for dual-sided case where non-zero extension is applied for both sides. - It is FFS on details of the single side case. - Issue 1-1-2: Guard band for extended CBW - WF - GB for extended CBW is a virtual concept to consider the boundary applying the shifted OOBE, which is not reflected in the specification and it can be calculated by extended CBW, UE CBW, NRB and extended NRB - Extended CBW is not limited to the existing UE CBW - RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band (close to the extended BW) given the relaxed extended GB and preliminary measurements performed suggests - Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - The following approach is considered as starting point to be further discussed in next meeting, notation should be further considered as well - For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below: - RBStart,Low_new = max(0, RBStart,Low – RBshift_low); - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB); - RBStart,Low_new ≤ RBStart ≤ RBStart,High_new, and LCRB ≤ ceil((NRB+ RBshift_low + RBshift_high)/2); - RBshift_low, RBshift_high ≤ NRB/2 - Other approach is not precluded - The approach shall be generic to cover all possible extension cases, including symmetric, asymmetric and different extension ratios - Sub-topic 1-2: Applicable requirements - Issue 1-2-1: Derivation of the extended IBE - Agreement in main session: - Fixed IBE used for extended CBW at each side of UE CBW as the baseline, the value is derived from the outmost allocated RB of UE CBW (illustrated in a figure to be developed) - The details and the related test cases can be discussed and decided later. - If significant issue is identified, other options can be further considered. - WF Further discuss whether to apply the extended IBE in the GB of extended CBW. - Option1: Yes - Option2: No - Issue 1-2-2: Relaxation of the extended IBE - Options - Option 1: using BPSK EVM in the IBE formula for all extended RBs - Option 2: no relaxation is needed. - WF - If there is no obvious system performance impact observed, option 1 is adopted - Sub-topic 1-3: Signaling - Issue 1-3-1: NW signaling - Agreement in main session: - The power enhancement feature is enabled by the gNB RRC signalling according to UE capabilities, - The details of the signaling including the contents are FFS - WF - It is up to the NW to judge the gap width on both sides of the UE - NW sends info of extension ratio (at least 1/2) per side - Issue 1-3-2: UE capability - Agreement in main session: - No separate UE capabilities for scenario 1 and scenario 2 - Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with 1/2*UE CBW extension only - Both dual-sided and single sided cases are supported. - Capability 2: Indicates whether UE supports MPR reduction for single UL carrier with both ½ and 1/4*UE CBW extension - Both dual-sided symmetric and single sided cases are supported. - For dual-sided case, it is FFS if asymmetric cases and a separated capability is introduced. - Note: the other ratio number depends on discussion of issue 1-1-1

Discussion

Derivation of new inner RB region in extended CBW

At original UE CBW, inner region have decided with follow equation.

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB – RBStart,Low – LCRB

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

Each parameter at above equation mean LCRB is Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks. NRB mean Transmission bandwidth configuration, expressed in units of resource blocks. Others parameter is coming based on NRB and LCRB.

Figure1. NRB and LCRB definition

According to NRB , LCRB and RBstart, RB allocation was decided whether it is outer region or inner region or edge region. NRB , LCRB and RBstart was decided at each UE Channel. RB allocation is automatically decided with following plot.

Figure2. RB allocation example (SCS=15KHz, UE CBW=20MHz, NRB=106)

If RB extension was applied to original UE Channel, some parameters related original UE CBW is same at extension CBW case. LCRB is same with original UE CBW. It cannot extend to extended UE CBW. LCRB should be limited with LCRB≤NRB. It is only changed with NRB and RBStart. New NRB to derive the new inner region  could be starting point as follow.

NRB,extension=NRB+ RBLow,extension+RBHigh,extension

Proposal 1: New NRB to derive the new inner region could be defined as follow

NRB,extension=NRB+ RBLow,extension+RBHigh,extension

Until last meeting, RAN4 agree that extension channel bandwidth should be extended with RB based. Because RAN4 need to consider how network indicate the information related to extension channel bandwidth. When network indicate channel bandwidth size to UE for extension. it is necessary to discuss more how UE decide the RB allocation whether it is outer region or inner region. Network indication like (RBLow,extension, NRB,extension), RB allocation could be easily changed. If network indicate this information(RBLow,extension, NRB,extension), new RB allocation could be calculated based on RBLow,extension, NRB,extension. It could be one of the method to convert from outer region to inner region. NRB,extension was used instead of NRB. And RBStart+ RBLow,extensionwas used instead of RBStart. Following as it was changed, (X,Y) at legacy UE CBW can be converted at the extension UE CBW dimension. It can be noted according to our companion paper[2].

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB,extension – RBStart,Low – LCRB

RBStart,Low  ≤  RBStart +RBLow,extension ≤  RBStart,High, and

LCRB  ≤  ceil(NRB,extension/2)

Figure3. Operation based on BS indication

We consider network indication when converting outer RB allocation to inner allocation as below

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB,extension – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart +RBLow,extension ≤  RBStart,High, and

LCRB  ≤  ceil(NRB,extension/2)

Proposal 2: Consider network indication when converting outer RB allocation to inner allocation as below;

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB,extension – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart +RBLow,extension ≤  RBStart,High, and

LCRB  ≤  ceil(NRB,extension/2)

Symmetric and asymmetric cases

For assymmetric extension, it is necessary to verify the extent to which the outer RB region changes into the inner RB region. The following trend exists. As previously discussed, when both sides are extended by half the UE CHBW, the existing outer RB region is converted into the inner RB region as follows.

Fig1. 1/2 extension of RB UE CHBW

Therefore, if the expansion on both sides is done by 1/2 UE CHBW, the outer RB region is already changed to the inner RB region, so there is no great meaning in changing the extension further to the inner RB region. However, if the value below 1/2 UE CHBW RB is done symmetrically or assymetrically, the change will be seen as follows.

Fig 2. Assymetric extension at right side(UE CHBW=10MHz)

As above, the ratio of changing to the inner region is maintained from the expansion from a specific extension area. In other words, when only one side is expanded unilaterally, the ratio of changing to the inner RB region is maintained when the expansion exceeds a certain value.

Fig 3. Assymetric extension at left side(UE CHBW=10MHz)

As shown above, when the extension is performed only in the left direction, the ratio of the outer RB region changing to the inner RB region is maintained when the extension exceeds a certain value, just like when the extension is performed only to the right. In other words, channel extension in only one direction does not affect the conversion to the inner RB region beyond a certain value. Therefore, we found that when the UE CHBW is extended in only one direction, the inner RB region conversion ratio is higher than when no extension is performed, but extension beyond a certain value is meaningless. In other words, extension is more beneficial for expanding the inner region than no extension. It seems impossible to perform these tests for all cases at this point. Due to time constraints, do not consider asymetric extensions in Rel-19.

Proposal 3: Due to time constraints, do not consider asymetric extensions in Rel-19.

- 2. Conclusion

In this contribution, we want to share some views on Power domain enhancements for single carrier and proposals are made as following:

Proposal 1: New NRB to derive the new inner region could be defined as follow

NRB,extension=NRB+ RBLow,extension+RBHigh,extension

Proposal 2: Consider network indication when converting outer RB allocation to inner allocation as below;

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB,extension – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart +RBLow,extension ≤  RBStart,High, and

LCRB  ≤  ceil(NRB,extension/2)

Proposal 3: Due to time constraints, do not consider asymetric extensions in Rel-19.

- 3. Reference

[1] R4-2507930, “WF on NR_ENDC_RF_Ph4_part1”

[2] R4-2510011, “draftCR on MPR applicability for single carrier”