3GPP TSG-RAN WG4 Meeting # 116		R4-2510033

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.27.5.1

Source: 	CMCC

Title: 			   Discussion on the RRM requirement of (e)Redcap UE for NTN phase3

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed (e)RedCap UEs over NR FR1-NTN. The WF has been approved in [1].

In this contribution, we further discuss the RRM impact and specific requirements.

- General consideration on RRM requirements

  - What RRM requirements are defined for (e)RedCap UE with FR1-NTN

#### Issue 1-1-2: MDT requirements

- Background

The agreement in the previous meeting:

RAN4#114: #### Issue 1-1-3: MDT requirements Agreement: - For MDT requirements in RRC_IDLE and RRC_INACTIVE state, - Deprioritize to define Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN.

<Way Forward>:

  - Proposal 1: Not to define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap with NTN.

  - Proposal 2: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN. The legacy requirement from NTN can be reused.

  - Proposal 3: RAN4 to discuss whether to send LS to RAN2 / RAN3 informing them about the RAN4 agreement on specifying MDT requirements in RRC_IDLE and RRC_INACTIVE for (e)RedCap UE supporting MDT.

  - Proposal 4:

    - RAN4 shall consider the necessity of defining MDT requirements and then decide whether to define relevant MDT requirements.

    - For measurement requirements, the legacy measurement requirements shall be reused for 1Rx and 2Rx RedCap UEs in NTN.

    - For accuracy relative time stamp requirements, RAN4 shall reuse the same drift of relative time stamp for 1Rx and 2Rx RedCap UEs in NTN.

We think it is beneficial for NTN network operation to support MDT capability for Redcap UE over NTN.

Firstly, for NTN network with large/moving cell coverage, it is barely impossible to use the traditional drive test, which requires support from both operation and testing equipment, the difficulty and cost of network optimization will be increased a lot. While MDT could help network to optimize the scheduling with low cost.

Secondly, for large cell coverage, especially for lonely area, it is hard to assume that network can acquire MDT report from handheld UE, the Redcap UE can serve as supplement in this case.

Thirdly, the link performance seen between handheld UE and Redcap UE may be different, especially for 1Rx Redcap and HD-FDD Redcap. Therefore, although NTN network can get MDT report from handheld UE, the MDT report from Redcap UE still can provide more information to network.

Lastly, we understand there is power consumption concern for Redcap UE. However, considering it is up to UE whether to support this capability or not, and UE who support this feature will stop the memory when the cached data over the threshold, and MDT will be started only after receiving the network configuration, we think all these approach can guarantee that the feature will be applied for UE without power consumption issue and only when necessary from network side.

- Proposal 1: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN. The legacy requirement from NTN can be reused.

Regarding the MDT requirement, for both 1Rx and 2Rx Redcap UE, the legacy NTN MDT requirements can be reused, including the measurement requirements and accuracy relative time stamp requirements.

- Proposal 2: Reuse legacy NTN MDT requirements for Redcap UE over NTN, including measurement requirements and relative time stamp requirements.

In last meeting, company suggest that RAN4 to discuss whether to send LS to RAN2 / RAN3 informing them about the RAN4 agreement on specifying MDT requirements in RRC_IDLE and RRC_INACTIVE for (e)RedCap UE supporting MDT. In our understanding, no new signaling and UE capability are needed for (e)RedCap UE supporting MDT, no RAN2/RAN3 work is actually involved. Therefore, no need to send LS to RAN2/RAN3.

- Proposal 3: No need to send LS to RAN2 / RAN3 informing them about the RAN4 agreement on specifying MDT requirements in RRC_IDLE and RRC_INACTIVE for (e)RedCap UE supporting MDT.

  - SMTCs and 'in parallel' related features

Issue 1-2-2: SMTC adjustment in idle/inactive

<Way Forward>:

- Proposal 1: SMTC adjustment in idle/inactive is kept mandatory for (e)RedCap UEs with NTN.

- Proposal 2: RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN.

  - Proposal 2a: Maintain the same principle used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode.

- Proposal 3: For SMTC adjustment in idle/inactive, we recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

Our original view is to maintain the same principle used in NR NTN, the controversial part is whether the SMTC adjustment in IDLE/Inactive is a mandatory or optional feature. However, considering the feature is introduced by RAN2, and whether it is mandatory or optional have no impact on RAN4 requirement, we agree with proposal 2, that for the SMTC adjustment in IDLE/Inactive capability, (e)RedCap UEs with NTN maintain the same principle used in NR NTN.

- Proposal 4: For the SMTC adjustment in IDLE/Inactive capability, (e)RedCap UEs with NTN maintain the same principle used in NR NTN.

- The impact of HD-FDD

Issue 1-3-5: The impact on NR Handover due to HD-FDD

<Way Forward>:

- Proposal 1: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted.

  - Proposal 1a: Regarding Issue 3-2-1 (The impact on NR Handover due to HD-FDD), adopt Option 1 with 'availability' defined as 'no collision with UL transmission at the UE'.

  - Proposal 2: Adopt option 2 (no spec impact) for HO, CHO and satellite switch for HD-FDD UE.

Agreement from RAN4#114bis:

- Option 1: agree on the principle below and further discuss the wording in the CR:

    - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU

    - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- One SSB is available during T∆

- One SSB is available during TIU

- Option 2: No spec impact

According to RAN1’s agreement, that for the Case 5 ‘Configured SSB collides with dynamically scheduled or configured UL transmission’, the existing priority rules can be reused for HD-FDD (e)RedCap UE in NTN, which means the SSB always prioritized over UL transmission. With this, Option 1 and Option 2 are actually leading to same requirement, due to the prerequisite of Option 1 can always be met. We are both ok with Option 1 (keep consistency with legacy) and Option 2 (no misleading about collision rule) .

- Proposal 5: Both ok with Option 1 (keep consistency with legacy) and Option 2 (no misleading about collision rule)

- UE capability

Issue 1-4-1: General principle for designing the UE capabilities for (e)RedCap UE with NTN

<Way Forward>:

- Option 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

  - FFS: The Rel-19 requirement can be release independent from Rel-17.

- Option 2: RAN4 to introduce new capability to declare support over RedCap over NTN.

- Option 3: to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN).

For UE capability, RAN1 hasn’t achieve any working assumption so far, hence, we think it is RAN4’s task to achieve the related agreement.

For UE capability declaring, both Option 1 and Option 2 are workable. Considering the principle of no need to introduce new capability if existing capability can be used, we slightly prefer the implicit way to save the workload of RAN2.

Proposal 6: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Conclusion

Based on the discussion above, the following proposals are concluded.

- Proposal 1: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN. The legacy requirement from NTN can be reused.

- Proposal 2: Reuse legacy NTN MDT requirements for Redcap UE over NTN, including measurement requirements and relative time stamp requirements.

- Proposal 3: No need to send LS to RAN2 / RAN3 informing them about the RAN4 agreement on specifying MDT requirements in RRC_IDLE and RRC_INACTIVE for (e)RedCap UE supporting MDT.

- Proposal 4: For the SMTC adjustment in IDLE/Inactive capability, (e)RedCap UEs with NTN maintain the same principle used in NR NTN.

Proposal 5: Both ok with Option 1 (keep consistency with legacy) and Option 2 (no misleading about collision rule)

Proposal 6: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Reference

- R4-2508287, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)