# 3GPP TSG-RAN WG4 Meeting #116	R4-2510085

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.21.4.1

Source:	China Telecom

Title:	Discussion on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation

Document for:	Discussion

- Introduction

In RAN4 #115 meeting, RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation were discussed and the WF was approved [1].

In this paper, we present our views on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation captured in the WF [1] and summary [2].

- Discussion

L1-CLI-RSSI

Status in the summary [2]:

- Issue 1-3-3: Measurement restriction - Background: WF from RAN4#114-bis - Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements - For Method #1& Method 3: - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR2, FFS for FR1.. - Proposals for collision between L1-CLI-RSSI measurement and L1/L3 RRM measurements in FR1 - Proposal 1a (CATT, ZTE) - Measurement requirements for L1-CLI-RSSI apply as default. - Proposal 1b (Samsung) - Measurement requirements for L1-CLI-RSSI apply, when SCS of L1-CLI-RSSI measurement and L1/L3 RRM measurements are same. - Proposal 1c (E////) - Measurement requirements for L1-CLI-RSSI apply, except when L1-CLI-RSSI measurement resources overlap MG - Proposal 1d (HW) - Measurement requirements for L1-CLI-RSSI apply, except when - L1-CLI-RSSI measurement resources overlap MG - L1-CLI-RSSI measurement resources are configured with a different SCS from the SSB used for L1/L3 measurement - Proposal 2 (Nokia) - Measurement requirements for L1-CLI-RSSI do not apply. - Recommended WF for collision between L1 CLI measurements and L1/L3 RRM measurements in FR1 - Discuss the following 2 options on high level: - Option 1: Measurement requirements for L1-CLI-RSSI apply as default, and define exception cases where requirements do not apply. - Option 2: Measurement requirements for L1-CLI-RSSI do not apply.

For collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements in FR1, in our understanding, UE is able to measure L1-CLI-RSSI and L1/L3 signals at the same time in FR1, which is different from the case in FR2. Therefore, measurement requirements for L1-CLI-RSSI apply, and some exception cases can be defined where requirements do not apply.

Proposal 1: Measurement requirements for L1-CLI-RSSI apply as default, and define exception cases where requirements do not apply.

- Conclusion

The contribution presents our views on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation, with the following proposals:

Proposal 1: Measurement requirements for L1-CLI-RSSI apply as default, and define exception cases where requirements do not apply.

- References

- R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #115, 19 – 23 May, 2025.

- R4-2505568, Topic summary for [115][219] NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #115, 19 – 23 May, 2025.