# 3GPP TSG-RAN WG4 Meeting #116	R4-2510089

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.23.3.1

Source:	China Telecom

Title:	Discussion on On-demand SSB SCell operation for enhancements of NES for NR

Document for:	Discussion

- Introduction

In RAN4 #115 meeting, On-demand SSB SCell operation for enhancements of network energy savings for NR were discussed and the WF was approved [1].

In this paper, we present our views on On-demand SSB SCell operation for enhancements of network energy savings for NR captured in the WF [1] and summary [2].

- Discussion

OD-SSB based deactivated SCell L3 measurement (Case 1)

Status in the WF [1]:

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW Sub-issue 2: OD-SSB in FMW collided with MG - Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion. - Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

In current specification, if SMTC is partially overlapping with MG occasion, SSB measurement is skipped. In our views, the same principle can be followed, i.e., UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion.

Proposal 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion.

OD-SSB based deactivated SCell L3 measurement (Case 2)

Status in the WF [1]:

Issue 1-2-2: Requirement for RAN1 Alt Time-C2 Agreement - Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity. - Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if - The periodicity of AO-SSB and OD-SSB are same - It doesn’t impact the WI’s completion.

The use case of Alt Time-C2 may be not very clear, however, If UE support the capability of RAN1 Alt Time-C2, the requirement can be based on OD-SSB periodicity to simplify the definition of measurement requirement for Alt Time-C2.

Proposal 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity for RAN1 Alt Time-C2.

Status in the WF [1]:

Issue 1-2-3: Requirement for scenario 3 (OD-SSB and AO-SSB are within different frequencies) Sub issue 1: RAN2 LS for scenario 3 Agreement: - RAN4 to confirm RAN2’s WS to introduce a new servingCellMO. Sub issue 2: Requirement for scenario 3 Agreement: RAN4 will further discuss whether and how to define the requirements for scenario 3, and meanwhile the discussion will not impact the WI completion.

There are many controversial issues for requirements definitions on scenario 3, and considering the limited time for the core part discussion, we prefer no requirement is defined for scenario 3 in Rel-19.

Proposal 3: No requirement is defined for scenario 3 in Rel-19.

OD-SSB general issues

Status in the WF [1]:

Issue 1-4-4: Additional Processing time for OD-SSB parameter update Agreement - Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates. - The additional processing time is 5ms or 2ms based on UE capability reporting. - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Option 2: When NW updates the OD-SSB parameters for an active Scell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE. - The additional processing time is 5ms - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Note 2: No capability is needed

For additional processing time for OD-SSB parameter update, same timeline can be applied with MAC CE based OD-SSB activation.

Proposal 4: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

Status in the WF [1]:

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline Agreement - RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS. - Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation. - Option 1: No UE additional processing time for OD-SSB deactivation - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

For OD-SSB deactivation timeline, no UE additional processing time is needed for OD-SSB deactivation, because UE is not required to process signals when receiving OD-SSB deactivation command.

Proposal 5: No UE additional processing time is needed for OD-SSB deactivation.

- Conclusion

The contribution presents our views on On-demand SSB SCell operation for enhancements of network energy savings for NR, with the following proposals:

Proposal 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion.

Proposal 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity for RAN1 Alt Time-C2.

Proposal 3: No requirement is defined for scenario 3 in Rel-19.

Proposal 4: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

Proposal 5: No UE additional processing time is needed for OD-SSB deactivation.

- References

- R4-2508277, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #115, 19 – 23 May, 2025.

- R4-2505570, Topic summary for [115][221] Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #115, 19 – 23 May, 2025.