# 3GPP TSG-RAN WG4 Meeting #116                             R4-2510163

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.17.2

Source: 	CMCC

Title: 			 Discussion on general aspects for AI/ML for NR air interface

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on general aspects.

- Discussion

LCM requirements Agreements: Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism. - For semi-persistent CSI reporting - Activation delay starts at the reception of the MAC-CE/DCI - For aperiodic CSI reporting - Activation delay starts at the reception of the DCI Switching delay and Fallback delay: - Deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement, including both AI/nonAI, discussion. Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed. CSI prediction: - CSI prediction, the details are subject to RAN1 decision - RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring Way forward (not agreements) Deactivation delay for CSI prediction: - The following questions were noted for further discussion - Can NW differentiate between the predicted CSI reporting and the measured CSI reporting? - In case of switch between AI, is there existing reporting to differentiate the reports before and after the deactivation

It was agreed that activation delay requirements are defined for periodic, semi-persistent and aperiodic CSI reporting for beam prediction. For periodic, especially option A , since companies have different views on the related machanism, LS was sent to RAN2 for clarification in last meeting. RAN4 can further discuss the activation delay for periodic CSI reporting after receiving RAN2 reply LS.

For semi-persistent CSI reporting, activation delay starts at the reception of the MAC-CE/DCI. And the starting point of activation delay for apriodic CSI reporting is the reception of the DCI. In order to define the activation delay, both starting point and ending point are necessary. According to RAN1 design, NW can differentiate between the predicted beam reporting and the measured beam reporting. Based on this, for both semi-persistent CSI reporting and aperiodic CSI reporting, the ending point of activation delay could be sending the report of inference results.

Proposal 1: for beam prediciton, for both semi-persistent CSI reporting and aperiodic CSI reporting, it is proposed that the ending point of activation delay is sending the inference report.

We will further discuss the components of funtionality activation delay. For the case that the reporting is triggered by MAC CE, funtionality activation delay comprise HARQ feedback, measurement and inference. Normally, HARQ related delay is THARQ + 3ms, where THARQ is the timing between activation command and acknowledgement.For measurement and inference, the main contributor is the time used to measure Set B, which can be considered as measurement period for inference. In last meeting, for measurement period for inference, it was agreed that the starting point is to use M, N, P for case 1, which means the legacy L1-RSRP measurement delaycan be used as baseline. In summary, for the case that the reporting is triggered by MAC CE, the funtionality activation delay is THARQ + 3ms +Tmeasurement period for inference. Similarly, for the case that the reporting is triggered by DCI, the funtionality activation delay is Tmeasurement period for inference.

Proposal 2: for beam prediction, for the case that the reporting is triggered by MAC CE, the funtionality activation delay is THARQ + 3ms +Tmeasurement period for inference.

Proposal 3: for beam prediction, for the case that the reporting is triggered by DCI, the funtionality activation delay is Tmeasurement period for inference.

Agreement Issue 2-2: Measurement period for inference Case 1: The starting point is to use M, N, P (reference the values from an existing clause in the same version of the spec where the measurement period is defined) Case 2: postpone discussion until RAN1 concludes

Deactivation delay for CSI prediction is FFS. According to the discussion in last meeting, one issue for further discussion is whether network can differentiate between the predicted CSI reporting and the measured CSI reporting. According to RAN1 agreements, the answer is yes. RAN1 agreed to introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction. Since network can differentiate between the predicted CSI reporting and the measured CSI reporting, it is feasible to define the deactivation delay for CSI prediction.

RAN1 agreements in RAN1#120bis Agreement For CSI prediction using UE-side model, at least for inference, introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction. - Note: terminology of “AI-CSI prediction” and “non-AI CSI prediction” is separate discussion - Detailed parameter name is upto RAN2

Observation 1: RAN1 agreed to introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction.

Following is the RAN1 agreements on CSI prediction, as highlighted in blue, the supported inference report type includes semi-persistent report and aperiodic report. Both are activated by DCI. Similar as beam prediction, if deactivation delay is defined, the starting point is the reception of the DCI.

RAN1 agreements in RAN1#121 Agreement For CSI prediction using UE-side model, for UE assisted performance monitoring, - Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - For monitoring report type, semi-persistent and aperiodic are supported - The following combination for inference report type and monitoring report type are supported Monitoring report type Inference report type; SP report; AP report. AP report; Not support; Support. SP report; Support; Support. - For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported.

Proposal 4: for CSI prediction, it is proposed to define deactivation delay, considering that RAN1 agreed to introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction.

Agreements in RAN4#112bis Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

For the target for AI/ML based beam management, our view is that AI/ML based performance requirement should not be worse than the legacy. According to the discussion in last meeting, some companies mentioned that the AI/ML based performance will be worse than the legacy, since the gain come from overhead reduction. From operator point of view, we do not think that this is a good way to go. If the predicted results are worse than the legacy measured results, it will have impact on the network deployment, which is not preferred.

Proposal 5: for AI/ML based beam management, AI/ML based performance requirement should not be worse than the legacy measurement requirements.

- Conclusion

This contribution provides discussion on general aspects. The proposals are:

Proposal 1: for beam prediciton, for both semi-persistent CSI reporting and aperiodic CSI reporting, it is proposed that the ending point of activation delay is sending the inference report.

Proposal 2: for beam prediction, for the case that the reporting is triggered by MAC CE, the funtionality activation delay is THARQ + 3ms +Tmeasurement period for inference.

Proposal 3: for beam prediction, for the case that the reporting is triggered by DCI, the funtionality activation delay is Tmeasurement period for inference.

Observation 1: RAN1 agreed to introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction.

Proposal 4: for CSI prediction, it is proposed to define deactivation delay, considering that RAN1 agreed to introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction.

Proposal 5: for AI/ML based beam management, AI/ML based performance requirement should not be worse than the legacy measurement requirements.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420336, WF on AI/ML air interface part 2, Ericsson

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R5-247647, Discussion on handling of AI/ML in RAN5, China Mobile, AT&T, CAICT, Ericsson, Huawei, MediaTek, Nokia, ZTE

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface