# 3GPP TSG-RAN WG4 Meeting #116                              R4-2510165

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.18.2

Source: 	CMCC

Title: 			 Discussion on general aspects for study on AI/ML for NR air interface Phase 2

Document for:	Discussion

- Introduction

In RAN #107 meeting, the revised SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are duplicated as following.

- Testability and interoperability [RAN4]: - Further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [2]. This contribution provides discussion on general aspects.

- Discussion

Generalization Agreements: The high level and general guidelines for the generalization test are provided as follow. The exact decisions are to be made case by case. - Study and , if feasible, define requirements for each AI/ML functionality - Define one test per UE capability as a minimum - FFS >1 test per UE capability - Define a minimum set of test configurations (including NW sided conditions if any) as mandatory for testing of AI/ML-enabled Feature Potential areas to consider for generalization testing: - gNB array parameters - port layouts, array size, antenna virtualization - propagation conditions - Deployment scenarios - Carrier frequencies - Speeds - Indoor/outdoor - Bandwidths - SNR

In last meeting, it was agreed to define one test per UE capability as a minimum, FFS >1 test per UE capability. We will further discuss this issue case by case based on RAN1 lastest agreed feature list [3].

For beam management, accroding to current RAN1 feature list [3], the feature group is categorized as Increased number of reported RSs for beam management, UE-side beam prediction for BM Case1 [for inference], UE-side beam prediction for BM Case1 with predicted RSRP [for inference], UE-side beam prediction for BM Case2 [for inference], UE-side beam prediction for BM-Case2 with predicted RSRP [for inference], Data collection for UE-side beam prediction [for BM case 1]. For each feature group, there are multiple components. Taking feature group of UE-side beam prediction for BM Case1 [for inference] as an example, there are as much as 12 components, such as maximum number of inference report(s) configured for BM-Case1 per BWP, supported combinations of the number of resources for Set B  and the number of resources for Set A, etc.

Observation 1: According to lastest RAN1 feature list, there are many feature group, and for each feature group, there are multiple components.

For our previous agreements that define one test per UE capability as a minimum, it is necessary to align the granularity of UE capability. The UE capability refer to feature group or each component of each feature group in RAN1 feature list. Taking beam prediction as an example, the issue is how RAN4 to define the test. Option 1 is that RAN4 respectively define one test for each feature group, the feature group includes UE-side beam prediction for BM Case1 [for inference], UE-side beam prediction for BM Case1 with predicted RSRP [for inference], UE-side beam prediction for BM Case2 [for inference], UE-side beam prediction for BM-Case2 with predicted RSRP [for inference]. Option 2 is to define mutiple tests for each feature group, and each test corresponding to one component in one feature group.

Proposal 1: for RAN4 previous agreements “define one test per UE capability as a minimum”, it is necessary to align the granularity of UE capability. Whether it refer to feature group or refer to each component of each feature group in RAN1 feature list.

If the UE capability refer to feature group in RAN1 feature list, only one test per UE capability is not enough. Even if the UE capability refer to each component of each feature group in RAN1 feature list, there is a risk that the performance may not be guaranted with only one test per UE capability. The reason is that the granularity of UE capability in RAN1 feature list is very rough. For example, RAN1 feature list may consider combinations of the number of resources for Set B and the number of resources for Set A, however, the AI/ML performance is also impacted by the channel condition, gNB array parameter, etc. Even for a certein combination of the number of resources for Set B and the number of resources for Set A, the AI/ML performance may be different for different channel condition, gNB array parameter, etc. From this point of view, it is proposed to consider >1 test per UE capability.

Proposal 2: it is proposed to consider >1 test per UE capability if the granularity of UE capability is rough.

- Conclusion

This contribution provides discussion on general aspects for study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2. The proposals are:

Observation 1: According to lastest RAN1 feature list, there are many feature group, and for each feature group, there are multiple components.

Proposal 1: for RAN4 previous agreements “define one test per UE capability as a minimum”, it is necessary to align the granularity of UE capability. Whether it refer to feature group or refer to each component of each feature group in RAN1 feature list.

Proposal 2: it is proposed to consider >1 test per UE capability if the granularity of UE capability is rough.

- Reference

- RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

- R4-2508051, WF on AI/ML part 2, Ericsson

- R1-2504673, Updated RAN1 UE features list for Rel-19 NR after RAN1 #121

- R4-2505107, WF on AI/ML air interface study, Ericsson

- R4-2503953, WF on AI/ML air interface study, Ericsson

- R4-2420336, WF on AI/ML air interface part 2, Ericsson

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R5-247647, Discussion on handling of AI/ML in RAN5, China Mobile, AT&T, CAICT, Ericsson, Huawei, MediaTek, Nokia, ZTE

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface