3GPP TSG-RAN WG4 Meeting #116	R4-2510225

# Bengaluru, India, August 25th – 29th, 2025

Title:	On conclusion of FR1 dynamic MIMO OTA

Source:	Samsung

Agenda item:	7.10.3.3

Document for:	Discussion

1.	Introduction

This is the final meeting for core part of this work item. For the dynamic MIMO OTA objectives, the only open issue is about the performance metric. The controversial point is how to handle the three percentiles (10%/50%/90%).

The WF of RAN4#114 is as following:

Issue 1-2: Dynamic MIMO OTA test and UE performance metric Agreement: - All measured throughput values for different device orientations and rotations shall be combined to derive a single CDF curve for the dynamic MIMO OTA test. FFS whether the performance metric should be based on three (10%/50%/90%) percentiles at the CDF. - Use the following as a starting point: - include both UMa and UMi channels via post measurement data processing. FFS how to do the post data processing.

In RAN4#115 meeting, proposals were collected as below [2, R4-2507869] but there was no progress.

Issue 3-2-3: considerations on percentile of CDF for UE dynamic MIMO OTA - Proposals - Option 1: use single percentile value, take 50%-ile CDF. (Samsung) - Option 2: Continue to consider the three different CDF %-iles, CTMT10%, CTMT50%, and CTMT90%. (Keysight, CAICT) Issue 3-2-4: Performance metric for UE dynamic MIMO OTA - Proposals - Proposal 1: down select one metric from the following options (Huawei) - Option 1: use a resolution of [0.2] dB to create CDF curves for the combined data pool from UMa and UMi channels and use [50%-ile] for performance threshold. - Option 2: calculate metrics for UMa and UMi separately for a given percentile on throughput CDF, then combine the two metrics. - Option 3: put measured throughput against reported 15 CQI values, with pass and fail criterion as [3] allowed failures to meet expected throughputs and non-reported CQI excluded from the 15 CQI values. - Proposal 2: it is proposed for RAN4 to discuss the dynamic MIMO OTA performance metrics between the two options: (Samsung) - Option 2: take a weighted average among 10%-tile, 50%-tile, 90%-tile CDF of CTMT, and consider the weighting factor ratio among 10%-tile, 50%-tile, 90%-tile as 9:5:1. - Proposal 3: Consider defining the UE performance metric for dynamic MIMO OTA as 114×[CDF10%TMTUMi+5×CDF50%TMTUMi+CDF90%TMTUMi+CDF10%TMTUMa+5×CDF50%TMTUMa+CDF90%(TMTUMa)]. (CAICT) - Proposal 4: Combine the UMi&UMa results by concatenating the respective TMTs, i.e., TMTUMi&UMa=TMTUMi, TMTUMa into a single data pool and determine the three different CTMTs (10%, 50%, 90%) from that single TMT. To determine a single metric from the three different CTMTs (10%, 50%, 90%), use a weighted averaging approach to give CTMTs with low TP values an almost equal weight as CTMTs with the highest TP value. (Keysight)

In this contribution, we share our views on how to conclude the issue in this meeting.

2. 	Discussion

The controversial point is how to handle the three percentiles (10%/50%/90%) at CDF. One simple approach is to only take 50%-tile, the other one is to take a weighted combination of the three percentiles.

For convenience of discussion, the CTMT is expressed as below

CTMT= w10CTMT10%+w50CTMT50%+w90CTMT90%

- For simple approach, w10=w90=0, and w50=1;

- For weighting approach, w10>w50>w90 and w10+w50+w90 =1.

As discussed in our previous contribution [3, R4-2506973], if weighting approach is adopted, the throughput result at 10%-tile should be increasingly weighted and the throughput result at 90%-tile should be decreasingly weighted, i.e. w10>w50>w90. And based on a theoretical analysis, a weighting as w10:w50:w90=9:5:1 was proposed.

Observation 1:	if weighting approach is adopted, the weight factors should in such a relationship as w10>w50>w90

Many a company deemed that the weighting factors should be derived based on measurement campaign, however, large quantity of measurement requires very long time, and in addition the measurement for UMi is not available yet.

Observation 2:	an accurate weighting factor derivation based on measurement requires large number of devices testing, moreover UMi measurement is not available yet.

To complete the work item on time, the weighting factors derived from theoretical analysis may be the only choice. In case there is obvious misalignment with future measurement, it is still possible to revisit with CR.

Proposal 1:	To complete the work item on time, RAN4 to take the weighting factors based on theoretical analysis, i.e. w10:w50:w90=9:5:1, with the understanding that future revisit based on measurement results are not precluded.

If no consensus could be achieved on how to handle the weighting factors for the three percentiles, we may need to fall back to the legacy simple averaged approach, i.e. only consider 50%-tile of CDF as final CTMT.

Proposal 2:	If no consensus could be achieved on the weighting factors, fall back to the legacy simple averaged approach, i.e. only consider 50%-tile of CDF as final CTMT

3. 	Conclusion

Observation 1:	if weighting approach is adopted, the weight factors should in such a relationship as w10>w50>w90

Observation 2:	an accurate weighting factor derivation based on measurement requires large number of devices testing, moreover UMi measurement is not available yet.

Proposal 1:	To complete the work item on time, RAN4 to take the weighting factors based on theoretical analysis, i.e. w10:w50:w90=9:5:1, with the understanding that future revisit based on measurement results are not precluded.

Proposal 2:	If no consensus could be achieved on the weighting factors, fall back to the legacy simple averaged approach, i.e. only consider 50%-tile of CDF as final CTMT

4. 	References

- R4-2502296    “WF for [114][328] MIMO_OTA_Ph3”, CAICT

- R4-2507869    Topic summary for [115][337] TRP_TRS_MIMO_OTA_Ph3, Moderator (vivo)

- R4-2506973    “On performance metric for FR1 dynamic MIMO”, Samsung