3GPP TSG-RAN WG4 Meeting # 116 	R4-2510247

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	Discussion on RF requirement for AIoT BS

Agenda Item:	7.26.3.1

Document for:	Approval

- Introduction

In this contribution, we share our analysis and views on the pending requirements for AIoT BS.

- Discussion

- Modulation quality

Agreement:

- Consider following options

  - Option 1: Take the table below as a starting point.

| Tari | Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Tari= 103M*15 (us) M = 2, 6, 12 and 24 | Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
|  | RF Envelope Ripple | Mh = Ml | 0 |  | [0.5~0.15] (A–B) | V/m orA/m |
|  | RF Envelop Rise Time | Tr,10-90 |  |  | 0.33Tari | µs |
|  | RF Envelop Fall Time | Tf,10-90 |  |  | 0.33Tari | µs |
|  | RF Pulsewidth | PW |  | 0.5Tari | [0.65/0.525] Tari | µs |

      - Note 1: The ripple is calculated based on per-chip, comparing peak value of each chip to its own average.

      - Note 2: SIP is excluded for ripple calculation.

    - Option 2: No need to define RF envelop parameters.

In the last meeting, the we made the following agreements on modulation quality:

It is noted that the SIP is excluded as described in Note 2. The whole modulation quality refers to RFID spec, and in RFID spec, the SIP defined in 3GPP is called delimiter and also need to comply with the RF envelope requirements:

We think it is better to align with the RFID spec.

Observation 1: The SIP (delimiter in RFID) is also complied with the RF envelope requirement in RFID spec.

Proposal 1: The SIP should not be excluded from the ripple requirement verification.

- REFSENS

Agreement:

- Define REFSENS for both OOK and BPSK. Depending on the simulations, they may or may not be the same.

- Use [X dB] as the desens target, with CW single tone input through a signal generator, CW input level to BS Ant is assumed as [TBD] dBm;

For REFSENS, we agreed that:

- The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself.

In the discussion of device, we already agreed the D2R power is the power of 1st sideband and excludes the CW:

From sensitivity perspective, the explanation above will impact the SNR and we prefer align such understanding in both device Tx and reader Rx.

Proposal 2: Align the D2R power definition between requirement of device backscatter and reader sensitivity SNR, which is the power of 1st sideband and exclude the CW.

There are 6 dB difference between OOK and BPSK is used in D2R link budget, and the rationale behind is that 3 dB difference due the residual CW power and 3 dB due to amplitude difference under same CW power. However, if the CW is removed in D2R power definition, the former 3dB does not need to be considered. The D2R power in sensitivity test normally generated by signal generator rather than backscattered by a real CW source, so the 3 dB due to the amplitude difference is also not valid.

Observation 2: Under same D2R power, the SNR of sensitivity should be similar.

Proposal 3: Same REFSENS is applied to both OOK and BPSK.

- Conclusion

In this contribution, we provide our views on the AIoT BS RF requirements.

Observation 1: The SIP (delimiter in RFID) is also complied with the RF envelope requirement in RFID spec.

Proposal 1: The SIP should not be excluded from the ripple requirement verification.

Proposal 2: Align the D2R power definition between requirement of device backscatter and reader sensitivity SNR, which is the power of 1st sideband and exclude the CW.

Observation 2: Under same D2R power, the SNR of sensitivity should be similar.

Proposal 3: Same REFSENS is applied to both OOK and BPSK.

- References

- R4-2508101, WF on requirements for A-IoT BS and CW,	Huawei