3GPP TSG-RAN WG4 Meeting # 116 	R4-2510255

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	Discussion on FR2 BC in initial access

Agenda Item:	9.3

Document for:	Approval

- Introduction

In [1], LS from RAN5 ask RAN4 to further clarify the baseline assumption in RAN4 when define the FR2 BC requirements, we provide our views on corresponding questions.

- Discussion

RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

In [1], there are 3 questions need to be answered and we can discuss them one by one.

Before RAN4 define the requirement for BC in initial access, a LS sent by RAN4 to check the feasibility of Beamlock Function (BLF). The rationale behind is that RAN4 want to define the requirement based a stable UE behavior and without Beamlock, UE may change its beam during the test and we will lose the reference to define the requirement. Whether the test procedure with or without Beamlock can be decided by RAN5 but the baseline assumption in RAN4 should be clearly delivered to RAN5.

Proposal 1: Confirm that RAN4 define the requirement of BC in initial access based on assuming the beam will not change after autonomously beam selection in IDLE mode during the test.

Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

- Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

For this question, the BLF is performed after beam selection is finished, so this procedure does not prevent UE to autonomously choose Uplink beams. RAN4 requirement is applied to the first locked corresponding beam, and the beam selection after beamlock doesn’t need to be considered.

Proposal 2: Confirm that Beamlock should be performed after beam selection is finished and does not prevent UE to autonomously choose uplink beam. RAN4 requirement is only applied to the first selected corresponding uplink beam after beam selection.

Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

As we discussed in question 1, RAN4 sent LS to RAN5 check the feasibility of BLF and define the requirement on BLF can work.

Proposal 3: Confirm that RAN4 define the requirement for BC in initial access in R18 based on the assumption that beam lock function is feasible and can be used.

- Conclusion

In this contribution, we provide our views on RAN5’s questions.

Proposal 1: Confirm that RAN4 define the requirement of BC in initial access based on assuming the beam will not change after autonomously beam selection in IDLE mode during the test.

Proposal 2: Confirm that Beamlock should be performed after beam selection is finished and does not prevent UE to autonomously choose uplink beam. RAN4 requirement is only applied to the first selected corresponding uplink beam after beam selection.

Proposal 3: Confirm that RAN4 define the requirement for BC in initial access in R18 based on the assumption that beam lock function is feasible and can be used.

- References

- R4-2509022	LS on Beam Correspondence Initial Access, Apple