3GPP TSG-RAN WG4 Meeting #116	R4-2510377

Bengaluru, India, 25th – 29th August 2025

Source: 	Ericsson

Title:  	Discussion on the impact of NTN testing for NGSO to RRM performance

Agenda Item:	7.6.5.2

Document for:	Discussion

# Introduction

In the WI NR Enhanced requirements and test methodology for NR and IoT NTN, one of the objectives: TE emulated channel model for NGSO satellite mobility has been discussed and almost completed in demod session, the detailed timing varying Doppler shift and propagation delay model was documented in WF [1]. As the WF captured, the time-varying Doppler shift and propagation delay model supports both LEO-600 and LEO-1200 scenarios.

This contribution outlines our point of view on the impact of NTN testing for NGSO to RRM performance.

# Discussion

Issue: NTN testing for NGSO to RRM performance

In TS38.133 V19.0.0, the high level test procedure for SAN RRM tests outlines the configuration of specific parameters used in NTN testing.

# B.5 High level test procedure for SAN RRM tests The following high level steps are conducted for test cases for SAN defined in clause A.14. - A set of ephemeris information are pre-defined for each satellite corresponding to respective epoch times in TS 38.508-1 [38]. - The same ephemeris information will be maintained during the test (constant ephemerisInfo in all SIB19 updates), i.e. SAN RRM test cases are defined with fixed constant Delay and Doppler shift from Satellite access node to UE unless otherwise stated. The range from which the constant Delay is selected is as follows: - For GSO an altitude of 35,786km is considered. The range of the one-way delay between UE and satellite is from 119.375 ms to 128.79 ms. - For NGSO an altitude of 600km and 1200km on a circular orbit are considered. The range of the one-way delay between UE and satellite is from 2 ms (lowest value for LEO orbit 600km) to 6.67 ms (highest value for LEO orbit 1200km). - The constant Doppler shift (i.e. frequency offset) is derived from the same ephemeris (i.e. orbit emulation) as the Delay. - UE location is determined for the test. During the test, the test system shall provide the UE location to the DUT using AT commands. - The ephemeris and the UE location should be designed such that elevation angle relative to the UE position shall not be smaller than 30 deg during entire test time. - Test equipment adjusts the time and frequency of transmission according to the pre-defined ephemeris (constant in all SIB19 updates) and UE location.

It appears that the TE emulated time varying Doppler Shift and propagation delay model serves an ‘more practical’ alternative of the existing constant delay and Doppler shift model by addressing its drawbacks related to the oversimplification of the channel characteristics. Figure 1 is an example of the time-varying Doppler shift (left) and propagation delay model with the initial elevation angle 30 degrees. Figure 2 also shows the corresponding elevation angle. It is observed that one LEO-600 satellite serves UE for 250 seconds if the elevation angle is more than 30 degrees. In the case of LEO-1200, the serving time is about 500 seconds.

Figure 1	Doppler shift and propagation delay for LEO-600 maximum positive Doppler scenario with the initial elevation angle 30 degrees.

Figure 2	Elevation angle for LEO-600 maximum positive Doppler scenario with the initial elevation angle 30 degrees.

Regarding how to capture the model, as duplicated in the below table, the BDaT session has agreed to specify the channel model in TS 38.101-5 (NR NTN) and 36.101 (IoT-NTN). At this moment, in RRM specification, we can add the option ‘TE emulated time varying Doppler Shift and propagation delay model’ in Clause B.5 and outline the parameters but the model itself can be referred to UE demodulation requirements specifications TS 38.101-5 for NR NTN and TS 36.102 for IoT-NTN, like TDL fading channel models.

Issue 1-3-1 Specification of channel Model - agreement - Capture the TE-emulated channel model into the normative part of Appendix, details can be discussed in the CR drafting stage

The next consideration is how to apply the model within test cases. For RRM tests, WID [2] mentions to consider at least the uplink timing requirements.

Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement

From the other side, the statements in WID doesn’t preclude other test cases except for uplink timing test cases.

In our view, the TE emulated time varying doppler shift and propagation delay model could be directly applied to NR NTN, NB-IoT NTN and MTC NTN, but may not necessary to replace the existing constant model across all relevant test cases.

Among test cases, the crucial cases shall be those inter-satellite cases, which may need to take more than one satellite movement into account and the doppler shift and propagation delay model may contribute more. So that they may need to be verified by the new model over than others. Additionally, we suggest to apply the new model for inter-frequency cases.

For NR NTN, at least below test cases may apply the new model.

In RRC_IDLE state mobility,

- Cell reselection to FR1 inter-frequency NR case

- Time-based measurement initiation to FR1 inter-frequency cell reselection

Timing requirements

In RRC_CONNECTED state mobility,

- Inter-frequency SAN Handover from FR1 to FR1

- Inter-frequency SAN time-based conditional Handover from FR1 to FR1

- Inter-frequency SAN location-based conditional Handover from FR1 to FR1

In Measurement procedure,

- Inter-frequency Measurements, e.g.,

  - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access

  - Event triggered reporting tests without gap under DRX

For NB-IoT NTN and MTC NTN, at least below test cases may apply the new model.

In RRC_IDLE state for satellite access,

- Cell re-selection for satellite access

Timing requirements

In RRC connection mobility control for satellite access,

- Random Access for Satellite Access

In UE measurement procedures in RRC_CONNECTED state for UE category NB1 for satellite access,

- Inter-frequency neighbour cell measurement

Proposal 1: In RRM specification, For the introduction of the new test model, add the option ‘TE emulated time varying Doppler Shift and propagation delay model’ in Clause B.5 and outline the parameters but the model itself can be referred to the BDaT specification.

Proposal 2: Apply the new test model to LEO-600 and/or LEO-1200. Discuss further the used models, i.e., LEO-600 and/or LEO-1200, considering the satellite serving time.

Proposal 3: Update test cases with the new test model.

- For NR NTN, at least below test cases may apply the new model.

In RRC_IDLE state mobility, - Cell reselection to FR1 inter-frequency NR case - Time-based measurement initiation to FR1 inter-frequency cell reselection Timing requirements In RRC_CONNECTED state mobility, - Inter-frequency SAN Handover from FR1 to FR1 - Inter-frequency SAN time-based conditional Handover from FR1 to FR1 - Inter-frequency SAN location-based conditional Handover from FR1 to FR1 In Measurement procedure, - Inter-frequency Measurements, e.g., - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access - Event triggered reporting tests without gap under DRX

- For NB-IoT NTN and MTC NTN, at least below test cases may apply the new model.

In RRC_IDLE state for satellite access, - Cell re-selection for satellite access Timing requirements In RRC connection mobility control for satellite access, - Random Access for Satellite Access In UE measurement procedures in RRC_CONNECTED state for UE category NB1 for satellite access, - Inter-frequency neighbour cell measurement

# Conclusion

Proposal 1: In RRM specification, For the introduction of the new test model, add the option ‘TE emulated time varying Doppler Shift and propagation delay model’ in Clause B.5 and outline the parameters but the model itself can be referred to the BDaT specification.

Proposal 2: Apply the new test model to LEO-600 and/or LEO-1200. Discuss further the used models, i.e., LEO-600 and/or LEO-1200, considering the satellite serving time.

Proposal 3: Update test cases with the new test model.

- For NR NTN, at least below test cases may apply the new model.

In RRC_IDLE state mobility, - Cell reselection to FR1 inter-frequency NR case - Time-based measurement initiation to FR1 inter-frequency cell reselection Timing requirements In RRC_CONNECTED state mobility, - Inter-frequency SAN Handover from FR1 to FR1 - Inter-frequency SAN time-based conditional Handover from FR1 to FR1 - Inter-frequency SAN location-based conditional Handover from FR1 to FR1 In Measurement procedure, - Inter-frequency Measurements, e.g., - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access - Event triggered reporting tests without gap under DRX

- For NB-IoT NTN and MTC NTN, at least below test cases may apply the new model.

In RRC_IDLE state for satellite access, - Cell re-selection for satellite access Timing requirements In RRC connection mobility control for satellite access, - Random Access for Satellite Access In UE measurement procedures in RRC_CONNECTED state for UE category NB1 for satellite access, - Inter-frequency neighbour cell measurement

# References

- R4-2504700	Way Forward for [114bis][323] NTN_testing_NGSO_channel_model_and_demod, Samsung

- RP-240857		New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN4 chair.