3GPP TSG RAN WG4 Meeting #116	 R4-2510783

Bengaluru, India, 25 Aug. – 29 Aug. 2025

Agenda Item:	7.25.3.2

Source: 	Ericsson

Title:	Discussion on CSI-RS based L1 measurement requirements

Document for:	Discussion

# Introduction

In this contribution we discuss CSI-RS based L1 measurement requirements.

# Discussion

## CSI-RS based L1 measurement requirements

### Measurement procedure

The following is concluded in RAN4#114bis meeting.

- Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting.

- Further discuss whether step 2 can be skipped for serving cell or not.

- Candidate options:

- Option A: RAN4 requirements regarding ‘skipping step 2’ do not apply for the case CSI-RS based L1 measurement for LTM candidate cell is configured in legacy CSI-RS based L1 measurement in serving cell in FR2. (QC, HW)

  - Option A1: Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells. (QC, OPPO)

  - Option A2: The CSI-RS based L1-RSRP measurement requirement is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to 'off', is one of the FR2 serving cells (QC).

- Option B: keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM. (MTK, Apple, E///)

In the last meeting, RX beam sweeping based on CSI-RS was agreed upon as an optional UE capability. For the remainder of the discussion, we refer to this feature as csi-rsBasedRxBeamSweeping. One of the open issues is its applicability when the existing serving cell is also configured as LTM candidate cell.

In legacy, for serving cell measurements, the UE would perform SSB-based RX beam sweeping before computing the CSI-RS-based L1-RSRP measurements. However, when the same cell is configured as a candidate cell, the UE is expected to use CSI-RS-based Rx beam sweeping for the candidate cell measurements. This different approach could introduce ambiguity and confusion in handling measurements for SCells. To address this potential issue, we are fine excluding the new capability to candidate cells which are serving cells as well.

- csi-rsBasedRxbeamSweeping is not applied to serving cells.

### Restriction on CSI-RS resource configuration

In the last meeting, restrictions on CSI-RS resource configuration were discussed and following WF was agreed.

- Option 1: Apple, ZTE, Ericsson - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms. - Option 2: vivo - When CSI-RS based L1 measurements cause scheduling restriction, all CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms and the two windows, if configured, are separated by 20ms.

CSI-RS based L3-RSRP uses the CSI-RS type, CSI-RS-Resource-Mobility. The periodicity and offset of CSI-RS-Resource-Mobility are {4ms, 5ms, 10ms, 20ms, 40ms}, whereas for CSI-RS for L1-RSRP, they are {4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640} slots.

The maximum periodicity for L3-RSRP is 40 ms, during which one intra-frequency layer can have two separate 5 ms windows where the CSI-RS is allowed. Given that the CSI-RS for L1-RSRP can have a periodicity of up to 640 slots, we would like to clarify the configuration restriction. Therefore, we propose that all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms.

We do not support Option 2, as it not only limits the configuration to two windows within 640 ms but also imposes additional constraints on their placement. Specifically, if Option 2 is adopted, the network would only be able to configure CSI-RS resources at two fixed locations within a 640 ms period, and these two windows must be separated by 20 ms. This significantly reduces flexibility in CSI-RS configuration.

- In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restrictions:

  - all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms

Sharing factor for resource collision between serving and neighbour cell

The following WF is agreed during the last meeting.

For FR2 CSI-RS L1 measurement requirement, if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain.

- Option 1: sharing factor shall be introduced. (HW, QC, E///)

- Option 2: introduce measurement restriction as legacy. (MTK, vivo)

For SSB based L1-RSRP measurements RAN4 defined PL1_sharing to share the measurement occasions between the candidate cells, and we suggest following the same approach for the CSI-RS based L1-RSRP measurements as well.

The legacy requirement option 2 was referring to is for the serving cell measurements of clause 9.5. We think neighboring cell measurements should be the appropriate assumption to follow here than the serving cell requirements. If we follow option 2 there is a risk that UE may not measure one of the cell and NW do not know if the cell is not detectable or UE applied measurement restriction and dropped the measurements.

- For FR2 CSI-RS L1 measurement requirement, if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain, introduce sharing factor similar to PL1_sharing.

### Early CSI acquisition on a candidate cell

The following options were discussed in the last meeting.

- Option 1: Define timeline for baseline early CSI acquisition. (vivo, Nokia, E///)

- Option 2: Define scheduling/measurement restriction requirements for optional early CSI acquisition. (QC)

- Option 3: not define RAN4 requirements for early CSI acquisition. (MTK, HW, Apple)

For the baseline early CSI acquisition, we believe that the UE should be prepared to send the CSI report within the LTM cell switch delay or on the first occasion of the CSI reporting request. Although the RRM specification typically does not include detailed requirements for CSI, there are certain clauses that specify the timing for UE to send CSI reports. We have referenced on section of TS 38.133 where some aspects of CSI reporting are mentioned.

TFirst_available_CSI is the delay uncertainty (in ms) in acquiring the first available downlink CSI reference resource. - TCSI_processing is the UE processing time for CSI reporting. - TCSI_reporting_after is the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), 3*Ttarget_PL-RS)

For a UE reporting this capability, we expect that UE should be able to compute the CSI report within the cell switch delay completion if there is a CSI-RS resource available and enough processing time for CSI processing is available. If UE cannot complete the CSI report within the cell switch delay, we think there is NO enhancement from the UE side. Hence, we suggest capturing it in the RRM specification stating that the UE should be ready to send a valid CSI report immediately upon the LTM cell switch delay completion.

One example of how we can define the early CSI reporting delay is:

TCSI_reporting_LTM is the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), DLTM).

- For the baseline early CSI acquisition, RAN4 to define early CSI acquisition delay (TCSI_reporting_LTM) as the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), DLTM).

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made the following observations and proposals.

- csi-rsBasedRxbeamSweeping is not applied to serving cells.

- In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction:

  - all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms.

- For FR2 CSI-RS L1 measurement requirement, if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain, introduce sharing factor similar to PL1_sharing.

- For the baseline CSI acquisition, RAN4 to define early CSI acquisition delay (TCSI_reporting_LTM) as the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max (TFirst_available_CSI + TCSI_processing, DLTM).

# References

[1] RP-241515, New WID: NR mobility enhancements Phase 4

[2] R4-2508282, WF on NR mobility enhancements Phase 4