3GPP TSG-RAN WG4 Meeting #116	   R4-2510792

# Bangalore, India, 25th – 29th , August, 2025

Agenda Item:	7.21.4.2

Source: 	Ericsson

Title:	Discussion on RRM impacts for SBFD operations (SBFD)

Document for:	Discussion

# Introduction

The agreements on SBFD operation during RAN4#115 meeting are captured in the approved WF [1].

In this paper we provide our view on the listed open issues on RRM core requirement within related WID [2] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

## 2.1 Other RRM core requirements for SBFD operation

### CSI-RS based L1 measurement

#### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Background: - Different cases for DUD - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). - RAN1 agreements related to CSI processing time Agreement RAN4 115 - Define relaxed accuracy requirements based on 24 RB for Case 3 following RAN4#114-bis agreement, based on the simulation assumption in Annex. - Further discuss the upper bound of the measurement period due to processing time and its applicability.

For the CSI-RS based L1 measurement, due to the puncturing in the frequency domain for the CSI-RS resources and the CSI-RS resource discontinuity in frequency domain, some companies suggest that the CSI-RS measurement delay requirement needs to be updated.

Based on RAN1 agreement, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy, based on TS 38.214 clause 5.4 if we look at the Z2 number of symbols, regardless of the SCS, the time is around 4 ms.

…

When the CSI-RS resource is configured across two DL sub band, the processing time will become 8ms. We think instead of adding another measurement sample which will be bounded by the periodicity of the measurement resource, it is a better solution to add a lower bound of 8ms in the measurement delay requirement.

- Proposal 1: If UE indicates that CSI processing time is scaled by 2, the measurement delay requirement can add a lower bound of 8ms.

#### Issue 2-1-1c: CSI-RS L1 measurement – different symbol types - Agreements - For L1-RSRP and L1-SINR - Measurement period is extended by using R16 NR-U approach. - For RLM/BFD/CBD - Confirm the following for defining the requirements - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type. - Existing measurement period apply.

The legacy NR-U approach for L1-RSRP and L1-SINR is extending the measurement period by counting the unavailable measurement occasions. When the unavailable measurement occasions reach the maximum allowed value, the UE will report RSRP_0 which will indicate to the network that the RSRP value is not valid for this case.

There are two aspects that will impact the CSI-RS based L1-RSRP for SBFD operation.

First, it is due to the network configuration of the valid symbol type, the UE will only perform measurements on the valid symbol. When the CS-RS resource is being configured in the invalid symbol, the measurement occasion will not be available to the UE.

Another aspect is that due to the dynamically scheduled UL transmission that overlapped with CSI-RS resource being configured, in which case the UL transmission will be prioritized.

RAN4 has already agreed to extend the measurement period follow the Rel-16 NR-U approach. And it is already reflected on the endorsed CR. The remaining issue is to define the L1max for CSI-RS based L1 measurement.

Issue 2-1-5 (new): L1max when using NR-U approach for L1 CSI-RS measurement

#### L1max for CSI-RS based L1-RSRP measurement

To evaluate the L1max value for L1-RSRP, there are two aspects that need to be balanced:

- Measurement delay

- Report value validity

If the L1max value is set too large, especially when the DRX cycle is very long, this will result in a very long measurement delay and UE will just report the value that was sampled a long time ago. This will have an impact on how useful the measurement results are. However, if the L1max value is set too low, the UE will report invalid RSRP value more often than necessary.

For DRX > 320ms, since the M value can be 3 or 1 depending on the network configuration, based on the existing agreement, the measurement period will only need to be extended when M=3.

We think the L1max value can be the same as 3 when the DRX cycle is longer than the 320ms. The measurement delay will only be impacted by the DRX cycle length in this case. When the L1max=3 it implies that 50% of the samples within the measurement window (M+L1max) are invalid symbols.

- Proposal 2: For CSI-RS based L1-RSRP, the maximum invalid measurement occasion L1max=3 for TDRX > 320ms

For shorter DRX cycle, it is upon the comparison of the CSI-RS periodicity and the DRX cycle. 40ms<Max(TDRX, TCSI-RS) ≤ 320ms, considering the scaling factor 1.5, we propose L1max=5, which is derived by ceil (L1max for DRX>320ms * 1.5).

- Proposal 3: For CSI-RS based L1-RSRP, the maximum invalid measurement occasion L1max=5 for 40ms<Max(TDRX,TCSI-RS) ≤ 320ms

When no DRX is being configured, the measurement delay will be fully impacted by the CSI-RS periodicity which is shorter than 40ms, we think higher L1max can be allowed to make sure the UE has enough samples while reporting the average value. Considering the CSI-RS periodicity is shorter than 40ms, we propose the L1max value to be 7. This is relevant to the 320ms DRX cycle reference. This implies that more than 70% of the samples within the measurement window M+L1max are invalid symbols.

- Proposal 4: For CSI-RS based L1-RSRP, the maximum evaluation period extension L1max=7 for TCSI-RS ≤ 40ms

For UE behavior, based on existing RAN4 agreement, the legacy RSRP_0 ‘not valid’ can be reused when the L1>L1max.

One issue we would like to raise is the consecutive number of invalid measurement occasions which will have impact on both the measurement delay and the measurement quality. From figure 1 we can see at t4 and t5 UE have valid samples, however at t6, t7and t8, the UE encounter continuous disturbance from measurement. If we only consider the L1max value limitation we can still have the samples at t4,t5 and t9 being averaged and reported to the network. However, these consecutive invalid samples will later continuously impact averaging at t5, t6,t7 t8 etc. until t11, this will be causing the UE report RSRP_0 to the network unnecessarily often. From a network point of view, this is a waste of report resources.

As we explained for longer DRX cycle, the L1max value to be the same as M=3, with considering 50% of invalid symbol within the averaging window to avoid long measurement delay. Considering the consecutive invalid sample will impact the averaging of the sample, also to combine the sample that has been taken too long time ago from illustrations the sample taken at t4 and t5 together current sample t9 it is not the optimal, especially when the DRX cycle is very long. we think limitation of the maximum consecutive invalid measurement occasion shall be also specified.

- Proposal 5: The consecutive invalid measurement occasion X shall be smaller than M for CSI-RS based L1-RSRP measurement.

Figure 1: Example of issue for L1-RSRP measurement when encountering consecutive invalid symbols

Expected UE behaviour can be:

- At t3, UE do not report due to M=3 and L1=1

- At t4, UE report L1-RSRP based on sample from t1,t2,t4 , L1=1

- At t5, UE report L1-RSRP based on sample from t2,t4,t5, L1=1

- At t6, UE report the same L1-RSRP based on sample fromt2,t4,t5, L1=2

- At t7, UE report the same L1-RSRP based on sample from t2,t4,t5, L1=3,X=2

- At t8, UE report RSRP_0, which is not valid RSRP value due toL1=4>L1max and X=3=M

- At t9, UE do not report due to M=3

- At t10, UE do not report due to M=3, L1=1

- At t11, UE do not report due to M=3, L1=1

- At t12, UE report L1-RSRP based on sample from t9,t11,t12, L1=1

- Proposal 6: RSRP_0 ‘not valid’ can be used for report when either L1>L1max or X≥M.

Detail wording can be reflected from our CR R4-2510795.

#### L1max for CSI-RS based L1-SINR measurement

Based on RAN4 existing agreement, the measurement period extension can follow the Rel-16 NR-U approach. However, the L1-SINR and NR-U were being specified during the same Rel-16 .

- Observation 1 : NR-U approach is not available to L1-SINR for measurement period extension.

Currently there are 3 set of requirements will be impacted by the SBFD operation due to invalid symbol type or CSI-RS resources overlapping with UL transmission.

##### L1max for CSI-RS based L1-SINR report with CSI-RS based CMR and no dedicated IMR configured

Base on the TS 38.215 clause 5.1.6 definition of CSI-SINR. Our interpretation of no dedicated IMR configured implies that IMR should be within the same resources elements where the CSI-RS resources being configured as CMR.

CSI signal-to-noise and interference ratio (CSI-SINR), is defined as the linear average over the power contribution (in [W]) of the resource elements carrying CSI reference signals divided by the linear average of the noise and interference power contribution (in [W]). If CSI-SINR is used for L1-SINR reporting with dedicated interference measurement resources, the interference and noise is measured over resource(s) indicated by higher layers as described in TS 38.214 [6]. Otherwise, the interference and noise are measured over the resource elements carrying CSI reference signals within the same frequency bandwidth.

This implies the SBFD impact due to valid/invalid symbol type indication or CSI-RS collision with dynamic UL transmission will be equal chance for both CMR and IMR. Hence the same value of L1max for CSI-RS based L1-RSRP can be reused for L1-SINR with CSI-RS based CMR and no dedicated IMR configured.

- Proposal 6: For L1-SINR report with CSI-RS based CMR and no dedicated IMR configured,  L1max=7 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX, L1max=5 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, L1max=3 for TDRX > 320ms

The purpose to define L1max is to avoid too long measurement and report delay especially when DRX cycle is long. We think the same UE behaviour as CSI-RSRP can be expected when L1>L1max.

One issue to notice is that for CSI-RSRP there is ‘RSRP_0 not valid’ value to be reported for L1-CSI-RSRP when L1>L1max. For CSI-SINR, currently there is no such a SINR value to be reported to indicate the report value invalid. Detail wording can be seen from our CR R4-2510800.

- Proposal 7: UE shall report SINR not valid when L1>L1max or X ≥ M.

- Proposal 8: New SINR report value shall be introduced from RAN4 to allow UE report SINR value not valid due to L1>L1max or X ≥ M.

##### L1-SINR report with SSB based CMR and dedicated IMR configured

Based on existing RAN1 agreement, the SSB will be prioritized over dynamic UL transmission. SSB will not be impacted for the valid symbol type indication. This implies the CMR measurement will not be impacted due to SBFD operation.

For the dedicated IMR configured, the legacy requirement applicable to both NZP-CSI-RS and CSI-IM resources being configured. Based on existing RAN1 and RAN4 agreement, the valid symbol type indication is towards only the NZP-CSI-RS resources. Here implies, the impact due to SBFD operation will be different for NZP-CSI-RS and CSI-IM resources.

RAN1 #121 agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation configured in CSI-ReportConfig applies to both CMRs and IMRs (limited to NZP-CSI-RS) associated with the CSI report.

The L1 value and L1max value needs to be reconsidered because for the CSI-RS based SINR report only IMR measurement will be potentially impacted by SBFD operation. This could results in the SINR value reported to the network is not correctly reflect the interference to the channel.

Another thing to mention is the legacy requirement have restriction on CSI-RS as CMR must be mapping to SSB as IMR.

TS 38.133 clause 9.8.4.2 table 9.8.4.2-1 NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity.

This implies that when the CSI-RS resource overlapping with Dynamic UL transmission, if we extend the CSI-RS as IMR measurement period, the SSB based CMR measurement also needs to be extended. This violates the priority rule agreed from RAN1.

RAN1 #117 agreement: Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

Observation 2: There is different priority rule between SSB and CSI-RS when overlapping with dynamic UL transmission.

This conflict is mainly due to the CMR and IMR are configured as SSB and CSI-RS which follow different priority rules when overlapping with dynamic UL transmission.

One option is that limit L1-SINR reporting with SSB based CMR and dedicated IMR configured to CSI-IM resources only under SBFD operation. Since the RAN1 agreement for valid symbol type only limited to NZP-CSI-RS. Restriction on IMR resource to only CSI-IM will exclude the impact from valid symbol type indication.

Due the requirement only applicable that CSI-IM resource shall have 1-to-1 mapping to the SSB configured as the CMR, when the dynamic UL transmission overlapping with the SSB, the dynamic UL transmission will be cancelled to prioritize the SSB measurement, this indicates the CSI-IM will be automatically prioritized since they follow the same periodicity and have 1-to-1 mapping.

This will have the minimum specification change, and no measurement period extension is needed. Detail wording can be reflected from our CR R4-2510801.

- Proposal 9: For L1-SINR report with SSB based CMR and dedicated IMR configured, restrict the IMR configuration only applicable to CSI-IM resources for SBFD operation. No measurement period needs to be extended.

##### L1-SINR report with CSI-RS based CMR and dedicated IMR configured

For CSI-RS based CMR, the CSI-RS resource can only be NZP-CSI-RS resource, for the dedicated IMR, the CSI-RS resource can be either NZP-CSI-RS or CSI-IM resource.

Based on the latest RAN1 agreement

RAN1 #121 agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation configured in CSI-ReportConfig applies to both CMRs and IMRs (limited to NZP-CSI-RS) associated with the CSI report.

Here implies, the impact due to SBFD operation will be different for NZP-CSI-RS and CSI-IM resources configured as IMR.

The consequence of different impact between CMR and IMR due to SBFD operation can lead to incorrect SINR being reported. To minimize the specification impact, we think for SBFD operation, we can restrict the IMR resources only within NZP-CSI-RS. The L1max value and UE behaviour when L1>L1max or N≥M can follow the same UE behaviour.as the CSI-RS based L1-RSRP. Detail wording can be reflected from our CR R4-2510799.

- Proposal 10: For L1-SINR report with CSI-RS based CMR and dedicated IMR configured, restrict the IMR configuration only applicable to NZP-CSI-RS resources for SBFD operation.

- Proposal 11: For L1-SINR report with CSI-RS based CMR and dedicated IMR configured, L1max=7 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX, L1max=5 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, L1max=3 for TDRX > 320ms.

- Proposal 12: For L1-SINR report with CSI-RS based CMR and dedicated IMR configured, UE shall report SINR not valid when L1>L1max or X ≥ M.

#### L1in,max and L1out,max for CSI-RS based radio link monitoring

For CSI-RS based RLM measurement, RAN4 agreed the requirement only applicable when all measurement occasions are within the same type of symbol. The measurement occasion will only be impacted when the CSI-RS colliding with dynamic UL transmission.

One legacy NR-U behavior to be noticed here is that during the in-sync evaluation procedure, layer 1 of the UE shall not send any in-sync indication for the cell to the higher layers when L1in> L1in, max. We think this can be re-used for CSI-RS based RLM evaluation as this extended measurement period will have a negative impact for the in-sync indication.

- Proposal 13: During In-sync evaluation procedure, the layer 1 or the UE shall not send any in-sync indication for the cell to higher layers when L1in> L1in, max or X ≥ M .

For DRX cycle longer than 320ms, the measurement delay will be dominated by the DRX cycle length, Since the Min=10, we think the L1in,max=5 can be reasonable to account for around 33% unavailable measurement occasion.

For shorter DRX cycle or no DRX cycle, we think we can allow more unavailable measurement occasions.

In the case 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, considering the scaling factor 1.5, we propose L1in, max=8, which is derived by ceil(L1in,max for DRX>320ms * 1.5) to account for around 45% unavailable measurement occasion.

For the case, Max(TDRX, TCSI-RS) ) ≤ 40ms or non-DRX , we propose the L1in.max= 10 which account for 50% unavailable measurement occasions.

- Proposal 14: CSI-RS based RLM in sync evaluation period maximum extension L1in,max=10 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX, L1in,max=8 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, L1in,max=5 for TDRX > 320ms.

For DRX cycle longer than 320ms, the measurement delay will be dominated by the DRX cycle length, Since the Mout=20, we think the L1out,max=10 can be reasonable to account for around 33% unavailable measurement occasion.

For shorter DRX cycle or no DRX cycle, we think we can allow more unavailable measurement occasions.

In the case 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, considering the scaling factor 1.5, we propose L1out, max=15, which is derived by ceil(L1out,max for DRX>320ms * 1.5) to account for around 42% unavailable measurement occasion.

For the case, Max(TDRX, TCSI-RS) ) ≤ 40ms or non-DRX , we propose the L1out.max= 20 which account for 50% unavailable measurement occasions.

- Proposal 15: CSI-RS based RLM evaluation period maximum extension L1out,max=20 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX, L1out,max=15 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, L1out,max=10 for TDRX > 320ms.

Details can be seen from our CR R4-2510797.

#### L1max for CSI-RS based beam failure detection

For CSI-RS based beam failure evaluation, we think the similar logic to calculate L1max. Since the MBFD = 10, the same value for RLM Lin,max can be reused.

The beam failure evaluation is comparing the measured RSRP value to the network configured rsrp-Threshold. When L1> L1max or X ≥ M, we think the UE shall not indicate beam failure due to too many measurement occasion extensions.

- Proposal 16: CSI-RS based BFD evaluation period maximum extension L1,max=10 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX, L1,max=8 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, L1,max=5 for TDRX > 320ms.

Details can be seen from our CR R4-2510798.

#### L1max for CSI-RS based candidate beam detection

For CSI-RS based CBD, the maximum measurement occasion extension can be the same as the CSI-RSRP since the MCBD=3 which is the same as the L1-RSRP.

The legacy NR-U UE behaviour when L1> L1max can also be re-used, the UE shall assume no new candidate beams are found for this evaluation period and indicate to the network accordingly.

- Proposal 17: For CSI-RS based CBD,  L1max=7 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX, L1max=5 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, L1max=3 for TDRX > 320ms.

Details can be seen from our CR R4-2510796

# Summary

The following have been proposed in the current contribution:

	Proposal 1: If UE indicates that CSI processing time is scaled by 2, the measurement delay requirement can add a lower bound of 8ms.

	Proposal 2: For CSI-RS based L1-RSRP, the maximum invalid measurement occasion L1max=3 for TDRX > 320ms

	Proposal 3: For CSI-RS based L1-RSRP, the maximum invalid measurement occasion L1max=5 for 40ms<Max(TDRX,TCSI-RS) ≤ 320ms

	Proposal 4: For CSI-RS based L1-RSRP, the maximum evaluation period extension L1max=7 for TCSI-RS ≤ 40ms

	Proposal 5: The consecutive invalid measurement occasion X shall be smaller than M for CSI-RS based L1-RSRP measurement.

	Proposal 6: RSRP_0 ‘not valid’ can be used for report when either L1>L1max or X≥M.

	Proposal 6: For L1-SINR report with CSI-RS based CMR and no dedicated IMR configured,  L1max=7 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX, L1max=5 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, L1max=3 for TDRX > 320ms

	Proposal 7: UE shall report SINR not valid when L1>L1max or X ≥ M.

	Proposal 8: New SINR report value shall be introduced from RAN4 to allow UE report SINR value not valid due to L1>L1max or X ≥ M.

	Proposal 9: For L1-SINR report with SSB based CMR and dedicated IMR configured, restrict the IMR configuration only applicable to CSI-IM resources for SBFD operation. No measurement period needs to be extended.

	Proposal 10: For L1-SINR report with CSI-RS based CMR and dedicated IMR configured, restrict the IMR configuration only applicable to NZP-CSI-RS resources for SBFD operation.

	Proposal 11: For L1-SINR report with CSI-RS based CMR and dedicated IMR configured, L1max=7 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX, L1max=5 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, L1max=3 for TDRX > 320ms.

	Proposal 12: For L1-SINR report with CSI-RS based CMR and dedicated IMR configured, UE shall report SINR not valid when L1>L1max or X ≥ M.

	Proposal 13: During In-sync evaluation procedure, the layer 1 or the UE shall not send any in-sync indication for the cell to higher layers when L1in> L1in, max or X ≥ M .

	Proposal 14: CSI-RS based RLM in sync evaluation period maximum extension L1in,max=10 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX, L1in,max=8 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, L1in,max=5 for TDRX > 320ms.

	Proposal 15: CSI-RS based RLM evaluation period maximum extension L1out,max=20 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX, L1out,max=15 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, L1out,max=10 for TDRX > 320ms.

	Proposal 16: CSI-RS based BFD evaluation period maximum extension L1,max=10 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX, L1,max=8 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, L1,max=5 for TDRX > 320ms.

	Proposal 17: For CSI-RS based CBD,  L1max=7 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX, L1max=5 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, L1max=3 for TDRX > 320ms.

# References

- R4-2508274 “WF on SBFD RRM”, Huawei, May ,2025

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024

- R4-1914356 L1-RSRP measurements in NR-U, Ericsson, November 2019