3GPP TSG-RAN WG4 Meeting #116																		R4-2510809

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.17.7

Source: 	OPPO

Title: 	Demodulation and CSI reporting requirements for CSI prediction

Document for:	Approval

- Discussion

Regarding the reference model for CSI prediction, the following agreements were reached during the RAN4#114 meeting:

Reference model for CSI prediction

Agreement:

Assume no reference AI model for CSI prediction as baseline

- If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model.

Besides, the "Way Forward on Updated simulation assumptions for AIML CSI prediction " was agreed in RAN4 #114bis to proceed with CSI evaluation, including the following steps:

Step-1: to check the accuracy of AI/ML output CSI

For step-1, interested companies are encouraged to provide the 2nd round of CSI prediction simulation results in RAN4#115 meeting.

Step-2: to check SGCS after CSI feedback

Simulation assumptions for step-2 are updated in RAN4#114bis meeting.

Interested companies are also encouraged to provide the 1st round of CSI prediction simulation results (SGCS as KPI, SGCS between the eigenvectors of predicted PMI derived from the ideal DL channel estimation and ground-truth eigenvectors), in RAN4#115 meeting.

Step-3: to check the TPs results, practical DL channel estimation

CSI prediction simulation (TPs as KPI, with practical DL channel estimation) is postponed to RAN4#116 meeting.

According to our understanding, RAN4 needs to first assess the CSI prediction evaluation results across different companies before determining whether a reference model is necessary.

    - If RAN4 achieves a relatively aligned CSI prediction evaluation results in step-2, further evaluation (step-3 comparisons) can proceed (during/after RAN4#116).

    - If alignment in step-2 proves difficult, RAN4 should identify the key factors causing discrepancies and may need to initiate following evaluations to reassess step-2 alignment (during/after RAN4#116).

    - If alignment remains unachievable after step-3, RAN4 may then trigger discussions and evaluations on a reference model for CSI prediction (e.g., during/after RAN4#116bis).

    - If alignment achieves after step-3, RAN4 will trigger evaluations on the CSI reporting requirements for CSI prediction(e.g., during/after RAN4#116bis).

Proposal 1: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction, RAN4 should first assess the CSI prediction evaluation results across different companies before determining whether a reference model is necessary.

Proposal 2: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction,

    - If RAN4 achieves a relatively aligned CSI prediction evaluation results in step-2, further evaluation (step-3 comparisons) can proceed (during/after RAN4#116).

    - If alignment in step-2 proves difficult, RAN4 should identify the key factors causing discrepancies and may need to initiate following evaluations to reassess step-2 alignment (during/after RAN4#116).

    - If alignment remains unachievable after step-3, RAN4 may then trigger discussions and evaluations on a reference model for CSI prediction (e.g., during/after RAN4#116bis).

    - If alignment achieves after step-3, RAN4 will trigger evaluations on the CSI reporting requirements for CSI prediction(e.g., during/after RAN4#116bis).

Regarding the simulation assumptions for AIML-based CSI prediction, following aspects are updated in RAN4#115 meeting[1],

  - De-prioritize 100Hz Doppler in following simulations

  - Keep only one assumption on gNB TX antennas. Companies are encouraged to provide simulation results under 16 gNB TX antennas as baseline in following simulations.

  - Assumptions on CSI prediction simulation step-3 (TPs as KPI, with practical DL channel estimation) are confirmed in RAN4 #115 meeting.

  - Clarifications on the SGCS calculation,

- for step-1, SGCS1 is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel

- for step-2, Companies are encouraged to provide the following two SGCS metrics:

  - SGCS 2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD.

  - SGCS 3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output  against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth.

Accordingly, we conducted and updated step-1, step-2 and step-3 evaluations for the CSI prediction using the simulation assumptions(highlighted in yellow in Table 1). The simulation results are shown in Table 2, Table 3 and Table 4.

Table 1: Updated Simulation assumptions for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Subcarrier spacing | 15kHz for FDD, 30kHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol for step-1 and step-2 simulation |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| gNB TX antennas | Baseline: 16(N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements Optional: 32(N1,N2)=(4,4) as defined in Rel-15 type1 |
| UE RX antennas | 4 |
| Channel model | TDLA30 with XP medium correlation as baseline |
| Doppler spread | 20Hz, 50Hz |
| Channel estimation for CSI prediction | Step-1: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Step-2: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., to check SGCS after CSI feedback). Step-3: - Practical DL channel estimation, use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| CSI-RS configuration | For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5). - For 32Tx Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0, l1); (5, 12). |
| Rank per UE | Step-1: Rank 1 and 2 to do quick check Step-2 and Step-3: Rank 2 as baseline |
| CSI feedback assumption | Step-1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account |
|  | Step-2 and Step-3: take CSI feedback into account - Use Rel-18 eTypeII-Doppler, N4 = 1， N4>1 can be further discussed in the future if needed |
| Test setup(timeline) | Only for step-3 For FDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 4 slots Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4) - Test setup for FDD test setup with Periodic CSI-RS configuration is captured in Table2 For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note : If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3 FFS test setup for aperiodic CSI-RS configuration |
| SNR assumption | Only for step-3 Claimed by companies for step-3 first round simulation (TP as KPI) FFS: other suggestions |
| CSI feedback overhead | PC7 as baseline |
| Baseline | FFS - Random PMI with Rel-15 Type I single panel codebook if needed (e.g., for Step-3 or performance requirements) |
| KPI | Step-1: - check SGCS before CSI feedback - SGCS is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), Step-2: - check SGCS after CSI feedback - Encouraged companies to provide the following two SGCS metrics: - SGCS 2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD. - SGCS 3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth. - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), Step-3: - check TPs after CSI feedback for both AI based CSI prediction and the baseline(Random PMI with Rel-15 Type I single panel codebook), as well as the SNR value corresponding to 90% of maximum TP of the AI based CSI prediction. - Companies are also encouraged to provide the TP curve with corresponding SNRs. - The test parameters for TPs simulation can be referred as 38.101-4 section of 6.3 - FDD with 16Tx4Rx: Table 6.3.3.1.8-1 - FDD with 32Tx4Rx: Table 6.3.3.1.4-1: - TDD with 16Tx4Rx: Table 6.3.3.2.8-1 - TDD with 32Tx4Rx: Table 6.3.3.2.4-1: |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training data and test data | Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz,); 20Hz Doppler spreads. 50Hz Doppler spreads. |
| Companies can provide the model complexity information (e.g., flops, backbone) of their AI/ML CSI prediction models. |  |

Table 2. Simulation results(step-1) for AIML based CSI prediction

|  | TX | Train | 20Hz |  |  |  |  | 50Hz |  |  |  |  | 100Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) |
| SGCS1 | 16 | Non-mix | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.8149 | 0.9585 | 0.9401 | 0.9585 | 0.9217 | 0.3853 |  |  |  |  |  |
|  |  | Mix | 0.9999 | 0.9999 | 0.9999 | 0.9999 |  | 0.9566 | 0.9374 | 0.9566 | 0.9181 |  |  |  |  |  |  |
| SGCS1 | 32 | Non-mix | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.8318 | 0.9597 | 0.9401 | 0.9596 | 0.9205 | 0.4170 | 0.2824 | 0.2123 | 0.2824 | 0.1422 | 0.2774 |
|  |  | Mix | 0.9997 | 0.9998 | 0.9999 | 0.9997 |  | 0.9591 | 0.9394 | 0.9591 | 0.9196 |  | 0.2871 | 0.2149 | 0.2871 | 0.1426 |  |
| SGCS1 is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3. Simulation results(step-2) for AIML based CSI prediction

|  | TX | Train | 20Hz |  |  |  |  |  | 50Hz |  |  |  |  |  | 100Hz |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) |
| SGCS2 | 16 | Non-mix | - | 0.7605 | 0.812 | 0.709 | 0.6772 | 0.4986 |  | 0.7202 | 0.7814 | 0.659 | 0.3500 | 0.1887 |  |  |  |  |  |  |
|  |  | Mix | - | 0.7603 | 0.8122 | 0.7084 |  |  |  | 0.7194 | 0.7809 | 0.6578 |  |  |  |  |  |  |  |  |
| SGCS3 | 16 | Non-mix |  | 0.9819 | 0.9859 | 0.9779 | 0.6933 | 0.5321 |  | 0.8111 | 0.8547 | 0.7674 | 0.3548 | 0.1961 |  |  |  |  |  |  |
|  |  | Mix |  | 0.9775 | 0.9829 | 0.9721 |  |  |  | 0.8079 | 0.8528 | 0.7629 |  |  |  |  |  |  |  |  |
| SGCS2 | 32 | Non-mix | - | 0.7239 | 0.7884 | 0.6593 | 0.6743 | 0.4752 | - | 0.687 | 0.7613 | 0.6127 | 0.3834 | 0.1872 |  | 0.2165 | 0.2862 | 0.1468 | 0.2811 | 0.1449 |  |
|  |  | Mix | - | 0.7236 | 0.7881 | 0.6591 |  |  | - | 0.6864 | 0.7608 | 0.6119 |  |  |  | 0.2206 | 0.2923 | 0.1489 |  |  |  |
| SGCS3 | 32 | Non-mix |  | 0.9786 | 0.9842 | 0.9729 | 0.7113 | 0.5360 |  | 0.8174 | 0.8631 | 0.7716 | 0.3956 | 0.2076 |  | 0.2343 | 0.3013 | 0.1673 | 0.2962 | 0.1668 |  |
|  |  | Mix |  | 0.9716 | 0.9786 | 0.9645 |  |  |  | 0.8151 | 0.8619 | 0.7683 |  |  |  | 0.2403 | 0.3091 | 0.1714 |  |  |  |
| SGCS 2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD. SGCS 3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4. Simulation results(step-3) for AIML based CSI prediction

| TX | Train | 20Hz Doppler spread |  |  |  |  | 50Hz Doppler spread |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | tue_AI/ML (bits/slot) | SNRue_AI/ML | trnd (bits/slot) | γAI/ML |  | tue_AI/ML (bits/slot) | SNRue_AI/ML | trnd (bits/slot) | γAI/ML |  |
| 16 | Non-mix | 21665 | 3.7 dB | 2470 | 8.8 |  | 21665 | 4.0 dB | 2790 | 7.8 |  |
|  | Mix | 21665 | 3.9 dB | 2640 | 8.2 |  | 21665 | 4.0 dB | 2790 | 7.8 |  |
| - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/MLusing the precoders configured according to the UE reports. - trnd is the throughput measured at SNRue_AI/ML with random precoding. trnd is obtained with Rel-15 type I single panel codebook |  |  |  |  |  |  |  |  |  |  |  |

After accounting for CSI feedback, for TDL channels, the SGCS2 (calculated by comparing the feedbacked SVD of the model output with the SVD of the ground truth raw channel) deteriorates significantly. This may be due to the codebook-based CSI feedback inherently introduces quantization errors compared to the SVD eigenvectors from the ground truth channel. However, when the ground truth eigenvectors also incorporate the quantization errors from CSI feedback, the SGCS3 (calculated by comparing the feedbacked SVD of the model output with the feedbacked SVD of the ground truth raw channel) shows much less degradation.

Regarding the simulation results of step-3, under the condition of TX=16 and rank=2, the throughput performance obtained by randomly selecting a Type I codebook is relatively poor. However, considering that the current baseline benchmark for PMI evaluation in RAN4 is also based on randomly selected Type I codebooks, this ensures fairness when comparing different candidate schemes. The key difference between different schemes(e.g. CSI prediction models from different companies) will primarily be reflected in their γ values relative to the baseline benchmark.

Observation 1: Regarding the updated simulation for AI/ML-based CSI prediction, the updated simulation results are shown in Table 2, Table 3 and Table 4.

Observation 2: For step-2, the SGCS calculated by comparing the feedbacked SVD of the model output with the SVD of the ground truth raw channel deteriorates significantly. The SGCS calculated by comparing the feedbacked SVD of the model output with the feedbacked SVD of the ground truth raw channel shows much less degradation.

Observation 3: For step-3, under the condition of TX=16 and rank=2, the throughput performance obtained by randomly selecting a Type I codebook is relatively poor. The key difference between these schemes (e.g. from different companies) will primarily be reflected in their γ values relative to the baseline benchmark.

For the training and test data consistency, following agreements have been made in RAN4#114 meeting:

Agreement:

- further discuss the following options

  - Option 1: Consistency between training data and test data should be ensured

    - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test

  - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations

According to our understanding, the consistency between training data and test data should be ensured. The only difference between option1 and option2 is: Option 1 aims to determine whether additional information/indication is needed to explicitly guarantee the consistency. Option 2 assumes that the current test environment is sufficiently stable and capable of reproducing test results. Thus, for both of them, ensuring consistency between training and test data is a prerequisite. The unclear parts are 1) whether the test emulated channel is reproducible, 2) whether additional information/specifications are required,.

Proposal 3: Consistency between training data and test data should be ensured

    - FFS: Whether the test emulated channel will be similar to the one used in RAN4 simulations

    - FFS: Whether anything special should be done during the test

- Conclusions

In this contribution, we have discussed the demodulation and/or CSI reporting requirements for CSI prediction, and got the following proposals:

Proposal 1: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction, RAN4 should first assess the CSI prediction evaluation results across different companies before determining whether a reference model is necessary.

Proposal 2: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction,

    - If RAN4 achieves a relatively aligned CSI prediction evaluation results in step-2, further evaluation (step-3 comparisons) can proceed (during/after RAN4#116).

    - If alignment in step-2 proves difficult, RAN4 should identify the key factors causing discrepancies and may need to initiate following evaluations to reassess step-2 alignment (during/after RAN4#116).

    - If alignment remains unachievable after step-3, RAN4 may then trigger discussions and evaluations on a reference model for CSI prediction (e.g., during/after RAN4#116bis).

    - If alignment achieves after step-3, RAN4 will trigger evaluations on the CSI reporting requirements for CSI prediction(e.g., during/after RAN4#116bis).

Observation 1: Regarding the updated simulation for AI/ML-based CSI prediction, the updated simulation results are shown in Table 2, Table 3 and Table 4.

Table 2. Simulation results(step-1) for AIML based CSI prediction

|  | TX | Train | 20Hz |  |  |  |  | 50Hz |  |  |  |  | 100Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) |
| SGCS1 | 16 | Non-mix | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.8149 | 0.9585 | 0.9401 | 0.9585 | 0.9217 | 0.3853 |  |  |  |  |  |
|  |  | Mix | 0.9999 | 0.9999 | 0.9999 | 0.9999 |  | 0.9566 | 0.9374 | 0.9566 | 0.9181 |  |  |  |  |  |  |
| SGCS1 | 32 | Non-mix | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.8318 | 0.9597 | 0.9401 | 0.9596 | 0.9205 | 0.4170 | 0.2824 | 0.2123 | 0.2824 | 0.1422 | 0.2774 |
|  |  | Mix | 0.9997 | 0.9998 | 0.9999 | 0.9997 |  | 0.9591 | 0.9394 | 0.9591 | 0.9196 |  | 0.2871 | 0.2149 | 0.2871 | 0.1426 |  |
| SGCS1 is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3. Simulation results(step-2) for AIML based CSI prediction

|  | TX | Train | 20Hz |  |  |  |  |  | 50Hz |  |  |  |  |  | 100Hz |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) |
| SGCS2 | 16 | Non-mix | - | 0.7605 | 0.812 | 0.709 | 0.6772 | 0.4986 |  | 0.7202 | 0.7814 | 0.659 | 0.3500 | 0.1887 |  |  |  |  |  |  |
|  |  | Mix | - | 0.7603 | 0.8122 | 0.7084 |  |  |  | 0.7194 | 0.7809 | 0.6578 |  |  |  |  |  |  |  |  |
| SGCS3 | 16 | Non-mix |  | 0.9819 | 0.9859 | 0.9779 | 0.6933 | 0.5321 |  | 0.8111 | 0.8547 | 0.7674 | 0.3548 | 0.1961 |  |  |  |  |  |  |
|  |  | Mix |  | 0.9775 | 0.9829 | 0.9721 |  |  |  | 0.8079 | 0.8528 | 0.7629 |  |  |  |  |  |  |  |  |
| SGCS2 | 32 | Non-mix | - | 0.7239 | 0.7884 | 0.6593 | 0.6743 | 0.4752 | - | 0.687 | 0.7613 | 0.6127 | 0.3834 | 0.1872 |  | 0.2165 | 0.2862 | 0.1468 | 0.2811 | 0.1449 |  |
|  |  | Mix | - | 0.7236 | 0.7881 | 0.6591 |  |  | - | 0.6864 | 0.7608 | 0.6119 |  |  |  | 0.2206 | 0.2923 | 0.1489 |  |  |  |
| SGCS3 | 32 | Non-mix |  | 0.9786 | 0.9842 | 0.9729 | 0.7113 | 0.5360 |  | 0.8174 | 0.8631 | 0.7716 | 0.3956 | 0.2076 |  | 0.2343 | 0.3013 | 0.1673 | 0.2962 | 0.1668 |  |
|  |  | Mix |  | 0.9716 | 0.9786 | 0.9645 |  |  |  | 0.8151 | 0.8619 | 0.7683 |  |  |  | 0.2403 | 0.3091 | 0.1714 |  |  |  |
| SGCS 2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD. SGCS 3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4. Simulation results(step-3) for AIML based CSI prediction

| TX | Train | 20Hz Doppler spread |  |  |  |  | 50Hz Doppler spread |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | tue_AI/ML (bits/slot) | SNRue_AI/ML | trnd (bits/slot) | γAI/ML |  | tue_AI/ML (bits/slot) | SNRue_AI/ML | trnd (bits/slot) | γAI/ML |  |
| 16 | Non-mix | 21665 | 3.7 dB | 2470 | 8.8 |  | 21665 | 4.0 dB | 2790 | 7.8 |  |
|  | Mix | 21665 | 3.9 dB | 2640 | 8.2 |  | 21665 | 4.0 dB | 2790 | 7.8 |  |
| - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/MLusing the precoders configured according to the UE reports. - trnd is the throughput measured at SNRue_AI/ML with random precoding. trnd is obtained with Rel-15 type I single panel codebook |  |  |  |  |  |  |  |  |  |  |  |

Observation 2: For step-2, the SGCS calculated by comparing the feedbacked SVD of the model output with the SVD of the ground truth raw channel deteriorates significantly. The SGCS calculated by comparing the feedbacked SVD of the model output with the feedbacked SVD of the ground truth raw channel shows much less degradation.

Observation 3: For step-3, under the condition of TX=16 and rank=2, the throughput performance obtained by randomly selecting a Type I codebook is relatively poor. The key difference between these schemes (e.g. from different companies) will primarily be reflected in their γ values relative to the baseline benchmark.

Proposal 3: Consistency between training data and test data should be ensured

    - FFS: Whether the test emulated channel will be similar to the one used in RAN4 simulations

    - FFS: Whether anything special should be done during the test

- References

- R4-2508102, WF on updated simulation assumptions for AI based CSI prediction

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2