3GPP TSG-RAN WG4 Meeting #116	R4-2510962

Bengaluru, IN, August 25th – 29th 2025

Agenda Item:	7.21.3.1

Source:	Ericsson

Title:	New SBFD BS RF requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of new BS RF requirements for SBFD. Discussion on impact on existing BS RF TX and RX requirements for SBFD and SBFD general aspects are presented in separate submissions [1-3].

# 2	Discussion

In-channel adjacent sub-band leakage

### Sub-topic 4-2: In-channel adjacent subband leakage ##### Issue 4-2-1: Assumption for in-channel adjacent subband leakage requirement - No agreement in this meeting and the way forward from RAN4#114bis can still be followed. ##### Issue 4-2-2: Absolute limit for in-channel adjacent subband leakage Agreement - For absolute limit for in-channel adjacent subband leakage: - For FR2 BS, legacy ACLR absolute basic limit (Table 9.7.3.3-2 for FR2 in TS 38.104) is followed. - For FR1 BS, - For FR1 LA SBFD BS, no relaxation compared with absolute ACLR limit (DU configuration) or absolute CACLR limit (DUD) are needed. - For FR1 WA and MR SBFD-capable BS, - Option 1: [3]dB relaxation is provided. - Option 2: no relaxation from corresponding existing requirement

During the discussion in RAN4#114bis, one question was raised on the assumption for in-channel adjacent subband leakage requirement. For example, is it possible that for one operator some sites deploy SBFD with a subband configuration of 80 MHz - 20 MHz (DU), while other sites deploy with another subband configuration of 60 MHz - 40 MHz (DU).

In our view, to ensure network performance and avoid inter-site interference and receiver saturation, operators have to deploy SBFD uniformly across all sites, rather than

- selectively at just a few locations where UL coverage enhancement is desired, while other sites deploy regular TDD.

- nor with varying subband configurations across sites.

Without this uniformity, even small configuration mismatches can lead to undesirable effects. When SBFD is inconsistently implemented, the overlapping frequency portions of UL and DL transmission between adjacent cells can lead to significant interference which saturates the receiver, degrading overall system performance.

Therefore, it is proposed to clarify it in the specification indicating that SBFD subband configurations should remain consistent across all sites.

- Operators cannot deploy SBFD selectively at just a few locations where UL coverage enhancement is desired, while other sites deploy regular TDD.

- Operators cannot deploy SBFD with varying subband configurations across sites.

Add a note in TS 38.104 where appropriate that “Operators shall deploy SBFD uniformly across all sites with exactly the same subband configuration.”

As to the requirement level, the proposal is to reuse the ACLR absolute value as defined in Table 6.6.3.2-2 in TS 38.104 [4].

Table 6.6.3.2-2 in TS 38.104 [4]: Base station ACLR absolute basic limit

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

Reuse the ACLR absolute value as defined in Table 6.6.3.2-2 in TS 38.104 for FR1.

In-channel adjacent sub-band Blocking and adjacent sub-band selectivity

### Sub-topic 4-3: In-channel adjacent subband blocking ##### Issue 4-3-1: In-channel adjacent subband blocking requirement Agreement - Wanted signal mean power, i.e., degradation level for Refsens level - For conducted in-channel adjacent subband blocking requirement, as defined for in-band blocking requirement, 6dB degradation from Refsens - 6dB degradation on top of OTA REFSENS degradation for in-channel adjacent subband blocking requirement, and own Tx is ON. - On the level of in-channel adjacent subband blocking requirement: - For FR1 WA BS: - [-31+3 = -28] dBm - FFS isolation between sectors can captured in specification - For FR1 MR BS: - [-32] dBm. - FFS isolation between sectors can captured in specification - For FR1 LA BS: - [-34] dBm.

In RAN4#113 meeting, it was agreed to define a new requirement of in-channel adjacent subband blocking requirement for SBFD capable BS and further study the requirement level.

When defining the requirement level of in-channel adjacent subband blocking requirement, spatial isolation is an important factor to consider. In SI, spatial isolation was widely discussed and was summarized in TR 38.858 [5], where the spatial isolation values evaluated by different companies were captured in Table 9.2.2.1-1 (partly copied below for reading purpose). Considering the values provided by all companies comprehensively, a compromise value of 80 dB spatial isolation between sectors is selected.

For a SBFD capable BS with 53 dBm TRP, -27 dBm is expected at the UL subband, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors.

During the discussion at the RAN4#114bis meeting, some companies propose that CLI should be considered when defining in-channel adjacent subband blocking. In our view, the inter-sector interference is not about avoiding the beam direction from one site to another. It is about avoiding beam directions that cause unintended radiation, which radiates backward from one sector to another in the near field, such as sidelobes radiating backward across the near field. The CLI handling schemes for interaction in the near field at the same site differ from what is currently being studied in RAN1 and have not been studied so far.

When considering inter-site interference, it is important to acknowledge that while CLI handling schemes can suppress a portion of the interference, they cannot completely eliminate it. On one hand, RX digital beamforming cannot be considered in the CLI handling scheme, because receiver blocking occurs at each sub-array, and once a sub-array is blocked, digital mitigation is no longer possible because the receiver and ADC are already saturated. On the other hand, the CLI handling scheme is not always feasible. In practical deployments, especially in dense multi-site TDD networks, various factors limit the effectiveness of these schemes. Therefore, the CLI handling scheme should not be considered when defining in-channel adjacent subband blocking requirement.

- In-channel adjacent subband blocking requirement measures the interaction between sectors in the near field, and CLI handling schemes differ from what is studied in RAN1 and have not been studied so far.

- CLI handling schemes for the case of inter-site interference could suppress a portion of the interference, but they cannot completely eliminate it, because it is not always feasible, and RX digital beamforming cannot be considered.

Define in-channel adjacent subband blocking requirement without considering CLI handling schemes.

For FR1 WA, a SBFD capable BS with 53 dBm TRP, specify -27 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors.

In addition to specifying the interfering signal power, the center frequency and type of interfering signal should also be defined. The purpose of defining in-channel adjacent subband blocking is to evaluate the impact of intra-operator inter-sector or inter-site interference. For the DU/UD case, the interfering signal is expected to originate from the DL subband excluding its own transmitter with its center frequency located at the midpoint of the DL subband. Similarly, in the DUD case, the interfering signals also originate from the DL subbands. Therefore, the bandwidth of the interfering signal should be equivalent to the DL subband, and its center frequency should align with the center of the corresponding DL subband. To support this, a new symbol FC,SBFD,DL for SBFD DL subband, which is similar to FC used in ICS, can be introduced to represent this reference point. Regarding the type of interfering signal, it is an NR DL signal, with a size specified by NRB,SBFD,DL, and as discussed in previous sessions, due to overhead, we believe it is unlikely that the UL/DL subband size will be less than 10 MHz, and that using 30 kHz SCS is practical.

- For in-channel adjacent subband blocking requirement, the interfering signals originate from the intra-operator inter-sector/inter-site DL subbands. Therefore, the bandwidth of the interfering signal should be equivalent to the DL subband, and its center frequency should align with the center of the corresponding DL subband.

Introduce a new symbol FC,SBFD,DL for SBFD DL subband, which is similar to FC used in ICS, to represent the reference point for interfering signal.

Use an NR DL signal with 30 kHz SCS and size specified by NRB,SBFD,DL as the type of interfering signal.

Table 9.2.2.1-1 in TR 38.858 [5]: FR1 WA BS Co-site Inter-sector Co-channel Interference Analysis Summary (Partial copy)

| FR1 |  |  | Samsung | Samsung | Ericsson | Huawei | Nokia |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  | Wide Area BS (subband filter-1) | Wide Area BS (subband filter-2 and EM conjugated structure) | Wide Area BS | Wide Area BS | Wide Area BS |  |
| BS TX Power dBPTX = ① dBm |  |  | 49 dBm | 49 dBm | 53 dBm | 53 dBm | 54 dBm |  |
| Number of co-site co-channel sectors considered |  |  | 1 | 1 | 2 | 2 | 2 (plus 1 sector for self-interference) |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαCSSI-frequency, TX = ② dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc |  |
|  |  | Frequency isolation techniques used | DPD utilized | DPD utilized | DPD, CFR | DPD | DPD, digital filtering |  |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI-spatial = ③ dBc | 75 dBc | 100 dBc | 75-90 dBc (Varies depending on scheduled beam directions) | 85 dBc | 60-80 dBc |  |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used | Based on 75dB for typical spatial isolation | Based on 75dB for typical spatial isolation and additional 25dB by installing EM conjugated structure between sectors | Typical site layout with around 400mm between sectors | Spatial separation between TX panel with absorbing material and choke structure. | Spatial separation between TX/RX panel; cross polarization |  |

For FR2-1, the spatial isolation values evaluated by different companies were captured in Table 9.5.2.1-1 (partly copied below for reading purpose). Considering the values provided by all companies comprehensively, a compromise value of 87 dB spatial isolation between sectors is selected.

For a SBFD capable BS with 40 dBm TRP, -47 dBm is expected at the UL subband, on the premise that operators do proper site dimensioning design in real networks and achieve 87 dB spatial isolation between sectors.

For FR2 WA, a SBFD capable BS with 40 dBm TRP, specify -47 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 87 dB spatial isolation between sectors.

Table 9.5.2.1-1 in TR 38.858 [5]: Co-channel inter-sub-band co-site inter-sector interference analysis(Partial copy)

| FR1 (or FR2-1) |  |  | Samsung | Huawei | Ericsson |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  | FR2-1 BS |  |  |  |  |
| BS TX Power dBPTX = ① dBm |  |  | 30 dBm | 35 dBm | 30 dBm | 35 dBm | 40 dBm |
| Number of co-site co-channel sectors considered |  |  | 1 | 2 | 2 | 2 | 2 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαCSSI-frequency, TX = ② dBc | 28 dBc | 28 dB | 28 dBc | 28 dBc | 28 dBc |
|  |  | Frequency isolation techniques used | Without DPD | DPD | Digital Filtering, CFR | Digital Filtering, CFR | Digital Filtering, CFR |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI-spatial = ③ dBc | 87 dBc | 100 dB | 75-98 dBc (depending on beam steering directions) |  |  |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used | Based on 87dB for typical spatial isolation | Spatial separation between TX/RX panel and surface wave suppression techniques | Typical site deployment with 400mm between sectors |  |  |

Based on the discussion above, the requirement of in-channel adjacent subband blocking is presented in draft CR [6].

Endorse draft CR R4-2510965 for introduction of in-channel adjacent subband blocking requirement in TS 38.104.

# Conclusion

In this contribution, discussion on new RF requirements for SBFD-capable BS is presented with the following observations and proposals.

Observation 1	Operators cannot deploy SBFD selectively at just a few locations where UL coverage enhancement is desired, while other sites deploy regular TDD.

Observation 2	Operators cannot deploy SBFD with varying subband configurations across sites.

Observation 3	In-channel adjacent subband blocking requirement measures the interaction between sectors in the near field, and CLI handling schemes differ from what is studied in RAN1 and have not been studied so far.

Observation 4	CLI handling schemes for the case of inter-site interference could suppress a portion of the interference, but they cannot completely eliminate it, because it is not always feasible, and RX digital beamforming cannot be considered.

Observation 5	For in-channel adjacent subband blocking requirement, the interfering signals originate from the intra-operator inter-sector/inter-site DL subbands. Therefore, the bandwidth of the interfering signal should be equivalent to the DL subband, and its center frequency should align with the center of the corresponding DL subband.

Proposal 1	Add a note in TS 38.104 where appropriate that “Operators shall deploy SBFD uniformly across all sites with exactly the same subband configuration.”

Proposal 2	Reuse the ACLR absolute value as defined in Table 6.6.3.2-2 in TS 38.104 for FR1.

Proposal 3	Define in-channel adjacent subband blocking requirement without considering CLI handling schemes.

Proposal 4	For FR1 WA, a SBFD capable BS with 53 dBm TRP, specify -27 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors.

Proposal 5	Introduce a new symbol FC,SBFD,DL for SBFD DL subband, which is similar to FC used in ICS, to represent the reference point for interfering signal.

Proposal 6	Use an NR DL signal with 30 kHz SCS and size specified by NRB,SBFD,DL as the type of interfering signal.

Proposal 7	For FR2 WA, a SBFD capable BS with 40 dBm TRP, specify -47 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 87 dB spatial isolation between sectors.

Proposal 8	Endorse draft CR R4-2510965 for introduction of in-channel adjacent subband blocking requirement in TS 38.104.

# References

R4-2510961, SBFD general aspects, Ericsson

R4-2510963, Impact on SBFD BS RF TX requirements, Ericsson

R4-2510964, Impact on SBFD BS RF RX requirements, Ericsson

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

TR 38.858, “Study on Evolution of NR Duplex Operation”, 3GPP

R4-2510965, Draft CR to 38.104 on adding receiver in-channel requirements for SBFD, Ericsson