3GPP TSG-RAN WG4 Meeting #116	R4-2511403

# Bangalore, India, 25th – 29th August 2025

Agenda item:		7.6.4.2

Source: 		Qualcomm Inc.

Title: 		UE RF requirements for NTN less than 5 MHz

Document for:		Approval

## 1	Introduction

3 MHz channel bandwidth was specified for TN in rel-18. In the work item approved for NTN, 3 MHz channel bandwidth is to be added with RAN1-design based on rel-18, with no expected RAN1 impact. In this contribution UE RF requirements for 3 MHz channel bandwidth are discussed.

## 2	Discussion

A-MPR

A detailed A-MPR evaluation considering also transceiver wideband noise, transceiver non-linearities, phase noise and digital processing artefacts in addition to PA emissions was done to check how much A-MPR is appropriate for fulfilling NS_24 requirements. It was observed that transceiver noise contribution is relevant and results in increased A-MPR.

Generally, transceiver emissions are not as directly related to output power as PA emissions, and usually PA emissions are dominant. However, -50 dBm/MHz at a small offset is more stringent than nearly all other 3GPP emission limits. When looking at existing requirements, e.g. for 5 MHz CBW for n256 at least 4 dB of A-MPR is applied for all RB allocations within the center frequency range of 1997.5 to 2002.5 MHz.

Similarly, for IoT NTN band 256, 3.5 dB A-MPR is allowed for PC3 for both cat M1 and NB-IoT for the whole operating band with no center frequency restriction. As such, the transceiver noise has been taken into account in 3GPP also before, but is has not been much discussed.

A simplified noise budget is presented in Table 1.

Table 1: Noise contributions from PA and transceiver

It can be seen, that even though PA emissions would provide a margin to the emission limit, due to transceiver contributions total margin is negative. Phase noise will improve at larger frequency offsets, but small A-MPR is needed for allocations close to upper edge of the band.

It would be beneficial to have similar A-MPR for 3 MHz as there is for 5 MHz CBW as well as NTN IoT, as this also better aligns the implementation complexity, as in practice same UE and same transceiver can be used for these.

Observation 1: 5 MHz CBW, cat M and NB-IoT are allowed A-MPR for all RB allocations closest to NS_24 limit.

Observation 2: It is beneficial to roughly align the A-MPR with 5 MHz CBW and NTN IoT as same UE and transceiver can support all of these.

Observation 3: Noise profile falls down at larger frequency offsets, which allows some A-MPR optimization compared to older specification.

Based on this discussion, it is propose to capture A-MPR for 3 MHz channel as in Table 2 and Table 3

Proposal 1: Define A-MPR for 3 MHz channel for NS_24 as in Table 2 and Table 3

Table 2: A-MPR regions for 3MHz, NS_24

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS | LCRB*12*SCS | A-MPR |
| 3MHz | 2000.5 < fc <= 2003.5 |  | > 1.08 | A1 |
|  |  |  | < 1.08 | A2 |
|  | 1997.5 < fc <= 2000.5 |  | >= 1.8 | A4 |
|  |  |  | < 1.8 | A3 |

Table 3: A-MPR values for 3 MHz, NS_24

|  | Modulation | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 6.0 | 3 | 2 | 2.0 |
|  | QPSK | 7.0 | 3 | 2 | 3.5 |
|  | 16QAM | 8.0 | 3.5 | 2.5 | 3.5 |
|  | 64QAM | 8.5 | 3.5 | 2.5 | 3.5 |
| CP-OFDM | QPSK | 9.0 | 3.5 | 2.5 | 5.0 |
|  | 16QAM | 9.0 | 3.5 | 2.5 | 5.0 |
|  | 64QAM | 9.0 | 3.5 | 2.5 | 5.0 |

LS from RAN2

RAN4 received an LS [2] from RAN2 stating “RAN2 further agreed that to avoid backward capability issues, TN approach is reused for less than 5MHz in NTN. Based on which, similar note as note 2 in Table 5.4.3.1-1 of 38.101-1 needs to be added in the Table 5.4.3.1-1 of 38.101-5.”

In addition to the this note, TS 38.101-1 includes in operating band table NOTE 20: Operating band n200 is a reserved value. This should be done also in TS 38.101-5.

Proposal 2: In addition to RAN2 request, also add a note in operating band table to indicate n200 is a reserved value.

## 3	Conclusion

In this contribution UE RF requirements for less than 5 MHz for NTN were discussed. Following observations and proposals were made.

Observation 1: 5 MHz CBW, cat M and NB-IoT are allowed A-MPR for all RB allocations closest to NS_24 limit.

Observation 2: It is beneficial to roughly align the A-MPR with 5 MHz CBW and NTN IoT as same UE and transceiver can support all of these.

Observation 3: Noise profile falls down at larger frequency offsets, which allows some A-MPR optimization compared to older specification.

Proposal 1: Define A-MPR for 3 MHz channel for NS_24 as in Table 2 and Table 3

Proposal 2: In addition to RAN2 request, also add a note in operating band table to indicate n200 is a reserved value.

## 4	References

[1]  R4-2508765, WF for [115][310] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi

[2]  R2-2504788/R4-2509016, Reply LS on UE capability signalling for NTN less than 5MHz, RAN2