3GPP TSG-RAN WG4 Meeting #116	R4-2511409

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on Fast SCell activation for UE supporting Rel-18 EMR core part

Agenda item:	7.14.4

Document for:	Discussion

Introduction

In the last meeting the remaining issue of capability was discussed but it is still left open.

## Sub-topic 2-1 Core requirements of Fast SCell activation for UE supporting Rel-18 EMR Issue 2-1-1: UE capability for EMR-based fast SCell activation FFS: - Option 1: Introduce a UE capability for R19 SCell activation based on EMR - Prerequisite feature groups: 39-8 or 39-9 in R18 RAN4 feature list - Need for the gNB to know if the feature is supported: No - Granularity: Per-UE - FDD/TDD differentiation: No - FR1/FR2 differentiation: Yes Option 2: Not to introduce additional UE capability

Discussion

Whether to define new UE capability

Issue 2-1-1: UE capability for EMR-based fast SCell activation

In the release 18, two capabilities were introduced:

| measValidationReportEMR-r18 Indicates whether the UE supports measurement validation and report based on EMR measurement during connection setup/resume for fast CA/DC setup. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. A UE supporting this feature shall also indicate support of idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| measValidationReportReselectionMeasurements-r18 Indicates whether the UE supports measurement validation based on reselection measurements during IDLE/INACTIVE state and reporting for fast CA/DC setup. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |

If UE implements only rel-18 features, the UE is able to use the release 18 defined reporting framework. However, no performance requirements related to these capabilities are guaranteed. This means that the implementations across UEs may vary.

UE may report measurement results based on rel18 reporting framework.

In release19 we introduced performance requirements that guarantee minimum performance for the requirements. This means for the network that there is guaranteed limit when the operation is performed. From the network point of view, it would be ideal if the UE supporting rel-18 feature would automatically support also rel-19 feature. However, since these features are in different releases, it would be unclear for the network whether the UE reports are based on rel-18 framework or based on rel-19 requirements.  When considering that UE may cherry-pick features from each release, introducing rel-19 capability on top of rel-18 functionality seems to make sense. In the capability design of rel-19 the rel-18 capabilities need to be pre-requisite for rel-19 feature.

Additionally, rel-18 UE would not automatically be able to pass rel-19 UE test cases.

It would be unclear for the network if UE implements rel-18 but does not meet rel-19 performance requirements and pass test cases for rel-19.

Define capability for Rel-19 feature. Use existing capabilities from rel-18 as pre-requisite.

Conclusion

The following Observations and Proposals were made:

Observation 1: UE may report measurement results based on rel18 reporting framework.

Observation 2: It would be unclear for the network if UE implements rel-18 but does not meet rel-19 performance requirements and pass test cases for rel-19.

Proposal 1: Define capability for Rel-19 feature. Use existing capabilities from rel-18 as pre-requisite.