3GPP RAN WG4 Meeting #116																	R4-2511500

Bangalore, India, 25th – 29th August 2025

Agenda item:	10

Source:	Apple

Title:	On practical aspects of contributions with JSON files

WI/SI:

Release:	Rel-19

Document for:	Decision

# 1	Introduction

The number of RAN WG4 band combinations have been growing all the time creating more challenges on how to introduce new band combinations, maintain and correct existing ones in a simple and automated way. To address these issues RAN WG4 decided to establish a new framework to store all band combinations in a database, which would ensure consistent way of storing corresponding information. Even more importantly, this approach can also enable a unified and structured way of making changes with an ability to trace back modifications.

As per WF at the RAN4#115 meeting [1], companies submitting band combination CRs are also invited to bring the corresponding changes to the JSON files used store band combination data. Following that request we submitted the several correction CRs ([2] [3] [4] and [5]) that contain attachments with modifications for the JSON files. While preparing these papers several practical issues arose, which we decided to highlight in this paper so that RAN WG4 can discuss and decide what we would be the best way to prepare such papers.

# 2	JSON files

## 2.1	General aspects

While preparing band combination CRs with JSON files several practical issues arose, which we decided to summarise in this paper for RAN WG4 to discuss further what would be the best practical way to organise our work.

a)	CR title page. Even though it is not a big practical issue, it is not clear whether the CR title page should provide any indication on whether the JSON file is attached. Our view is that it would be nice to have some indication on the CR cover page so that other companies, and a secretary, know that the JSON attachment exists. And if so agreed, it would be also nice to clarify which field on the CR cover page should contain that information.

b)	Format of changes. One of the first issues which we faced is in which format we should be submitting JSON file modifications. The simplest approach is to attach the original JSON file with the modifications. While it is doable and is perfectly fine, this would be somewhat against the principles of "software development" approach which we try to adopt as it may result in the same issues with implementation of changes if several companies bring changes to the same band combination. In this case each company may submit a modified JSON file, not necessarily with the same changes, which will require somebody to figure out what the actual changes are and to merge them accordingly. Thus, a more elegant approach would be to submit a "diff" file containing actual changes (or both modified JSON file and a diff file). Having "diff" files firstly simplifies and automates the process of applying changes coming from different sources. Furthermore, if the changes are conflicting, the corresponding tools can immediately indicate it thus eliminating human errors if the same changes would be applied manually.

c)	"Diff" file format. In case if we agree to attach (at least) a diff file, then it would be nice to clarify which format we should use because there exist several formats for actual changes. We would suggest using so-called "unified diff" format, which can be produced by quite many free tools, such as Unix "diff -u" command line tool ported to almost all environments and platforms.

d)	Further considerations. Referring back to points discussed in a), b) and c), if there is a clear way to detect that a particular contribution has changes to the JSON files and the changes are in the "diff" format, it is possible to create a script that would automatically extract all "diff" files and would try to apply them. This can be done right after the submission deadline so that conflict, if any, can be resolved even before the official start of the meeting.

e)	Structure of the .zip archive. If RAN WG4 plans to follow the approach of submitting changes both in the "word" and JSON formats, then we would like to get more clarifications on how the final .zip archive should be structured. We suggest following the approach that all files should start with the same Tdoc number as the filename so that once extracted we can see which JSON (diff) file relates to which contribution.

## 2.2	JSON schema

While working on the existing JSON schema it has been found that certain information could be useful to be included for processing purposes.

- RAT/Generation. For data processing of band combinations, it can be useful to have a field which provides the RAT or generation. Currently this needs to be identified by checking the bands used in the band combination which will get more challenging when moving to 6G.

- Release version. Introduce a field containing the latest release version of the band combination.

- Carrier Aggregation. The bcID field is written down without any prefix i.e. ‘CA_’, ‘DC_’. Currently, only CA combinations are converted to JSON. When introducing DC, EN-DC and NE-DC it would be beneficial to introduce a new field which captures the very nature of the band combination. Especially, as in certain cases the bcID would be the same for CA and DC without any way to distinguish between.

- WI-code. When writing CRs it is mandatory to provide the work item codes for each band combination which has been modified/corrected. It’s a time-consuming task to find all the work items and check which WI the combination is defined in. It would be a considerable quality of live improvement if the WI-code would be captured in the JSON files. To reduce workload, the WI-code field could be mandatory only for newly introduced band combinations and those which are modified or corrected.

- Single Bands. While not utterly necessary the following additional fields could ease data processing.

  - Duplex Mode. Add field with the duplex mode of the band (e.g. FDD, TDD, SUL …)

  - Frequency range. Add field specifying the frequency range (e.g. FR1, FR2-1, FR2-2 )

  - Actual frequencies of UL and DL. Add fields with the UL/DL frequencies

Based on the presented summary we ask RAN WG4 to discuss at least the following aspects so that all companies can follow the same way of organising contributions with the JSON files.

Proposal 1:	To discuss whether a CR title page should contain any information indicating that the corresponding JSON file is attached.

Proposal 2:	To discuss the exact format of the JSON file with modification (e.g. the original file with the modifications and/or the "diff" file containing actual changes)

Proposal 3:	To discuss how the corresponding files should be structured in the .zip archive (e.g. flat structure with all files prefixed with the Tdoc number)

Proposal 4:	Introduce new fields to the JSON files capturing important information such as RAT/Generation, release version, carrier type (CA, DC, EN-DC, NE-DC etc.), WI-code. For single bands the following fields could be beneficial, Duplex Mode, Frequency Range and UL/DL frequencies.

# References

- R4-2508014, "WF for the ETSI MCC band combination database", Nokia, Ericsson, 3GPP RAN4 #115

- R4-2509547, "CR to 38.101-3 Rel-18 Band Combination Bug Fixes CATF", Apple, 3GPP RAN4 #116

- R4-2509549, "CR to 38.101-1 Rel-19 Band Combination Bug Fixes CATF", Apple, 3GPP RAN4 #116

- R4-2509550, "CR to 38.101-1 Rel-18 Band Combination Bug Fixes CATF", Apple, 3GPP RAN4 #116

- R4-2509552, "CR to 38.101-1 Rel-17 Band Combination Bug Fixes CATF", Apple, 3GPP RAN #116