3GPP TSG-RAN WG4 Meeting # 116	R4-2511611

Bengaluru City, IN, Aug 25 – Aug 29, 2025

Agenda Item: 7.20.5

Source: MediaTek Inc.

Title: Discussion on RRM performance requirements for Rel-19 MIMO

Document for: Discussion

# Introduction

From the revised WID [1] in RAN plenary RAN#105 meeting, RAN4 to specify necessary performance requirements for the MIMO enhancements. RAN4 had some open issues in WF [2] in the last meeting without conclusion yet. We provided our view on performance requirements for CJTC reporting.

# Discussion

In the last meeting, RAN4 has some open issues captured in the WF [2]:

#### Issue 2-1-2: Test scope <Way forward > - Further discuss based on the table below: QC; Nokia; Samsung; Apple; CMCC; Huawei; Ericsson. High level: Define TCs to verify UEIBM; Y; Y; Y; Y; Y; Y; Y. Whether to cover Event-2/Event-1/Event-7?; Y; Event-2 1 TC for Event-2 No for Event-7; Y; Event-2 FFS on others; Event-2&7. Whether to cover Mode A and Mode B?; Mode A FFS on Mode B; Y; Mode A at least. Event-trigger for Intra-cell BM for SSB; Y; Y; Y; Y; Y. Event-trigger for Intra-cell BM for CSI-RS; Y; Y; Y; Y; Y. Event-trigger for Inter-cell BM for SSB; Y; Y; Y; FFS; ?. Event-trigger for Inter-cell BM for CSI-RS; FFS. Whether to cover no time window and eventInstanceCount-r19 > 1?; Y; Y; Y. FR2?; Y. Beam switching delay for TCI states switching; Y Both known and unknown TCI; No MAC CE-based TCI state switching FFS on DCI-based TCI state switching.

Regarding testing the requirements for UE-initiated/event-driven beam management, we prefer to focus on event-2 report triggering. Also, this can be tested for

- Event-trigger for Intra-cell BM for SSB

- Event-trigger for Intra-cell BM for CSI-RS

- Event-trigger for Inter-cell BM for SSB

Proposal 1: RAN4 to prioritize event-2 for testing each of

- Event-trigger for Intra-cell BM for SSB

- Event-trigger for Intra-cell BM for CSI-RS

- Event-trigger for Inter-cell BM for SSB

Issue 2-2-1: Whether to define reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P? <Way forward > - Option 1: (Samsung, Apple, CMCC) - RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P. #### Issue 2-2-2: Whether to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P? <Way forward > - Option 1: (Samsung) - RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F. - Option 2: (Apple) - RAN4 to discuss feasibility of defining accuracy requirements for time offset, frequency offset reporting based on number of samples agreed in core part. - Option 3: (Ericsson) - Develop accuracy requirements for delay, frequency and phase offset UE measurement reports for: - Different number of samples - High and low SNR - Normal and extreme conditions

RAN1 introduce CJTC reporting for delay offset, frequency offset and phase offset. Since RAN4 agreed to define the core requirements for delay offset and frequency offset, mapping table for CJTC reporting including both CJTC-Dd and CJTC-F are required to be defined.

Proposal 2: RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F.

From the simulation result on table 2 and table 3. it can be observed that for low SNR value (-3dB) the quantization (reported interval) is very inaccurate even when the number of samples is increased. Therefore, to guarantee reasonable benefit from the reported intervals of the delay and frequency offsets, we prefer to adopt higher SNR side condition (3dB) for accuracy requirements.

Observation 1: At low SNR value (-3dB) the quantization (reported interval) is very inaccurate even when the number of samples is increased.

Proposal 3: RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F based on the number of samples that will be agreed in the core part.

Regarding testing the measurement period for CJT reporting, RAN4 to consider two test cases for testing CJTC reporting for delay offset and frequency offset.

Proposal 4: RAN4 to consider testing CJTC reporting for delay offset and frequency offset.

For Asymmetric DL sTRP / UL mTRP, on the other hand, RAN4 to discuss whether testing both Scenario#1 and Scenario#2 is required. In our view Scenario#2 is not very different from the legacy multi-DCI based multi-TRP operation. Therefore, RAN4 can focus on testing Scenario#1 in the performance part.

Proposal 5: RAN4 to prioritize testing Scenario#1 for asymmetric DL sTRP / UL mTRP in the performance part.

# Summary

In this paper, the discussion of R19 MIMO is provided. The following observations and proposals are concluded:

Proposal 1: RAN4 to prioritize event-2 for testing each of

- Event-trigger for Intra-cell BM for SSB

- Event-trigger for Intra-cell BM for CSI-RS

- Event-trigger for Inter-cell BM for SSB

Proposal 2: RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F.

Observation 1: At low SNR value (-3dB) the quantization (reported interval) is very inaccurate even when the number of samples is increased.

Proposal 3: RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F based on the number of samples that will be agreed in the core part.

Proposal 4: RAN4 to consider testing CJTC reporting for delay offset and frequency offset.

Proposal 5: RAN4 to prioritize testing Scenario#1 for asymmetric DL sTRP / UL mTRP in the performance part.

# Reference

[1] RP-242394, Revised Work Item: NR MIMO Phase 5, Samsung

[2] R4-2508271, WF on RRM requirements for NR_MIMO_Ph5_Part1