3GPP TSG-RAN4 Meeting #116	R4- 2512844

Bengaluru, India, Aug.25th – 29th, 2025

# Source:	CMCC, ZTE Corporation, Huawei, Murata Manufacturing Co Ltd.

# Title:	TP for TR 38.719-02-01 to introduce CA_n28A-n41(2A)

# Agenda Item:	6.3.3

# Document for:	Approval

- Introduction

- In the current specifications, there is no MSD requirements are defined for UL_n28 or UL_n41.

Band n28 is an FDD band with UL 703 – 748 MHz and DL 758 – 803 MHz. Band n41(2A) is a TDD band whose uplink occupies two non-contiguous segments, 2515 – 2615 MHz (100 MHz) and 2635 – 2675 MHz (40 MHz), giving an aggregated 160 MHz envelope.

This TP will focus on band combination CA_n28A-n41(2A).

- Reference

[1] TR38.718-02-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2),v0.9.0

[2] TR38.719-02-01,Rel-19 NR inter-band CA/DC configurations including inter band CA for 2 different bands DL with up to 2 different bands UL, v0.6.0.

- Text Proposal

----- Start of TP -----

- 5.X  CA_n28-n41

- 5.X.1	Common for 1 band UL and 2 bands UL CA

- 5.X.1.1	Operating bands for CA

Table 5.X.1.1-1: CA band combination CA_n28-n41

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n41 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

- 5.X.1.2	Channel bandwidths per operating band for CA

Table 5.X.1.2-1: Supported bandwidths per CA band combination CA_n28-n41

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n41(2A) | CA_n41(2A) CA_28A-n41A | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 |  |

- 5.X.1.3	UE Co-existence studies

The co-existence studies for UL CA_n41(2A) are studied as in the table 5.81.1.3.1-1:

Table 5.X.1.3.1-1: Intra-band ULCA IMD overlap with the other DL band analysis

| All in MHz | flow | fhigh | BB IMD range |  |  |
| --- | --- | --- | --- | --- | --- |
| n41 fUL | 2496 | 2690 | Order | flow | fhigh |
| n28 fDL | 758 | 803 |  |  |  |
| 2CCBW | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 10 | 190 |  | 10 | 190 |
| Close to UL IMD range |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 20 | 380 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2306 | 2880 |  | 30 | 570 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range |  |  |
|  | 2116 | 3070 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 1926 | 3260 |  | 4802 | 5570 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6(4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 1736 | 3450 |  | 4612 | 5760 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range |  |  |
|  | 1546 | 3640 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 1356 | 3830 |  | 7298 | 8260 |
| Analysis | There are no co-existence issues caused by CA_n41(2A) impact band n28 DL. |  |  |  |  |

- 5.X.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n28-n41 are already defined in the spec TS38.101-1.

- 5.X.1.5	REFSENs requirements

According to the co-existence study, no additional MSD requirements should be defined.

- 5.X.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

- 5.X.2	Specific for 2 bands UL CA

- 5.X.2.1	Maximum output power for inter-band CA

Table 5.X.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n28A-n41A | 23 | +2/-3 |

- 5.X.2.2	UE co-existence studies

In this section, we only focus on the triple beat IMDs caused by UL CA_n28A-n41A. For double beat IMDs caused by CA_n28A-n41A, it was already covered in the co-existence studies of the fallback configurations of DL CA_n28A-n41A with UL CA_n28A-n41A.

For two-band inter-band CA or DC combinations, the precondition is that:

-	Only FDD-FDD and TDD-FDD combination need to be considered with 3 uplink (UL) component carriers (CC) in two clusters within FR1.

-	The 2 UL bands are part of the same band group or belong to adjacent band groups as defined in Table 1.

In the other words, only same band group or adjacent band group TB IMDs issues shall be considered.

For band n28 and n41, band n28 belongs to FR1-1(LB) group while band n41 belongs to FR1-3(HB), which means there is no need to define the additional TB IMDs requirements.

- 5.X.2.3	REFSENS requirements

Based on the co-existence, there is no need to define the additional REFSEN exception requirements.

  - ----- End of TP -----