3GPP TSG-RAN WG4 Meeting #117				                                              R4-2520153

Dallas, USA, November 17th – November 21st, 2025

Title:	Discussion on general parts of RAN4 driven AI use cases

Source:	CATT

Agenda Item:	8.8.1

Document for:	Discussion

1 Introduction

In RAN4#116bis meeting, RAN4 driven AI/ML use cases for 6G radio were discussed and the agreements are captured in the way forward [1]. In this contribution, we will continue to discuss the general parts of RAN4-led AI/ML use cases in 6G radio.

2 Discussion

  - Post-deployment enhancement

In RAN4#116bis meeting, companies proposed to study post-deployment enhancement in 6G study and RAN4 agreed this proposal and some study directions were provided for further discussion:

##### Issue 2-2: Post-deployment enhancement Agreement: - RAN4 will discuss the potential post-deployment testing enhancements scope in 6G [FL Recommended Discussion Point] - For post-deployment enhancement, RAN4 shall further study post-deployment testing and validation enhancements in 6G study: - Study directions: - Solutions for enhanced UE-side AI/ML functionality behaviour/performance reporting - Other directions are not precluded - Companies are encouraged to provide the relevant specification impacts if any.

Actually study on post-deployment was brought up in Rel-18 study phase and after discussions in multiple meetings, two solutions were provided [2]:

RAN4#114: Way forward: Option 1; Option 2. New name; Pre-activation functionality/model update testing; Post deployment management based on LCM. Description; Conduct the validation of a change in AI functionality before its deployment/activation in already deployed UEs • Validation takes into account the UE hardware in which the model is to be deployed/activated.; Using performance monitoring and LCM procedures • Performance monitoring will be designed in other groups • RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.. Sub-options; Possible collection of input data to a model during conformance testing for use later on.. Further potential clarifications (not resolved); The following proposals have been made for option 1 but not discussed. o Whether the testing of a new model is on a device or in a lab. o Whether models that have been updated post conformance testing should be kept in a device. o Whether parallel operating of one model and (in-device) testing of another can be assumed. o Whether signaling from a UE of changes/updates are needed. - Whether there is any relation to GCF procedures. - Whether it is possible to differentiate “major” and “minor” model changes and only apply option 1 to major changes. - Definition of Fine-tuning and Model update - Model update can have a performance impact. - Fine-tuning might have little performance impact. - Hybrid Validation approach - A hybrid approach integrating both pre-activation functionality/model update testing (Option 1) and post-deployment management based on LCM (Option 2) aims to provide a balanced validation strategy.; The following proposals have been made for option 1 but not discussed. - Whether the metric used for performance testing and the metric used for monitoring can be aligned. - Whether monitoring used for post-deployment testing should be NW sided only.. Significant issues; Whether the option is reasonable or would cause too much complexity and test time burden when introducing updates.; Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.) Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs..

And in RAN4#115 meeting, RAN4 agreed Option 2, performance monitoring-based, is prioritized [3]:

RAN4#115: Agreements: Option 2 (Post-deployment post-activation functionality testing based on performance monitoring) is prioritized. - it should be further investigated/established that reliable and testable monitoring procedures can be established for all use cases

Observation 1: RAN4 spent much of time discussing post-deployment testing in R18/R19 and performance monitoring based option was prioritized.

In 6G radio, considering that there might be a possibility of having AI/ML functionality behaves better, we are okay to discuss and study this topic. But before RAN4 start detailed discussion, the issues that should be addressed by post-deployment testing should be clearly identified, and the feasibility/procedures of possible solutions and the impacts of these solutions on spec should also be clarified at the beginning, which helps improve the efficiency of discussion.

Proposal 1: Before starting detailed discussion, the issues that should be addressed by post-deployment testing should be clearly identified.

Proposal 2: The feasibility/procedures of possible solutions and the impacts of these solutions on spec need to be clarified at the beginning.

  - Model standardization and deployment

In last meeting, one proposal is about model standardization and deployment:

RAN4#116bis Proposal: For 6G, further study is needed on model standardization and deployment, RAN4 needs to consider following two aspects: - Effective Model Design, to ensure the reference models developed by RAN4 are practical and feasible for field implementation. - Constraints on Model Deployment, e.g., to study and define what makes a model "complex" or "simple” and understand the power consumption differences between different models.

There are two bullets and the first one is about effective model design which means RAN4 need to ensure the reference models developed by RAN are practical and feasible for field implementation. In general, we are okay with this principle when RAN4 develop standardized AI/ML models. A model that can ensure good balance between AI/ML functionality performance and complexity is more possible to be commercially used. And from spec perspective, a simple AI/ML model that can provide good performance is always preferred. But for reference models, we think they will not restrict the UE implementation and UE vendors can implement any AI/ML models as long as they can meet the RAN4 requirements.

Proposal 3: RAN4 consider balance between AI/ML performance and complexity when developing standardized AI/ML models, which improves the possibility of commercial use of AI/ML models standardized by RAN4.

On the second bullet, it seems not very clear what the spec impacts are and it is more like a UE implementation issue. For example, in order to pass some requirements, UE vendors can choose a simple AI/ML model or implement a very complex one, which leads to different power consumptions, but both AI/ML models can meet RAN4 requirements. But we are okay to further discuss this part with more clarification and inputs.

Observation 2: “Constraints on Model Deployment” is more like a UE implementation issue.

Proposal 4: Provide more clarification and inputs to facilitate the discussion on the constraints on model deployment.

3 Conclusion

In this paper, we provided our views on general parts of RAN4 driven AI/ML use cases in 6G radio, and the following observations and proposals are given:

Post-deployment enhancement

Observation 1: RAN4 spent much of time discussing post-deployment testing in R18/R19 and performance monitoring based option was prioritized.

Proposal 1: Before starting detailed discussion, the issues that should be addressed by post-deployment testing should be clearly identified.

Proposal 2: The feasibility/procedures of possible solutions and the impacts of these solutions on spec need to be clarified at the beginning.

Model standardization and deployment

Proposal 3: RAN4 consider balance between AI/ML performance and complexity when developing standardized AI/ML models, which improves the possibility of commercial use of AI/ML models standardized by RAN4.

Observation 2: “Constraints on Model Deployment” is more like a UE implementation issue.

Proposal 4: Provide more clarification and inputs to facilitate the discussion on the constraints on model deployment.

- References

[1] R4-2514637, WF on 6G AI, Samsung, RAN4#116bis.

[2] R4-2503953, WF on AI/ML air interface study, Ericsson, RAN4#114.

[3] R4-2514636, Topic Summary for [116bis][107] 6G AI, Moderator (Samsung), RAN4#116bis.