3GPP TSG-RAN WG4 Meeting #117	                                                                                            R4-2520162

Dallas, US, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Discussion on future spectrum sharing for 5G NR and 6G

Agenda item:	8.9

Document for:	Approval

- Introduction

In RAN4 #116-bis meeting, the WF [1] for spectrum sharing for 5G NR and 6G has been approved. However, there remain some issues need to be discussed. This paper shares some views on spectrum sharing for 5G NR and 6G.

- Discussion

In RAN4 #116-bis meeting, a WF for spectrum sharing for 5G NR and 6G has been approved, as shown below:

# 2 Open issues - Sub-topic 1-1: General aspects - Issue 1-1-3: Spectrum sharing with NTN - Proposals from companies - P1 (CATT): The MRSS should incorporate NTN network in the scope, including 6G TN + 6G NTN, NR TN + 6G NTN, as well as 6G TN + NR NTN - P2 (Xiaomi): - RAN4 shall study potential solutions and impact on co-channel interference handling and mobility which also related to regulation update including following scenarios: - NTN operation use TN bands/spectrum - 5G NTN and 6G NTN share dedicated NTN spectrum - P3 (ZTE): for MRSS BS, propose to consider the TN BS with 5G-6G TN MRSS in the existing TN MSR specification and NTN SAN with 5G-6G NTN MRSS in the new NTN MSR specification - Way forward - Further discuss - Sub-topic 1-2: Aspects to facilitate MRSS - Issue 1-2-1: Channel rater - Proposals from companies - P1 (Apple): 7.5kHz shifting is not needed for 6G-5G MRSS. - P2 (CMCC): - For MRSS between NR and 6GR, it is proposed to discuss whether 100KHz channel raster for low band are still needed. - No need to consider 7.5KHz uplink shift for MRSS between NR and 6GR. - P3 (Spreadtrum, UNISOC): - For the re-farming bands, 6GR channel raster can be 10 kHz when these bands in NR can support 10 kHz for MRSS between 5G and 6G - There is no need to define UL shift 7.5 kHz for MRSS between 5G and 6G - P4 (OPPO): For the 5G-6G MRSS bands, the 6G channel raster design shall make sure it is compatible with 5G. Besides of that, no other issue is foreseen in MRSS channel raster design - P5 (Xiaomi): Support 5kHz channel raster in day 1 for below 3GHz - Remove per band channel raster concept with following value per sub-frequency range basis Frequency range; Channel raster. <3GHz; 5kHz. 3GHz ~ 24.25kHz; 30kHz. 24.25GHz ~ 52GHz; 120kHz. - Way forward - Discuss following impact - Whether 7.5kHz shifting is needed for 5G-6G MRSS - Whether 100KHz channel raster for low band are needed for 6G-5G MRSS - How to handle the bands in NR can support 10 kHz for MRSS between 5G and 6G - Other impacts are not precluded - Issue 1-2-2: Sync rater - Proposals from companies - P1 (CATT): The new sync raster design for 6G could be leveraged in the design of the multi-RAT spectrum sharing mechanism. - P2 (Xiaomi): RAN4 needs to further evaluate sync raster design for 6GR on 5G migration bands pending on RAN1 progress - P3 (Spreadtrum, UNISOC): Keep a single set of sync raster s in 6G with and without MRSS - Way forward - Further discuss - Issue 1-2-3: Channel bandwidth - Proposals: - P1(Apple): Do not consider MRSS between 6G&5G for small channel bandwidths, e.g., 3MHz (or even 5MHz). - P2(CMCC): it is proposed to consider the impact on MRSS when discussing irregular channel bandwidth. The solution to support irregular channel bandwidth may have impact on MRSS support - Way forward - Discuss whether to have some restriction on supported bandwidth for 5G-6G MRSS - Issue 1-2-4: Waveform - Proposals from companies - Option 1 (CMCC): - it is proposed to consider the impact on MRSS when discuss the waveform for 6GR - RAN1 agreed that the waveforms defined in 5G NR are supported as the basis for 6GR - For other waveform for 6GR, impact on MRSS need to be considered - Way forward - Discuss whether and how the waveform are impacted by MRSS. - Issue 1-2-5: Numerology - Proposals from companies - P1 (CMCC): the numerology discussion for 6GR in legacy band has impact on MRSS. It is proposed to follow RAN1 agreements to take 15KHz SCS for FDD and 30KHz SCS for TDD. - P2 (OPPO): For MRSS operation via FDD sharing, 6G should adopt the same SCS with 5G when sharing carrier/channel - P3 (ZTE): for the MRSS BS, the commercialized NR subcarrier spacing (e.g. FR1 FDD with 15kHz, FR1 TDD with 30kHz and FR2-1 with 120kHz SCS) should be the baseline for 6G BS - P4 (Ericsson): During the discussion on system parameters, take into account that alignment between 5G and 6G slot length and numerology will facilitate MRSS - P5 (Xiaomi): On 5G migration spectrum/bands, harmonize the numerology between 5G and 6G by taking existing NR commercialization deployment choice into account. - Way forward - Further discuss numerology impacted by MRSS - Issue 1-2-6: RF requirements - Proposals from companies - P1(ZTE): for MRSS BS, apply new 6GR BS RF requirements to MRSS BS supporting both 5G and 6G. - P2 (LGE): RAN4 to start RF discussion on the spectrum aggregation with MRSS e.g., 6G PCell(MRSS)+6G SCell - P3 (Ericsson): - When designing UE RF requirements, take into account that 6GR UEs will operate on the same carrier as legacy UEs due to MRSS. - When designing BS RF requirements, take into account that compatibility between 6GR and legacy requirements is needed to facilitate multi-standard BS that can handle both 6GR and legacy RATs - P4 (ISSDU): RAN4 to evaluate 5G/6G RF coexistence in FR1 under MRSS with a quantifiable baseline approach. - Way forward - Further discuss RF requirements impacted by MRSS - Issue 1-2-7: RRM requirements - Proposals from companies - P1 (LGE): RAN4 to start RRM discussion on the MRSS depending on the progress of other WGs. - P2 (Ericsson): RAN4 should study MRSS based measurement and handover requirements if needed, taking into account the RAN1/2 design of MRSS. - P3 (ISSDU): RAN4 to study adaptive RRM coordination for MRSS under partial frequency overlap and different numerology configurations, focusing on dynamic stability and measurable RRM performance. - Way forward - Further discuss RRM requirements impacted by MRSS - Issue 1-2-8: Whether to reuse legacy NR signals/channels for 6GR - Proposals from companies - P1 (Samsung): NR signals/channels (e.g., SSB) should not be reused for 6GR in MRSS. - P2 (CATT): The multi-RAT spectrum sharing should accommodate different synchronization signals design of NR and 6G. - P3 (vivo): For MRSS, from RRM measurement point of view, reusing of legacy NR signal/channels for 6G UE’s measurement purpose is not preferred - P4 (Nokia): based on RAN1 progress on MRSS, RAN4 shall discuss the 6G RRM requirements considering the potential to leverage 5G reference signaling to improve performance. The feasibility and applicable scenarios shall be carefully discussed based on the details of MRSS solution. - Way forward - Further discuss. - Issue 1-2-9: switching time - Proposals from companies - P1(OPPO): For MRSS operation via TDD/dynamic sharing, the switching time between 5G configuration and 6G configuration need study. - Way forward - Further discuss - Issue 1-2-10: Others - Proposals from companies - P1 (Tejas Networks): - Initiate RAN4 studies on RF coexistence for NR-6GR MRSS; define extended ACLR for wide bandwidths, interference models for advanced beamforming such as holographic MIMO, and AI/ML-based adaptive emission control to minimize interference-related performance degradation. - Define RRM procedures for MRSS that enable continuous measurements, support conditional handovers, and allow cross-RAT spectrum aggregation such as NR CA with 6GR secondary cells. Include timing alignment requirements and ensure robust mobility handling even at extremely high speeds, with a strong emphasis on supporting high handover reliability. - Establish MRSS conformance testing guidelines; incorporate radiated tests for 6GR beam interference on NR UEs, AI/ML validation for interference prediction, and scenarios simulating partial 6G rollout in NR-dominant spectrum. - P2 (ISSDU): - Evaluate baseline NR/6GR coexistence in FR1 (e.g., 3.5 GHz), comparing uncoordinated (ρ=1, Δ=0) vs coordinated sharing (ρ<1, Δ>0) - Include EIRP sweeps and deployment conditions in the baseline evaluation, with KPIs such as average spectral efficiency - Use reference curves of uncoordinated vs coordinated sharing as inputs to further RAN4 discussions and potential RAN1 alignment - Extend the study to include partial overlap (ρ<1) with selective or group-based power backoff (Δ>0). - Evaluate static versus periodic adaptation of overlap/guard settings - Include representative cases with 15 kHz (FDD) and 30 kHz (TDD) numerologies in the evaluation - Way forward - Further discuss - Sub-topic 1-3: Inter-RAT mobility between 6GR and NR - Issue 1-3-1: Inter-RAT handover interruption - Proposal from companies - Option 1 (CMCC): it is proposed to minimize handover interruption time for inter-RAT handover between NR and 6GR. - Way forward - Discuss whether and how to minimize handover interruption time for inter-RAT handover between NR and 6GR - Issue 1-3-2: Inter-RAT measurement - Proposal from companies - Option 1 (CMCC): it is proposed to support inter-RAT measurements without gaps, including inter-RAT NR measurement without gap and inter-RAT 6GR measurement without gap, from 6G day-1. - Option 2 (Xiaomi): RAN4 shall study potential inter-RAT RRM measurement impact including w/o and with gap under MRSS scenario. - Way forward - Further discuss

When introducing the Dynamic spectrum sharing (DSS) scheme for 5G NR to share spectrum with LTE, a critical issue arises: LTE will continuously transmit an “always-on” CRS signal with fixed time-frequency resource allocation, while NR PDSCH need to avoid the CRS signal to ensure no collision between two different RATs. This issue catalysed a key solution of the DSS: CRS rate matching. By puncturing REs that are not available for NR PDSCH, NR UE will be scheduled to avoid the REs used by LTE CRS.

However, in the 6G era, we are fortunate to no longer face this issue, as NR does not have CRS-like signals that occupy the entire spectrum and always on. In contrast, spectrum sharing for 5G/6G will present some new “sweet troubles”. Given the significantly greater flexibility of the NR scheduling compared to LTE, it is foreseeable that spectrum sharing between 5G/6G RATs will resolve the majority of collisions by scheduling.

Therefore, the first priority is to specify the scenario for 5G/6G MRSS. This will help determine the direction for subsequent cooperation between different RATs and prove beneficial for identifying potential solutions.

Observation 1: RAN4 should specify the scenario of 5G/6G MRSS.

From our understanding, it is necessary to clarify whether the spectrum sharing between 5G/6G is tightly coupled. It is foreseeable that spectrum sharing is more likely to be deployed by same operator or co-operators (other operators cannot use certain license spectrum). Furthermore, it is necessary to explore whether MRSS is only applicable to co-location BS or can also be applied to non-co-location BS. When MRSS is deployed in a co-located scenario, the technical solution could be relatively clean and simple. This is because under co-location deployment, 5G and 6G RATs could minimise collisions and maximise spectrum efficiency through a tightly coordinated scheduled mechanism.

However, the non co-coloted scenario should also be considered, which will enhance the flexibility of 6G deployment. Non co-located scenario also introduces new challenges, such as RAT may conduct some unexpected scheduling behaviours, which requires additional design to address potential collisions caused by such unexpected scheduling.

In summary, MRSS requires network side monitoring and precise scheduling, whilst affected UEs need to avoid the occupied time-frequency resources by other RATs. Consequently, specific network side design considerations should be implemented for MRSS to ensure its proper functioning.

Proposal 1: The multi-RAT spectrum sharing should be discussed from network perspective and intra operator, both co-located and non co-located scenario should be considered.

In addition, based on current discussions and relatively common understanding, 6G will be deployed directly by Standalone (SA) mode, but the access resources for 6G RAT might be significantly different from 5G RAT. For instance, from a frequency domain perspective, the spectrum allocations under MRSS deployment for 5G/6G may partially or fully overlap. Moreover, 6G RAT may adopt entirely new synchronization signals designed distinct from 5G SSB. Given that synchronization signals transmission occupies fixed time-frequency resources and is periodic, a robust mechanism should be specified during MRSS design to prevent potential collisions.

Proposal 2: The multi-RAT spectrum sharing should accommodate different synchronization signals design of NR and 6G.

Considering that the NR SSB design has been adopted by millions of existing devices, modifying the NR design might be challengeable. Therefore, the transmission of 6G synchronization signal need to avoid the time-frequency resources occupied by NR SSB during spectrum sharing. As the frequency point of NR SSB transmission usually aligned with the sync raster, the potential frequency point of NR SSB is predictable from a frequency domain perspective. This issue can be resolved in a “tricky” manner: by configuring some offset for 6G sync raster from NR sync raster. Thereafter, the 6G synchronization signal could avoid NR SSB transmission naturally.

However, the specific offset for 6G sync raster might be difficult to derive. For NR sync raster, the interval of sync raster staring from 1200 kHz, and progressively increases with higher frequency (1.44MHz or more). Considering that the NR SSB will occupy 20RBs (approximately 5MHz), if the sync raster design of 6G is similar to NR design, such simple offset will still be unable to accurately avoid NR SSB transmissions. One possible solution is to substantially increase the interval of the 6G sync raster (or reduce it), which may help to stagger the sync raster. Anyway, since the synchronization signal of 6G will be different from NR SSB, a new design of sync raster could also be considered.

Proposal 3: The new sync raster design for 6G could be leveraged in the design of the multi-RAT spectrum sharing mechanism.

For MRSS channel raster design, given that support for 100 KHz channel raster is primarily intended to achieve LTE compatibility, this issue does not cause a significant challenge for NR. Therefore, RAN4 may consider investigating whether MRSS only supports configurations with 10 kHz channel raster.

Proposal 4: RAN4 may consider investigating whether the MRSS between NR and 6G only support 10 kHz channel raster configuration.

From MRSS numerology perspective, the previous meeting made some discussion and some numerology assumptions may supported by MRSS. Overall, given RAN1's strong opinion on a ‘single numerology’ approach during 6G design, we believed that it reasonable to take 15kHz SCS for FDD and 30kHz SCS for TDD as an assumption. However, a risk that cannot be overlooked is that for certain legacy NR frequency bands, the network may conduct some configuration operations that do not align with this assumption. For instance, FDD operation in the n7 band often configures the SCS at 30 kHz. Under this assumption, this situation may restrict the MRSS operation at n7 band.

Proposal 5: RAN4 may take 15 KHz SCS for FDD / 30 KHz SCS for TDD as an assumption. However, this assumption may impede MRSS operation in certain legacy NR band such as n7.

For MRSS BS RF requirements, it may be necessary to specify whether NR and 6G BS will define the same (or highly similar) set of RF requirements. Given that the potential evolution path for 6G BS technology remains unclear, the introduction of certain technologies significantly diverging from NR design could potentially introduce entirely different RF requirements (such as geometric shaping). It is obviously that such “brand new” 6G BS RF requirements might be totally not applicable to NR BS. Therefore, we believed that a more feasible approach at this stage would be: for MRSS BS, any additional BS RF requirements arising from MRSS support should apply the 6G BS RF requirements.

Proposal 6: For MRSS BS, any additional BS RF requirements arising from MRSS support should be subject to the 6G BS RF requirements.

In addition, with the further integration of TN and NTN networks, the MRSS should also incorporate NTN networks into its considerations. Unfortunately, given that TN and NTN network are not integrated within NR, they can, to a certain extent, be regarded as two independent legacy networks. Therefore, the MRSS incorporating NTN network should encompass two different, but complementary deployment scenarios: NR TN + 6G NTN, 6G TN + NR NTN and NR NTN + 6G NTN.

Proposal 7: The MRSS should incorporate NTN network in the scope, including NR TN + 6G NTN, 6G TN + NR NTN and NR NTN + 6G NTN.

- Conclusion

This contribution provides our views on future spectrum sharing for 5G NR and 6G. The following proposals are concluded as follows:

Observation 1: RAN4 should specify the scenario of 5G/6G MRSS.

Proposal 1: The multi-RAT spectrum sharing should be discussed from network perspective and intra operator, both co-located and non co-located scenario should be considered.

Proposal 2: The multi-RAT spectrum sharing should accommodate different synchronization signals design of NR and 6G.

Proposal 3: The new sync raster design for 6G could be leveraged in the design of the multi-RAT spectrum sharing mechanism.

Proposal 4: RAN4 may consider investigating whether the MRSS between NR and 6G only support 10 kHz channel raster configuration.

Proposal 5: RAN4 may take 15 KHz SCS for FDD / 30 KHz SCS for TDD as an assumption. However, this assumption may impede MRSS operation in certain legacy NR band such as n7.

Proposal 6: For MRSS BS, any additional BS RF requirements arising from MRSS support should be subject to the 6G BS RF requirements.

Proposal 7: The MRSS should incorporate NTN network in the scope, including NR TN + 6G NTN, 6G TN + NR NTN and NR NTN + 6G NTN.

- References

- R4-2514646, WF for [116bis][108] 6G spectrum sharing, CMCC, RAN4 #116-bis