3GPP TSG RAN WG4 Meeting #117    		                          R4-2520227

Dallas, US, 17th Nov – 21st Nov 2025

Agenda item:		7.6.4.2

Source:			Samsung

Title:			Discussion on HD-FDD for Ku band with FR1 numerology

Document for:		Discussion and decision

Introduction

In the RAN4#116bis, we have already discussed the RRM requirements for HD-FDD for Ku band with FR1 numerology and latest agreements are included in [1]. There are still some open issues in [1]. In this contribution, we continue to discuss the FFS parts of RRM requirements for HD-FDD VSAT for Ku band with FR1 numerology.

- Discussion

In the RAN4#116bias meeting, the potential issues for further discussion are captured in WF [1] as below:

Issue 2-1-3: Handover interruption time - Option 1: HD-FDD has no impact on NR handover. - Option 2: FFS impact due to reduced Rx time - Further discuss the following conditions for HD-FDD - For RACH-based HO and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added: - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added: - One SSB is available during T∆ - One SSB is available during TIU

In previous meetings, RAN4 have already analyzed the impact on NR handover due to HD-FDD and also discussed the conditions for the further discussion part. The final conclusion is that HD-FDD has no spec impact on NR handover. Considering that HD-FDD VSAT with FR1-NTN does not introduce any special characteristics for handover, we support option 1 that the existing requirements for Handover for SAN can also be applied for HD-FDD VSAT with FR1-NTN.

Proposal 1: RAN4 to reuse the conclusion that the existing requirements for Handover for SAN can also be applied for HD-FDD VSAT with FR1-NTN.

Issue 2-1-5: UE UL timing accuracy - Further discuss the following options - Option 1: HD-FDD has no impact on UE UL timing accuracy. - Option 2: FFS impact due to Tx/Rx pattern.

The current RRM spec has already covered the requirements for VSATs equipped with electronically steered antennas (including VSAT types 2, 3, 5, 6) without HD-FDD and has analyzed the UL timing accuracy requirements for RedCap UEs with FR1-NTN, without separately listing the HD-FDD requirements for UL timing accuracy. Based on our understanding, we support that the existing requirements for UE timer accuracy for satellite access can also be applied for HD-FDD VSAT with FR1-NTN.

HD-FDD has no impact on UE UL timing accuracy.

Proposal 2: For the impact on UE UL timing accuracy for HD-FDD, we support that the existing requirements for UE timer accuracy for satellite access can also be applied for HD-FDD VSAT with FR1-NTN.

Issue 2-1-2/4/6/7: other RRM requirement impact due to HD-FDD - RAN4 to further discuss if any RRM requirement impact due to HD-FDD for the following aspects - Maximum interruption in paging reception - initial access/random access - inter and/or intra-measurement requirement - scheduling availability

For maximum interruption in paging reception, RAN4 has already defined that FDD and HD-FDD RedCap UEs reuse the application requirements specified for NTN. For our understanding, HD-FDD VSAT UEs can also reuse the application requirements for NTN.

UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. The FDD and HD-FDD RedCap UE shall meet all applicable requirements specified in clause 4.2C.5, except that - ‘Tdetect,NR_Intra’ is replaced with ‘Tdetect,NR_Intra_RedCap’ , and - ‘Tdetect,NR_Inter’ is replaced with Tdetect,NR_Inter_RedCap , and - clause 4.2C.2.3 is replaced with 4.2E.2.3, and - clause 4.2C.2.4 is replaced with 4.2E.2.4. The 1 Rx RedCap in HD-FDD shall meet all applicable requirements specified in clause 4.2C.2.5 under the following conditions - at least 1 SSB is available at the UE in the serving cell during the last 160 ms duration.

RAN4 has already analyzed the impact of HD-FDD on intra/inter measurements and scheduling availability. We prefer to reuse the current agreements. The agreements are as follows [2][3]:

#### Issue 1-3-1: The impact of HD-FDD on scheduling availability of UE related to CSI-RS and PRS Background Online agreement Agreement: - When UE indicates CollisionHandlingOfHDFDDOperation, RAN4 to clarify that measurement requirements for CSI-RS, scheduling restriction of PDSCH, and PRACH apply provided that no collision between CSI-RS/PDSCH and PRACH #### Issue 1-3-4: The impact of HD-FDD on intra/inter measurements online agreement Agreement: - For scheduling availability of UE performing intra/inter measurements, the existing principle can be reused, regardless of the network indication.

Proposal 3: RAN4 to reuse the previously principles of RRM requirement impact due to HD-FDD for the following aspects:

- Maximum interruption in paging reception

- initial access/random access

- inter and/or intra-measurement requirement

- scheduling availability

Issue 2-1-9: UE capability - RAN4 to assess the need for new UE capabilities for HD-FDD VSAT UE with NTN.

In R19, RAN4 defined a new UE capability for (e)RedCap UE with NTN to explicitly declare that only Rel-19 UEs need to meet the requirements for (e)RedCap UE to NTN-FR1 from Rel-19, while Rel-17/18 UEs do not need to satisfy these requirements. If no new capability is defined, from the network perspective, when there are Rel-17/Rel-18 UE and Rel-19 UE in the same network, the network cannot distinguish between them. Considering that the requirements for HD-FDD VSAT UE with NTN have not yet been clarified, it is too early to discuss whether to add a new capability for HD-FDD VSAT UE with NTN. Therefore, we propose to defer the discussion of this open issue until the requirements for HD-FDD VSAT UE with NTN are clarified.

Observation 1: Due to the unclear requirements for HD-FDD VSAT UE with NTN, it is too early to assess whether a new UE capability is needed. Therefore, we propose that RAN4 to defer the discussion of this open issue until the requirements for HD-FDD VSAT UE with NTN are clarified.

Conclusion

This contribution discusses the impact of HD-FDD for FR1-NTN core part operation on RAN4 RRM. The following are the proposed points in this contribution.

Proposal 1: RAN4 to reuse the conclusion that the existing requirements for Handover for SAN can also be applied for HD-FDD VSAT with FR1-NTN.

Proposal 2: For the impact on UE UL timing accuracy for HD-FDD, we support that the existing requirements for UE timer accuracy for satellite access can also be applied for HD-FDD VSAT with FR1-NTN.

Proposal 3: RAN4 to reuse the previously principles of RRM requirement impact due to HD-FDD for the following aspects:

- Maximum interruption in paging reception

- initial access/random access

- inter and/or intra-measurement requirement

- scheduling availability

Observation 1: Due to the unclear requirements for HD-FDD VSAT UE with NTN, it is too early to assess whether a new UE capability is needed. Therefore, we propose that RAN4 to defer the discussion of this open issue until the requirements for HD-FDD VSAT UE with NTN are clarified.

Reference

[1] R4-2515082, Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2

[2] R4-2508287, WF on RRM requirements for NR_NTN_Ph3_Part1

[3] R4-2512146, WF on RRM requirements for NR_NTN_Ph3_Part1