3GPP TSG-RAN WG4 # 117	R4-2520235

Dallas, USA, 17th November 2025 – 21st November 2025

Agenda item:	6.11.8

Source:	MediaTek Inc.

Title:	Discussion on CSI reporting requirements for CSI prediction

Document for:	Discussion

# Introduction

In RAN4#116bis, RAN4 agreed to continue discussion on CSI reporting requirements for CSI prediction issues [1]. In RAN4#116bis, updated simulation assumptions for AI/ML CSI prediction were agreed [2]. In this contribution, we provide our views and simulation results on CSI reporting requirement framework for CSI prediction.

# Discussion

In this Chapter, we provide our views on CSI prediction requirement issues and share simulation results.

The way forward on simulation assumptions for CSI prediction was agreed at the previous meeting [2]. Table 1 lists the simulation parameters used. For Step-1 and Step-2, we have limited our simulation scope to FDD with a 10 MHz bandwidth and a 15 kHz SCS. We provide simulation results for all previously used Doppler frequencies (20 Hz, 50 Hz, and 100 Hz) and both agreed TX configurations: 32TX with (N1, N2) = (4, 4), and 16TX with (N1, N2) = (4, 2). The Step‑1 and Step‑2 datasets were generated with a link‑level simulator using ideal channel estimation.

For Step‑3, we provide simulation results for both FDD and TDD using the agreed Doppler frequency (20 Hz) and TX configuration (16TX with (N1, N2) = (4, 2)). The Step‑3 dataset was generated with a link‑level simulator using practical channel estimation. The training and testing dataset consists of channel realizations only for the agreed Doppler frequency. The Step‑3 dataset includes a range of SNRs, and training is performed across the full SNR range.

Companies were encouraged to report the model complexity of their AI/ML CSI prediction models to facilitate alignment. For this evaluation, we used models with fewer than 10,000 parameters and requiring fewer than 2 MFLOPs per inference for both the 16TX and 32TX configurations.

Table 1: Simulation parameters for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD, 4GHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD, 14 subbands, 8RB as a bundle for TDD |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| gNB TX antennas | 16 (N1, N2) = (4, 2) (Step-1 to Step-3) 32 (N1, N2) = (4, 4) (Step-1, Step-2) |
| UE RX antennas | 2RX for FDD 4RX for TDD |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Beam steering | Beam steering approach with dual cluster beams (TS38.101-4 Annex B.2.3.2.3A) |
| Doppler spread | 20Hz |
| Delay spread | 30ns |
| Channel estimation | Ideal DL channel estimation (Step-1, Step-2) / Practical channel estimation (Step-3) |
| Rank per UE | Rank 1 and 2 |
| CSI feedback assumption | Check the CSI prediction performance of raw channel per layer Check the CSI prediction performance with Rel-18 codebook quantization |
| KPI for Step-1 | Average of SGCS for intermediate results over all subbands per layer SGCS1: Compares the SVD of model output (before feedback) with the SVD of the ground truth Raw channel |
| KPI for Step-2 | Average of SGCS for intermediate results over all subbands per layer SGCS 2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD SGCS 3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth |
| KPI for Step-3 | Throughput γ1: AI/ML based CSI prediction is compared with Rel-15 Type I random PMI γ2: AI/ML based CSI prediction is compared with Rel-16 eType II CB(sample-and-hold) Channel prediction SGCS1 and SGCS3 Sample-and-hold SGCS3 |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training and validation datasets for Step-1 and Step-2 | Set1: 16TX containing 20Hz Doppler ideal estimates Set2: 16TX containing 50Hz Doppler ideal estimates Set3: 16TX containing 100Hz Doppler ideal estimates Set4: 16TX containing 20Hz, 50Hz, and 100Hz Doppler ideal estimates (mixed) Set5: 32TX containing 20Hz Doppler ideal estimates Set6: 32TX containing 50Hz Doppler ideal estimates Set7: 32TX containing 100Hz Doppler ideal estimates Set8: 32TX containing 20Hz, 50Hz, and 100Hz Doppler ideal estimates (mixed) |
| Testing datasets for Step-1 and Step-2 | Set1: 16TX containing only 20Hz Doppler ideal estimates Set2: 16TX containing only 50Hz Doppler ideal estimates Set3: 16TX containing only 100Hz Doppler ideal estimates Set4: 32TX containing only 20Hz Doppler ideal estimates Set5: 32TX containing only 50Hz Doppler ideal estimates Set6: 32TX containing only 100Hz Doppler ideal estimates |
| Training and validation datasets for Step-3 | Set1: 16TX containing only 20Hz Doppler practical estimates |
| Testing datasets for Step-3 | Set1: 16TX containing only 20Hz Doppler practical estimates |
| MCS for Step-3 | MCS-17, MCS-19 |

In the following Table 2, we share our Step-1 and Step-2 average of SGCS for intermediate results over all subbands per layer for 4 different measures. Firstly, we have non-predicted (sample-and-hold) raw channel SGCS. Secondly, we have predicted raw channel SGCS. Thirdly, we have predicted Rel-18 quantized precoder SGCS2 where comparison is done to ground truth of unquantized SVD precoder. Finally, we have predicted Rel-18 quantized precoder SGCS3 where comparison is done to ground truth Rel-18 quantized precoder. Note, that Step-1 and Step-2 SGCS results have not changed compared to our previous discussion papers [3][4].

Table 2: Step-1 and Step-2 results of average of SGCS over all subbands per layer

| Test | Training Doppler [Hz] | Testing Doppler [Hz] | Channel sample-and-hold [SGCS per layer] | Channel prediction [SGCS per layer] | Rel-18 precoder prediction SGCS2 [SGCS per layer] | Rel-18 precoder prediction SGCS3 [SGCS per layer] |
| --- | --- | --- | --- | --- | --- | --- |
| FDD 16TX | 20 | 20 | [0.795, 0.650] | [0.999, 0.999] | [0.907, 0.825] | [0.992, 0.985] |
|  | 50 | 50 | [0.325, 0.187] | [0.969, 0.944] | [0.892, 0.801] | [0.955, 0.922] |
|  | 100 | 100 | [0.301, 0.166] | [0.750, 0.572] | [0.774, 0.597] | [0.821, 0.675] |
|  | Mixed (20, 50, 100) | 20 | [0.795, 0.650] | [0.957, 0.923] | [0.879, 0.778] | [0.941, 0.898] |
|  |  | 50 | [0.325, 0.187] | [0.872, 0.777] | [0.822, 0.688] | [0.873, 0.781] |
|  |  | 100 | [0.301, 0.166] | [0.666, 0.456] | [0.677, 0.471] | [0.717, 0.533] |
| FDD 32TX | 20 | 20 | [0.814, 0.670] | [0.999, 0.998] | [0.869, 0.735] | [0.990, 0.982] |
|  | 50 | 50 | [0.391, 0.201] | [0.972, 0.946] | [0.853, 0.708] | [0.961, 0.924] |
|  | 100 | 100 | [0.309, 0.160] | [0.743, 0.561] | [0.774, 0.597] | [0.836, 0.690] |
|  | Mixed (20, 50, 100) | 20 | [0.814, 0.670] | [0.954, 0.911] | [0.844, 0.693] | [0.944, 0.894] |
|  |  | 50 | [0.391, 0.201] | [0.881, 0.770] | [0.797, 0.611] | [0.891, 0.787] |
|  |  | 100 | [0.309, 0.160] | [0.664, 0.444] | [0.673, 0.438] | [0.749, 0.563] |

From these results we can make the following observations.

Observation #1: FDD 16TX and FDD 32TX results with the same Doppler perform quite similarly.

Observation #2: In lower Doppler, accuracy is better than in higher Doppler.

Observation #3: Rel-18 codebook quantization performance is close to raw channel prediction performance.

In the following Table 3, we present SGCS1 and SGCS3 metrics for channel prediction and the SGCS3 metric for sample-and-hold channel, all measured at the SNR closest to the 90% of maximum throughput point of channel prediction simulation. We observe channel prediction gains over sample-and-hold in all tests.

Observation #4: In Step‑3, channel prediction shows gains over sample-and-hold in all tests when evaluated using SGCS.

Table 3: Step-3 results of average SGCS over all subbands per layer (SNR at 90% throughput, prediction window 1/5ms/5ms, TDLA30)

| Test | Training Doppler [Hz] | Testing Doppler [Hz] | Channel prediction SGCS1 [SGCS per layer] | Channel prediction SGCS3 [SGCS per layer] | Channel sample-and-hold SGCS3 [SGCS per layer] |
| --- | --- | --- | --- | --- | --- |
| FDD 16TX MCS-17 | 20 | 20 | [0.913, 0.863] | [0.902, 0.861] | [0.730, 0.610] |
| FDD 16TX MCS-19 | 20 | 20 | [0.932, 0.894] | [0.912, 0.876] | [0.734, 0.617] |
| TDD 16TX MCS-17 | 20 | 20 | [0.922, 0.865] | [0.876, 0.760] | [0.702, 0.520] |
| TDD 16TX MCS-19 | 20 | 20 | [0.931, 0.880] | [0.881, 0.771] | [0.704, 0.525] |

In Figures 1 and 2, and Table 4, we share our PDSCH Rank 2 throughput simulation results using various MCS and maximum Doppler 20Hz. We have simulated MCS-17 and MCS-19 to find suitable and feasible operation point for testing.

We report the SNR required to achieve 90% relative throughput for both predicted PMI and sample‑and‑hold (legacy) PMI. We also report the throughput ratio γ1 between predicted PMI and random PMI; this ratio would serve as the PMI reporting requirement for the predicted PMI. In addition, we report the throughput ratio γ2 between predicted PMI and sample-and-hold PMI to quantify the prediction gain. Finally, for reference, we report the throughput ratio γ between sample-and-hold PMI and random PMI.

Figure 1: Step-3 throughput results of PDSCH Rank2 MCS-17 in FDD and TDD with Doppler 20Hz.

Figure 2: Step-3 throughput results of PDSCH Rank2 MCS-19 in FDD and TDD with Doppler 20Hz.

Table 4: Step-3 PDSCH Rank 2 results collection of SNR, γ1 and γ2. Prediction results are given for prediction window 1/5ms/5ms.

| Duplex mode | MCS | Prediction SNR [dB] | Sample&Hold SNR [dB] | Prediction γ1 (vs. random PMI) | Prediction γ2 (vs. s&h) | Sample&Hold γ (vs. random PMI) |
| --- | --- | --- | --- | --- | --- | --- |
| FDD | 17 | 7.4 | 11.7 | 3.6 | 1.3 | 2.0 |
|  | 19 | 9.1 | 13.5 | 3.2 | 1.3 | 1.9 |
| TDD | 17 | 3.6 | 5.3 | 3.5 | 1.2 | 2.5 |
|  | 19 | 5.3 | 7.0 | 3.0 | 1.3 | 2.4 |

In Figures 1 and 2, and Table 4, we present our PDSCH Rank 2 throughput simulation results for the prediction window 1/5ms/5ms, as defined in the simulation assumptions, where we predict for the next CSI‑RS occasion.

From presented results we can make the following observations.

Observation #5: Prediction gain measured in SNR varies between 4.3 and 4.4 dB in FDD.

Observation #6: Prediction gain measured in SNR is 1.7 dB in TDD.

Observation #7: Prediction gain measured in γ1 varies between 3.0 and 3.6 in FDD/TDD

Observation #8: Prediction gain measured in γ2 varies between 1.2 and 1.3.

Issue 1-7: Doppler and MCS Agreement: Introduce tests for 20Hz Doppler with MCS [17 or 19] with 16Tx ports and 2Rx in FDD. Introduce test for TDD with the details FFS

We think that both MCS-17 and MCS-19 can ensure robust SNR conditions during testing in both FDD and TDD based on our simulation results. During the meeting we need to further check simulation result alignment to conclude if it is feasible to define requirements. In the last meeting, 3 companies had quite aligned results for FDD.

Observation #9: Both MCS-17 and MCS-19 have feasible SNR operation point in both FDD and TDD.

Proposal #1: During the meeting check if simulation results are aligned in both FDD and TDD.

Proposal #2: Introduce 4RX TDD requirements if there are enough aligned simulation results.

Issue 1-8: Generalization Agreement: Test setup for the generalization will be further discussed based on the following options. - Option 1: Introduce multiple tests with different MCS, which will lead to different SNR points. - Option 2: Introduce multiple tests with different channel models. The details of the channel models are FFS. - Option 3: the combination of option 1 and 2. It is FFS on how to quantify the generalization performance.

With existing simulation assumptions, we could maximize SNR difference between FDD and TDD tests by selecting MCS-19 for FDD 2RX requirements and MCS-17 for TDD 4RX requirements.

Proposal #3: Use MCS-19 for FDD 2RX requirements.

Proposal #4: Use MCS-17 for TDD 4RX requirements if defined.

Due to strong time pressure to complete the work item on schedule, we consider it too ambitious to include other channel models for generalization.

Proposal #5: Do not consider different channel models for generalization.

# Conclusion

In this contribution we provide the view on the CSI reporting requirement framework for CSI prediction issues. The following observations and proposals are made:

Observation #1: FDD 16TX and FDD 32TX results with the same Doppler perform quite similarly.

Observation #2: In lower Doppler, accuracy is better than in higher Doppler.

Observation #3: Rel-18 codebook quantization performance is close to raw channel prediction performance.

Observation #4: In Step-3, channel prediction shows gains over sample-and-hold in all tests when evaluated using SGCS.

Observation #5: Prediction gain measured in SNR varies between 4.3 and 4.4 dB in FDD.

Observation #6: Prediction gain measured in SNR is 1.7 dB in TDD.

Observation #7: Prediction gain measured in γ1 varies between 3.0 and 3.6 in FDD/TDD

Observation #8: Prediction gain measured in γ2 varies between 1.2 and 1.3.

Observation #9: Both MCS-17 and MCS-19 have feasible SNR operation point in both FDD and TDD.

Proposal #1: During the meeting check if simulation results are aligned in both FDD and TDD.

Proposal #2: Introduce 4RX TDD requirements if there are enough aligned simulation results.

Proposal #3: Use MCS-19 for FDD 2RX requirements.

Proposal #4: Use MCS-17 for TDD 4RX requirements if defined.

Proposal #5: Not to consider different channel models for generalization.

# Reference

- R4-2514631, WF on [116bis][112] R19 AI for air interface, Qualcomm inc.

- R4-2514634, WF on updated simulation assumptions for AI based CSI prediction, OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, CMCC, CTC, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple

- R4-2509655, Discussion on CSI reporting requirement framework for CSI prediction, MediaTek inc.

- R4-2513766, Discussion on CSI reporting requirements for CSI prediction, MediaTek inc.

- 3GPP TS 38.214, Physical layer procedures for data (Release 19), V19.1.0 (2025-09)