3GPP TSG-RAN WG4 Meeting #117	R4-2520253

Dallas, USA, Nov. 17-21, 2025

Title: 	On remaining UE RF issues for NR_LBCA_Sw_enh

Source: 	Huawei, HiSilicon

Agenda item:	7.8.3

Document for:	Approval

# Introduction

RAN4#116bis approved an WF of [1], where several items remain unsolved. This contribution shares our views on some of the remaining issues. Note that our view on BS-to-BS as well as UE-to-UE co-existence common to Rel-19 and Rel-20 is summarized in [2].

# Discussion

## General

Although we share our views on some of the remaining items listed in the approved WF of [1], we should wait for the conclusion of BS-to-BS co-existence issue which may or may not impact on UE RF requirements. In addition, the conclusion may impact on RAN1 design if any as well as relationship and/or fallback behavior b/w Rel-19 & Rel-20, given that it may be affected by RAN1 design.

## Diversity SDL+FDD DL combined filter feasibility

Our view on diversity SDL+FDD DL combined filter feasibility for CA_n28-n67 is summarized as below.

Regarding diversity SDL+FDD DL combined filter feasibility for CA_n28-n67,

- Infeasible: Full band filter (738-803 MHz)

- Feasible: A portion of n28 DL + n67 (738-788 MHz)

## Time mask

With respect to time mask, there are following bullets in the WF [1], and we divide them into two groups. One is top two sub-bullets colored in yellow and the other is the last sub-bullet colored in aqua.

For top two sub-bullets (yellow)

In our view, it is premature to discuss whether or what additional clarifications are needed without knowing RAN1 impact due to the introduction of dualDL (Case 3)

For the last sub-bullets (aqua)

In our view, the raised issue must not be specific to NR_LBCA_Sw_enh, but rather it must be applicable to NR_LBCA_Sw (Rel-19) and other features associated with switching like uplink Tx switching.

Since network must know whether or not TA for a UE is large and the UE’s switching period via its UE capability signaling, the network can provide the UE sufficient switching gap (sufficient symbols).

## Co-existence issue

Our understanding is that BS-to-BS co-existence is common to Rel-19 and Rel-20 low band CA carrier switching features, while UE-to-UE co-existence may be a bit different between Rel-19 and Rel-20 specifically in terms of blocking.

When a UE supporting Rel-20 Low band CA carrier switching is under dualDL operation, the UE is receiving both PCell and SCell signals.

The virtue of the dualDL (Case 3) for Rel-20 is that it allows the UE to keep obtaining PCell DL signals regardless of Case 1 (FDD) or Case 3 operation. This is a big difference from Rel-19 which doesn’t allow UE to receive PCell DL when the UE is under Case 2 (SDL) operation.

When the UE is under dualDL operation and is affected by blocking due to e.g., a UE close to the victim UE, both FDD DL (PCell) and SDL (SCell) on dualDL may be impacted. Ideally, if there is no blocking issue in the field (and also no-other dualDL specific co-existence issue), dualDL may give more robust gain than Rel-19 Low band CA carrier switching. If RAN1 design is established assuming that PCell is always available in each case 1 and case 3 operation, while if the UE is often affected by blocking and PCell during case 3 operation is not available more than expected, the net gain may be what we expect.

## Relationship and/or fallback behavior b/w Rel-19 & Rel-20

From UE RF perspective, it would be possible for UE supporting Rel-20 NR_LBCA_Sw_enh to support Rel-19 NR_LBCA_Sw. It’s noted that actual UE’s performance for the UE, e.g., REFSENS etc. during Rel-19 NR_LBCA_Sw operation, may be different from a UE supporting only Rel-19 NR_LBCA_Sw due to Rx filter design difference.

At this moment, however, it is not clear if Case 3 (dualDL) has impact on RAN1 specification.

# Conclusions

In this paper we provided our views on R20 LB-LB SW.

Proposal 1: Discuss the actual detailed UE RF requirements, RAN1 impact and relationship and/or fallback behavior b/w Rel-19 & Rel-20 after obtaining the conclusion for BS-to-BS and UE-to-UE co-existence issue which may or may not impact on UE RF requirements.

Observation 1: Regarding diversity SDL+FDD DL combined filter feasibility for CA_n28-n67,

- Infeasible: Full band filter (738-803 MHz)

- Feasible: A portion of n28 DL + n67 (738-788 MHz)

Observation 2: A following case is not specific to NR_LBCA_Sw_enh, but rather applicable to NR_LBCA_Sw (Rel-19) and other features associated with switching like uplink Tx switching

- Case: TA is very large while an UL transient period preceding the first UL symbol after the switch does not include the switching period

Observation 3: Network can know whether TA for a UE is large or not and the UE’s switching period via its UE capability so that the network can provide the UE with sufficient switching gap (symbols).

Proposal 2: No more discussion is needed for the case in Observation 3 under this WI.

Observation 4: BS-to-BS co-existence is common to Rel-19 and Rel-20 low band CA carrier switching features, while UE-to-UE co-existence may be a bit different between Rel-19 and Rel-20 specifically in terms of blocking.

Observation 5: The virtue of the dualDL (Case 3) for Rel-20 is that it allows the UE to keep obtaining PCell DL signals regardless of Case 1 (FDD) or Case 3 operation. This is a big difference from Rel-19 which doesn’t allow UE to receive PCell DL when the UE is under Case 2 (SDL) operation.

Observation 6: When a UE is under dualDL operation and is affected by blocking, FDD DL (PCell) and/or SDL (SCell) may be impacted. If RAN1 design is established (if needed) assuming that PCell is always available in each case 1 and case 3 operation, while if the UE is often affected by blocking and PCell during case 3 operation is not available more than expected, the net gain may be what we expect.

Observation 7: From UE RF perspective, it would be possible for UE supporting Rel-20 NR_LBCA_Sw_enh to support also Rel-19 NR_LBCA_Sw if there is no RAN1 impact due to the introduction of case 3 (dualDL).

Proposal 3: Discuss relationship and/or fallback behavior between Rel-19 NR_LBCA_Sw and Rel-20 NR_LBCA_Sw_enh after all the remaining issues are resolved.

# Reference

- R4-2515156, Way Forward for [116bis][320] NR_low_band_CA_switching, Moderator (Apple)

- RP-252694, Revised WID on Low NR band carrier aggregation via switching, Telus

- RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Moderator (Huawei)