# 3GPP TSG-RAN WG4 Meeting #117                            R4-2520453

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	7.12.2

Source: 	CMCC

Title: 			 Discussion on general aspects for AI/ML for mobility

Document for:	Discussion

- Introduction

In RAN #108 meeting, the new WID on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR was approved [1]. In RAN #109 meeting, the revised WID on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR was approved [2]. The objectives for RAN4 are duplicated as following.

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any

In last meeting, there is initial discussion on Artificial Intelligence (AI)/Machine Learning (ML) for mobility, and a WF was approved [3]. This contribution provides discussion on general aspects.

- Discussion

- Issue 1-2-2: Scenarios and sub-use cases Online agreements on Tuesday: Agreement: RAN4 to discuss the potential RRM requirements for cell-level based prediction for the following scenarios: Evaluation scenario; Methodology. FR1 to FR1 intra-frequency temporal domain case A; intra-cell. FR1 to FR1 intra-frequency temporal domain case B; Intra-cell. FR1 to FR1 inter-frequency (frequency domain) (co-located case); Inter-cell. FR2 to FR2 intra-frequency temporal domain case A; Intra-cell. FR2 to FR2 intra-frequency temporal domain case B; Intra-cell. FFS whether to discuss the requirements for the following scenario: Evaluation scenario; Methodology. FR2 to FR2 inter-frequency (frequency domain) (co-located case); Inter-cell. Agreement: - As starting point, evaluate sub-case 2, L3-L3 measurement prediction as baseline for both FR1 and FR2. - Interested companies can also provide results for other sub-cases. - It is not precluded to add additional sub-case for FR2 in future meetings. - Further discuss how to define requirements based on the available simulation results.

For AI/ML for mobility, there are 6 scenarios, scenario 2, 3, 4 are prioritized. Scenario 4 is FR2 intra-frequency temporal domain case A (i.e., prediction of measurements in a prediction window by using historical actual measurements in an observation window). Scenario 2 is FR1 intra-frequency temporal domain case B (similar to case A, but predicted measurement result(s) in previous prediction window are skipped). And scenario 3 is FR1 inter-frequency prediction. The study on temporal domain case A is to verify performance improvement, i.e. how much handover performance can be improved based on predicted measurement results/event in future. The motivation of study on temporal domain case B is about measurement reduction, i.e. whether the degradation of handover performance is acceptable while partial measurement is skipped.

Table 1 Evaluation scenarios for RRM measurement prediction

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

During study phase, there is discussion on whether to consider other spatial domain prediction. The conclusion, is that except scenario 2, 3, 4, discussion on other scenarios are not precluded in work item.

TR 38.744 section 6.2.1 The impact on RRM requirements for L3 cell-level RSRP (i.e., Point C as defined in Figure 9.2.4-1 in TS 38.300) measurement prediction has been studied for scenarios 2, 3, and 4 as defined in Table 5.2.1.1-1 and the sub-use cases 1, 2, and 3 as defined in section 4.2. Editor Note: The impact of L3 beam level (Point E as defined in Figure 9.2.4-1 in TS 38.300) measurement prediction has not been studied. However, it doesn’t preclude the discussion on L3 beam level measurement prediction in the work item phase. Editor Note: From RAN4 perspective, it is not precluded that other scenarios in Table 5.2.1.1-1 will be discussed in work item.

According to the WID [2], temporal domain prediction case A, temporal domain prediction case B, and frequency domain prediction for co-located cases are considered. For RAN4 work, we can follow the WID and focus on these three scenarios. In last meeting, it was agreed to consider FR1 to FR1 intra-frequency temporal domain case A, FR1 to FR1 intra-frequency temporal domain case B, FR2 to FR2 intra-frequency temporal domain case A, FR2 to FR2 intra-frequency temporal domain case B, FR1 to FR1 inter-frequency (frequency domain) (co-located case). The open issues is whether to consider FR2 to FR2 inter-frequency (frequency domain) (co-located case). In our view, FR2 to FR2 inter-frequency (frequency domain) (co-located case) is not considered in Rel-19, and the performance is not evaluated in RAN2. It is proposed not to consider FR2 case, and focus on FR1 for frequency domain prediction.

Proposal 1: for frequency domain prediction, it is proposed that RAN4 focus on FR1 to FR1 inter-frequency co-located case, and not consider FR2 to FR2 inter-frequency co-located case in Rel-20.

Agreement: - As starting point, evaluate sub-case 2, L3-L3 measurement prediction as baseline for both FR1 and FR2. - Interested companies can also provide results for other sub-cases. - It is not precluded to add additional sub-case for FR2 in future meetings. - Further discuss how to define requirements based on the available simulation results. Ad-hoc agreements on Thursday: Agreement: As starting point, evaluate non-cluster (single-cell input and single-cell output) approach for frequency domain prediction as baseline. - Interested companies can also provide results for cluster approach. Further discuss the detailed simulation assumptions. - Further discuss how to define requirements based on the available simulation results. Way forward: - Consider the following frequency pairs as baseline for frequency domain prediction evaluation. - Candidate option 1: UE use the measurement on 2GHz to predict 4GHz. - Candidate option 2: UE use the measurement on 1.8GHz to predict 3.5GHz. - Candidate option 3: UE use the measurement on 1.8GHz to predict 2.1GHz. - Other options are not precluded. - Further discuss how to define requirements based on the available simulation results.

One FFS is frequency pairs for frequency domain prediction evaluation. Frequency domain prediction focus on co-located case. In practical, most likely, 2.6GHz and 4.9GHz are deployed in co-located manner. It is preferred to use the frequency pair of 2.6GHz and4.9GHz, i.e. UE use the measurement on 2.6GHz to predict 4.9GHz.

Proposal 2: for frequency domain prediction evaluation, it is proposed to use the measurement on 2.6GHz to predict 4.9GHz.

- Conclusion

This contribution provides discussion on general aspects. The proposals are:

Proposal 1: for frequency domain prediction, it is proposed that RAN4 focus on FR1 to FR1 inter-frequency co-located case, and not consider FR2 to FR2 inter-frequency co-located case in Rel-20.

Proposal 2: for frequency domain prediction evaluation, it is proposed to use the measurement on 2.6GHz to predict 4.9GHz.

- Reference

- RP-251864, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- R4-2514827, WF on NR_AIML_Mob_RRM, OPPO

- TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR