3GPP TSG-RAN WG4 Meeting #117	R4-2520531

Dallas, United States, 17th Nov 2025 - 21st Nov 2025

Source:	Nokia

Title:	Views on 6G SI demodulation topics

Agenda item:	8.7

Document for:	Discussion

Introduction

In this contribution we highlight our views on the 6G SI content for topics concerning RAN4 demodulation performance requirements. We propose several objectives to consider in this study, that we believe would benefit RAN4 specifications and the ecosystem in general for the era of 6GR. Additionally we highlight focus areas for the ongoing discussion [7].

Discussion

Performance testing and requirements

In the first meeting on 6G Demod an initial agreement was reached regarding the mission of the 6G demod SI and, hence, the role of demodulation requirements in 6G [7]:

General performance requirement aspects - Agreement - In 6G SI, RAN4 will study how to strive to develop a demod/test performance requirement framework for the minimum performance to better reflect real field conditions than 5G, if necessary, by taking into consideration of the requirement/test coverage/alignmentablity and the test feasibility, complexity and cost/resources.

While this initial agreement is indirectly weighting on the “performance” side of our earlier “Performance requirements vs. functional requirements” proposal, we see it as important make a clear, direct, and unanimous statement to reaffirm/align the mission of RAN4 demodulation performance.

In technical specifications, there are two types of requirements: Functional and non-functional, i.e., performance, requirements. The prior ones are defined to define support for specific use cases and functionalities. The latter ones are defining quantitative performance metrics to define “how well” a feature’s implementation needs to work or be acceptable for the whole system.	
It is our understanding that RAN4 defines minimum performance requirements to prevent a race to the bottom between NW and UE implementation, as well as, system configuration, due to inherent complexity vs. performance trade-offs in product implementations. Additionally, minimum performance requirements aid in NW planning, (NW/UE) troubleshooting, and (NW/UE) product development. Interoperability between networks and devices is guaranteed using performance requirements, and interoperability device test costs are reduced.

Over the course of 5GNR we have repeatedly heard statements like “we are only doing functional requirements in any case”, albeit that statements mentioning functional requirements are captured very rarely in meeting minutes (e.g., [5]) and usually elicit strong reactions from the chair of the session.

One of the latest examples are the discussion whether to introduce False Acceptance Ratio (FAR) requirements in LowPower-WakeUpSignal (LP-WUS) or not. From the RAN1 agreements a device supporting LP-WUS decoding can autonomously change from a specialized receiver to main radio without any signalling to the network. This means that when test is conducted in normal mode (no test mode) for the LP-WUR and the count of missed detection is based on the full wake up sequence from idle to connected mode the test equipment cannot distinguish between wake-up sequences there has been triggered by a correct decoding of the LP-WUS or triggered in the main radio (MR) by the Paging Occasion (PO) sequence. This means that a test in normal mode of the LP-WUR is based on MR results and can neither be used as a criterion for a functional LP-WUR as the test can be conducted by the MR only, nor can it be used for performance requirement. Instead, it can be argued that the requirements are functional requirements, which are outside the scope of defining demodulation performance requirements.

In RAN4#116bis it was not agreed to measure FAR but only MDR should be measured. Now it does not make sense to do any testing of the LP-WUR as all devices even without having a wakeup receiver as the main radio can pass the requirements, i.e., perform decoding of the PO, hence we see the expected final requirements as functional requirements instead of performance requirements.

Furthermore, various online exchanges (e.g., during Rel-16 NR-U) have evoked the notion that having RRM performance requirements specified is often enough to somehow functionally guarantee demodulation “performance”. Lately in NR_MIMO_Ph5-Perf this has been the case in the discussions regarding the new CJT reporting feature where it has several times been argued that no demodulation performance requirements are needed as RRM will introduce reporting requirements. Problem is that the introduced RRM requirement will very likely have a very big pass range that will make it much more seen as a functional requirement instead of a performance requirement.

Given our understanding about the reason for using non-functional performance requirements in RAN4 demodulation and CSI reporting requirements given above, we do not think that such statements have a place in RAN4.

Hence, we propose:

RAN4 to explicitly state the mission of RAN4 DMD being to produce performance requirements, not functional requirements.

There is a potential overlap between RRM reporting performance requirements and CSI reporting requirements (and TA demodulation requirements). RAN4 should also re-discuss the separation between RRM and Demod performance tasks. Often the groups seem to be distinguished via (e.g., timing or power) reporting accuracy in RRM versus reporting impact on demodulation performance in Demod. However, this causes some explicit overlap that RAN4 should either explicitly acknowledge, and state whether performance requirements in one domain may replace the same in the other domain, or resolve the overlap via explicit splitting of responsibilities.

RAN4 to discuss appropriate performance requirement task separation between RRM and Demod, when reports are involved.

Additionally, it was brought up in RAN4#116b, whether a generalized SNR derivation procedure can be adopted for 6GR. Since Rel-15, BS demod is using a standard SNR derivation procedure with outlier removal [9], which shall be re-used for 6GR in both BS and UE demodulation.

RAN4 shall reuse the Rel-15 BS demod SNR derivation procedure with outlier removal for both BS and UE demodulation.

Channel models

We have previously stated our preference for the rCDL channel model (or an evolution thereof) for MIMO features [8]. Following the discussions at the last meeting, many contributors seem open to the use of rCDL, if the (TDL vs. CDL use case) selection criteria are clear.

Following the intent behind the rCDL channel model [1][2], i.e., guaranteeing that in the field the UEs and NW will make a reasonably efficient use of MIMO features specified, e.g., providing appropriate channel state information to the network such that the network can make optimal use of the available MIMO layers, gives a potential selection criteria.

Reliable MIMO feature performance requirements need to be set under adequate test conditions that mimic MIMO effects, e.g., spatial effects, observed in deployment as much as possible. The legacy TDL channels do not exhibit these effects nor match the expected statistics of a measured channel.	
The rCDL SCM developed in 5GA was found to be suitable for this task, and it aligns with RAN1/2 assumptions during MIMO feature development. Additionally, with this increase in requirement quality, it shall also be possible to reduce the number of requirements overall, to guarantee a reliable demodulation and CSI reporting implementation performance for all NW/UE combinations in the field.

As such rCDL is especially relevant for MIMO features with >1 layer being transmitted in the same time/frequency resources. For single layer cases the legacy models can be sufficient for some goals and use cases (e.g., single layer PDCCH). However, a complete feature coverage using a SCM as baseline is preferred for comparability. Additionally, a limited number of TDL requirements for comparison with 5G may be considered.

RAN4 to use rCDL as baseline for MIMO features with >[1] layer (counting both signal and interference layers) being transmitted. For single layer cases the legacy models can be sufficient.

Other channel model related aspects raised in the last meeting, are not fit for discussion in this study item.	
For example, spatial channel model for AIML use cases are excluded by the current agenda item. The rCDL SCM derived in 5G, can fulfil the need for random, but partially predictable, time-wise evolution of the spatial characteristics of a SCM for AIML use cases via application of the legacy beamsteering framework, or via mu_desired table evolution, or via direct DoA/DoD angle evolution. However, such extensions shall be handled by the features that use the channel model, and not by the demod-led study, to save time units.

AIML extensions to the SCM framework shall be studied by the AIML 6GR study, if needed.

Additionally, frequency related aspects of the SCM do not need to be discussed, as RAN4 can and shall take modifications to that effect directly from the RAN1 work.

Frequency related aspects not to be discussed in RAN4 and potential CDL modifications to be directly adapted from RAN1 6G study.

Receiver assumptions

When considering UE receivers, it may be beneficial to have both baseline (e.g., linear) and advanced (e.g., non-linear) receiver requirements from day 1. However, reliable network assistance information will not be available in day 1 deployments, hence only the performance robust and reliable network transparent R-ML is practically relevant for release 21. In existing 5G specification, the use of R-ML receiver has triggered the definition of network support by introducing both RLC as well as DCI signalling to support the UE in reducing blind detection search space. One target for 6G shall be to fully remove the need for Network signalling and instead ensure that the UE implements blind detection algorithms, which will result in a more reliable and stable deployment performance as the UE will always have the correct information available. Any network transparent approaches can be considered.

RAN4 to consider MMSE-IRC and network transparent R-ML as baseline UE receivers from day 1.

Concerning the network side, only using MMSE-IRC as a baseline receiver makes sense. For devices with many Rx branches (e.g., BSs) the performance between linear and non-linear algorithms is decreasing with the number of branches (assuming constant number of transmitted layers) and the difference is close to zero for the number of branches envisioned for 6GR. Since furthermore the algorithmic complexity for R-ML is increased exponentially with the number of receiver (base band) branches, it does not make sense to consider pure R-ML on the network side. Any potential intermediate massive MIMO receiver implementations may differ greatly between architectures and thus will not be able to be aligned in RAN4 demod.

RAN4 to consider MMSE-IRC as baseline BS receiver.

TxEVM and SNR

Deployed UEs regularly see up to 35dB of receive SNR. However, in 5G we have applied a receive (soft) SNR limitation of 20dB to both UE and BS requirements. On the UE side this limit is non-obviously implemented, via a test equipment (TE) TxEVM assumption for impaired results derivation. However, the commonly used TxEVM values of “6% at QPSK, 6% at 16QAM, 6% at 64QAM, 3% at 256QAM, and 2.5% at 1024QAM” were grossly underestimating the true TxEVM performance of the TEs, as crest factor reduction is not employed in TEs, furthermore these values disregard that the TE TxEVM is independent from the used modulation order (assuming same power).

Additionally, RAN4 demod requirements are traditionally decoupled from RF requirements and impairments, such as to not conflate demodulation and RF impact on the performance. Hence it is difficult to understand, why such TxEVM was considered in 5GNR [3].	
The base station TxEVM in deployment is rather dynamic and matched to transmission conditions, such as to not impede the effective receive SNR required for a specific transmission configuration; one might think of adapted crest factor reduction or scheduling of groups of UEs with low pathloss.	
I.e., a more deployment oriented constraint on the BS TxEVM in a BB demod test (not RF test) is to dynamically chose a TE TxEVM value that does not impact the effective receive SNR operating point by more than [x] dB, where [x] may be 1dB; similar to current derivation of Es values in DL and AWGN_offset in UL.

The base station TxEVM in deployment is rather dynamic and matched to transmission conditions, such as to not impede the effective receive SNR required for a specific transmission configuration. A more deployment-oriented constraint on the BS TxEVM in a BB demod test (not RF test) is to dynamically chose a TE TxEVM value that does not impact the effective receive SNR operating point by more than [x] dB.

The TxEVM performance of practical TEs is below 1.5% in FR1 without carrier aggregation, and independent of modulation order. Using overly conservative TxEVM values in impaired simulations to derive performance requirements, means the DUT will be subjected to a much better effective SNR in the test than in the simulations (at the baseband receiver and for upper SNR limits).

It is understood that SNR limitations were imposed by consideration of worst-case configuration for the test equipment, e.g., OTA setup (with noise power transmission amplification), full power, full resource allocation, full CA, max bandwidths, and then enforced also on the majority of non-limited configuration, e.g., conducted test without carrier aggregation and with limited RE time/frequency domain allocation.

Ultimately, the direct (BS) and indirect (UE) limitation of SNR operating point ranges, means that 5GNR does not have requirements for high TPUT, high MCS, and other highly modem implementation stressing configurations. Furthermore, RAN4 demod has often chosen requirements configurations to lower the SNR operating point, that result in requirements outside the design operating points of the modems.

As such we propose:

Distinguish and decouple RF TxEVM assumptions from baseband demodulation TxEVM assumptions. I.e., RAN4 shall not be re-using or imposing a RF TxEVM value for demod requirements.

RAN4 shall abandon the SNR operating point limitations via fixed 20dB rule, or fixed test equipment TxEVM assumptions, and adopt a SNR limitation derivation based on actual TDRA/FDRA configuration.

A deployment-oriented constraint on the BS TxEVM in a BB demod test (not RF test) is to dynamically chose a TE TxEVM value that does not impact the effective receive SNR operating point by more than [x] dB.

New TE functionalities

ATP extension with OLLA

5GNR made a first foray into employing a known TE algorithm to include the outer loop into link adaption. This arguably does not constitute higher layer aspects but still drives towards more dynamic and deployment aligned performance requirements.

During the Rel-17 discussions about “Application Layer Throughput” (ATP) it was proposed to also include OLLA, when defining requirements for ATP. One of the main arguments to include OLLA was that OLLA is always implemented in the NW side, hence existing results without OLLA would not represent actual deployment scenarios.	
In addition, including OLLA would secure the UEs implementation also being able to handle situations where the UE requested CQI would not be set by the NW, if it does not result in approximately 10% BLER, which is only one target used for a deployment case.	
It can also be noted that UE is designed for certain operating point and should not be forced into non-optimal MCS/SNR combinations. Including OLLA in the simulations would partially correct transmissions towards the 10% (or any other agreed) BLER operation point as intended by the CQI definition.	
To underline above it can be seen in the simulation results from the discussions in “Application layer Data Throughput Performance” (ATP) [11] that 10% BLER was far from achieved across the tested SNR range, hence the introduced requirements for ATP are far from actual deployment scenarios.

Of course, including OLLA would mean that the UE is not tested against the UE requested CQI but more what the throughput will actually be in real deployment, hence it can be argued that testing performance both with and without OLLA will have merits to achieve full performance test coverage. It can be further discussed if all performance tests should include OLLA or if it can be left to a smaller number of selected testcases.

In the previous WI, including OLLA was not ultimately postponed as:

- Including OLLA would require an agreement into the implementation to be used in the TE. RAN4 Demod has not done this before without it already being specified by RAN 1 (i.e. PMI and CQI requirements). It will take time to agree on the “behaviour” of the base station/TE for handling OLLA.

- Due to the expected complexity of aligning and defining OLLA for the TE there was no time to further study in the WI.

Including OLLA will provide more deployment aligned requirements. Further study is required to analyse and define a minimum performance procedure to implement OLLA in TE.

In RAN4#106, Nokia provided a proposal for a simplified model of OLLA (see [6]) based on academic publications, which we believe is a good starting point for discussing a 6G implementation in the TE. The same model shall then be used in simulation configuration enabling full simulation including OLLA implementation.

Study the impact of including OLLA in ATP requirements with relation to actual deployment. Compare results with existing ATP requirements defined without OLLA. Use proposed OLLA model from [R4-2300703] as starting point.

Study how a simple implementation model for OLLA can be defined. Target of the model is implementation in TE.

Dynamic and state transition requirements

Current demodulation and CSI reporting requirements are often neglecting to dynamically react to DUT feedback and to consider scheduling aspects (i.e., dynamic states of requirements). This leads the DUT to be tested at operating points that are not in line with deployment and do not match receiver implementation performance in the presence of deployment resource allocation and scheduling.	
Additionally, this makes it impossible to predict and lower bound performance scaling in deployment environments.

5GNR demodulation and CSI reporting requirements are often neglecting to dynamically react to DUT feedback and to consider scheduling aspects.

In the past (e.g., Rel-16/Rel-17 enhanced MIMO codebooks) the test equipment complexity was sometimes quoted as a limiting factor for more dynamic or complex test cases. Discussed and WF captured comebacks in later releases never came to be (e.g., [10]):

Issue 4-1-1: Whether to define PMI requirement for Rel-17 FeTye II PS codebook - Agreement: - RAN4 will not introduce requirement of Rel-17 FeType II PS CB in Rel-17 Timeframe RAN4 can further discuss detailed parameters in RAN4#104 for reference of future work.

There are two levels of dynamic deployment aspects that shall be taken into account in 6GR demodulation performance requirements:

- TE applies DUT feedback.	
E.g., RI, PMI, etc.

- TE takes decisions via known algorithms.	
E.g., SU/MU scheduling, dynamic resource allocation/slots, applying timing offset reports (CJT), OLLA, etc.

RAN4 to study inclusion of demodulation requirements with increased dynamic application of DUT feedback in the TE.

RAN4 to study inclusion of demodulation requirements that include dynamic TE decisions using known algorithms, e.g. , dynamic resource allocation/slots, SU/MU scheduling, MU precoding, applying timing offset reports (CJT), OLLA, etc.

UE classification and applicability

According to initial agreements and discussions in RAN1 [4], it is likely that UE classifications will be a concept in 6GR.

Pending further RAN1/2 decisions, UE classifications can be used to replace (or very much limit) usage of applicability rules, which have made the 5GNR requirements applicability incredibly difficult to follow and quite contentious; in 5GA a large portion of discussion time was spent heckling over legacy and new applicability rules.

It is not yet clear, whether there will be baseline features for all UE classifications or all UE classifications will have fully independent feature sets. Nevertheless, RAN4 should define demod performance requirements in line with UE classification definition rules, while keeping an eye on future expandability.	
In any case, by default all mandatory features per UE classification shall have mandatory demod requirement(s), and optional features should have mandatory requirements, if the feature is supported.

RAN4 to study demod requirement handling for UE classifications and agree on questions such as a baseline set of requirements for all devices vs. individual requirements for each UE classification.

Feedback-less channels/signals

In 5GNR demod requirements have been plagued ever more by broadcast and feedback-less channels/signals being declared non-testable. For example, PBCH/SIB was the first one in Rel-15, then common search space PDCCH, PRACH Msg4, and parts of NTN, were to the long list of non-testable channels and signals; current discussions may further expand the list with the new CJT reporting signalling discussed in MIMO_ph5 and parts of LPWUS.	
In 6GR the number of feedback-less channels and signals may increase, especially due to power saving targets. As such RAN4 should study and agree to assume broadcast and feedback-less channels/signals to be testable.

One argument used in 5GNR to make the non-testable declaration was the lack of UE feedback to the system simulator (via 3GPP or other interfaces), so that the TE could not derive the performance indicators for the UE. There are many approaches to overcome this in 6GR, for example:

- Test modes/test loops.

- BS testing style test harness.

- KPI derivation at DUT and feedback to TE/SS.

- MDT style interfaces.

- Indirect inference of performance indicators.

- Test UEs.

- Etc.

Seeing the number of potential solutions to the 5GNR perceived problem, we propose to:

RAN4 to assume broadcast and feedback-less channels/signals to be testable. RAN4 to recommend to RAN5 to define needed test solutions.

Conclusion

In this contribution, we have presented our views on the 6G SI content for topics concerning RAN4 demodulation performance requirements. We propose several objectives to consider in this study, which we believe would benefit RAN4 specifications and the ecosystem in general for the era of 6GR.	
The following proposals have been made:

== Performance testing and requirements

- In RAN4#116bis it was not agreed to measure FAR but only MDR should be measured. Now it does not make sense to do any testing of the LP-WUR as all devices even without having a wakeup receiver as the main radio can pass the requirements, i.e., perform decoding of the PO, hence we see the expected final requirements as functional requirements instead of performance requirements.

- RAN4 to explicitly state the mission of RAN4 DMD being to produce performance requirements, not functional requirements.

- RAN4 to discuss appropriate performance requirement task separation between RRM and Demod, when reports are involved.

- RAN4 shall reuse the Rel-15 BS demod SNR derivation procedure with outlier removal for both BS and UE demodulation.

== Channel models

RAN4 to use rCDL as baseline for MIMO features with >[1] layer (counting both signal and interference layers) being transmitted. For single layer cases the legacy models can be sufficient.

AIML extensions to the SCM framework shall be studied by the AIML 6GR study, if needed.

Frequency related aspects not to be discussed in RAN4 and potential CDL modifications to be directly adapted from RAN1 6G study.

== Receiver assumptions

RAN4 to consider MMSE-IRC and network transparent R-ML as baseline UE receivers from day 1.

RAN4 to consider MMSE-IRC as baseline BS receiver.

== TxEVM and SNR

The base station TxEVM in deployment is rather dynamic and matched to transmission conditions, such as to not impede the effective receive SNR required for a specific transmission configuration. A more deployment-oriented constraint on the BS TxEVM in a BB demod test (not RF test) is to dynamically chose a TE TxEVM value that does not impact the effective receive SNR operating point by more than [x] dB.

Distinguish and decouple RF TxEVM assumptions from baseband demodulation TxEVM assumptions. I.e., RAN4 shall not be re-using or imposing a RF TxEVM value for demod requirements.

RAN4 shall abandon the SNR operating point limitations via fixed 20dB rule, or fixed test equipment TxEVM assumptions, and adopt a SNR limitation derivation based on actual TDRA/FDRA configuration.

A deployment-oriented constraint on the BS TxEVM in a BB demod test (not RF test) is to dynamically chose a TE TxEVM value that does not impact the effective receive SNR operating point by more than [x] dB.

== New TE functionalities

= ATP extension with OLLA

Including OLLA will provide more deployment aligned requirements. Further study is required to analyse and define a minimum performance procedure to implement OLLA in TE.

Study the impact of including OLLA in ATP requirements with relation to actual deployment. Compare results with existing ATP requirements defined without OLLA. Use proposed OLLA model from [R4-2300703] as starting point.

Study how a simple implementation model for OLLA can be defined. Target of the model is implementation in TE.

= Dynamic and state transition requirements

5GNR demodulation and CSI reporting requirements are often neglecting to dynamically react to DUT feedback and to consider scheduling aspects.

RAN4 to study inclusion of demodulation requirements with increased dynamic application of DUT feedback in the TE.

RAN4 to study inclusion of demodulation requirements that include dynamic TE decisions using known algorithms, e.g. , dynamic resource allocation/slots, SU/MU scheduling, MU precoding, applying timing offset reports (CJT), OLLA, etc.

== UE classification and applicability

RAN4 to study demod requirement handling for UE classifications and agree on questions such as a baseline set of requirements for all devices vs. individual requirements for each UE classification.

== Feedback-less channels/signals

RAN4 to assume broadcast and feedback-less channels/signals to be testable. RAN4 to recommend to RAN5 to define needed test solutions.

- References

- RP-252186 TR 38.753 v2.0.0, Nokia, RAN#109.

- RP-252957 WID: NR demodulation performance: Phase 6, Apple, RAN4#109.

- R4-2318350 Discussion on UE Demodulation for non-colocated FR1 intra-band NR-CA, Nokia, RAN4#109.

- R1-2506618 FL summary#3 on overview of 6GR air interface, NTT Docomo, RAN1#122.

- R4-1913101 RAN4 #92-Bis Meeting Report, ETSI MCC, RAN4#93.

- R4-2300703 On Absolute physical layer throughput requirements with link adaptation, Nokia, RAN4#106.

- R4-2514588 WF on [116bis][106] 6G Demod, Feature Lead, RAN4#116bis.

- R4-2513025 Views on 6G SI demodulation topics, Nokia, RAN4#116bis.

- R4-1904713 WF for requirement SNR derivation procedure, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, RAN4#90Bis.

- Chair minutes, 3GPP TSG-RAN WG4 Meeting #104-e.

- R4-2113123 Summary of simulation results for NR UE Application Layer Data Throughput Performance, RAN4#100-e

Note that in 5GNR some hard-fought exceptions to the 20dB limit exist. For example, a 30dB SNR for the non-collocated work-item which required a large power difference between the two sources. Such exceptions required extensive discussion and constraints (in this case AWGN channel, or reduction of AWGN_offset on the BS side).	
Additionally other high SNR test cases are marked as non-testable in RAN5, especially for OTA.